| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, Korea, 20-28 January 2011 | Document: JCTVC-D210-rev1 WG11 Number: m18970 |
| --- | --- |

| Title: | Study on Nokia planar intra prediction |  |  |
| --- | --- | --- | --- |
| Status: | [Input Document to JCT-VC] |  |  |
| Purpose: | [Report and Proposal] |  |  |
| Author(s) or Contact(s): | Keiichi Chono Kenta Senzaki Hirofumi Aoki Junji Tajime Yuzo Senda 1753, Shimonumabe, Nakahara-Ku Kawasaki, Kanagawa, 211-8666 JAPAN | Tel: Email: | +81 50 431 7595 chono@ct.jp.nec.com k-senzaki@bp.jp.nec.com hao@bu.jp.nec.com tajime@ak.jp.nec.com y-senda@bc.jp.nec.com |
| Source: | [Information and Media Processing Labs, NEC Corporation] |  |  |

_____________________________

# Abstract

This contribution presents implementation analysis and simulation results of Nokia planar intra prediction. It is confirmed that the BD-rate results match those provided by proponent. Subjective quality analysis shows that Nokia planar intra prediction significantly reduces blocking artifacts. In order to further enhance video quality of Nokia planar intra prediction, it is recommended that the concept of planar intra prediction based on conditional dithering is studied in the context of core experiments.

- Introduction

This contribution describes implementation analysis, simulation results, and its video quality enhancement idea of Nokia planar intra prediction.

- Implementation analysis

The following improved implementations are observed from the software provided by Nokia:

- Planar intra prediction is used in all slice types;

- Planar intra prediction is disabled for CUs which are smaller than 16x16, and no side-information is signaled in those CUs when entropy coding is LCEC;

- The use of Plana intra prediction is jointly signaled with PU partitioning size information as follows;

  - In LCEC, symbols ”1”, ”01”, and ”00” stand for ”2Nx2N”, ”NxN”, and ”Planar”, respectively.

  - In CABAC, symbols “1”, ”01”, and ”00” stand for ”Planar”, ”2Nx2N”, and ”NxN” respectively.

- The selection of Plana intra prediction is intentionally prioritized in smooth region by deriving its R-D cost from distortion only. Smooth regions are detected by checking the energies of pixel intensities and their low-frequency DCT coefficients.

# Simulation

Software provided by Nokia was successfully compiled by Microsoft Visual C++ 2008, and its 64-bit encode and decode executables were tested on Windows-based computing platforms listed below.

| Computing platform | Usage |
| --- | --- |
| Windows 7 64-bit Professional Intel Xeon 3.33GHz and Memory 32GB | Up to 12 processes for encoding and decoding ran at the same time. |

Attached an Excel sheet, NEC_NEW_Planr_results.xls, contains the detailed results of common test condition described in JCTVC-C500 [1]. Table 1 shows the summary of those results. It was confirmed that BD-PSNR and BD-Rate results are the same as those of provided by Nokia.

Table 1: Summary of results.

Simulation results show that Planar prediction does not significantly impact average BD-rates, and encode and decoder times.

# Subjective quality analysis

We compared Anchor and Planar results for Kimono sequences. Attached file, “NEC-Planar-pics.pdf”, shows some representative screen shots of coded results. We observed blocking artifacts in the face and sky regions are significantly reduced by Planar prediction, especially in Low Complexity setting with high QP values. However, we observed that even with Planar intra prediction, banding artifacts associated with bit depth reduction of IBDI are not suppressed as JCTVC-D043 does. Therefore it is suggested that the concept of conditional dithering in JCTVC-D043 is incorporated into the design of Planar intra prediction for its further video quality enhancement. The description of proposed planar intra prediction based on conditional dithering is attached in appendix.

# Conclusions

This contribution has presented implementation analysis and simulation results of Nokia Planar intra prediction. Subjective quality analysis has showed that Nokia Planar intra prediction significantly reduces blocking artifacts. In order to further enhance its video quality, it is recommended that the concept of planar prediction based on conditional dithering is studied in the context of core experiments.

# Patent rights declaration(s)

NEC Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Reference

- JCT-VC, “Common test conditions and software reference configurations”, JCTVC-C500, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010.

Appendix: The description of proposed planar prediction based on conditional dithering

(The modified portion of JCT-VC B205 is highlighted with red colour.)

5.1.1.3.1 Specification of intra planar prediction

This intra prediction mode is invoked when planar_flag == 1. The values of the prediction samples predPartL[ x, y ],

with x, y = 0..PuPartSize – 1, are derived bypredPartL[ x, y ] = Clip0-255(((PuPartSize – (y+1))*TR(x) + (y+1)*BR(x) + (PuPartSize – (x+1))*LC(y) + (x+1)*RC(y) + Dither [x, y] )/(PuPartSize*2))

where the values of TR pixel array are specified as

– If the samples p[ x, -1 ] with x = 0..PuPartSize – 1 are marked as "available for intra prediction.”

TR(i) = p[i,-1 ] with i = 0..PuPartSize - 1

– Otherwise if the samples p[ -1, y ] with x = 0..PuPartSize – 1 are marked as "available for intra prediction.”

TR(i) = ((PuPartSize – (i+1))*M(-1,0) + (i+1)*BRS + (PuPartSize>>1))/PuPartSize with i = 0..PuPartSize - 1

– Otherwise

TR(i) = 128 with i = 0..PuPartSize - 1

where the values of LC pixel array are specified as

– If the samples p[ -1, y ] with y = 0..PuPartSize – 1 are marked as "available for intra prediction.”

LC(i) = p[ -1, i ] with i = 0..PuPartSize - 1

– Otherwise if the samples p[ x, -1 ] with x = 0..PuPartSize – 1 are marked as "available for intra prediction.”

LC(j) = ((PuPartSize – (j+1))*M(0,-1) + (j+1)*BRS + (PuPartSize>>1))/PuPartSize

– Otherwise

LC(i) = 128 with i = 0..PuPartSize - 1

where the values of BR pixel array are specified as

BR(i) = ((PuPartSize – (i+1)*LC[ PuPartSize-1 ] + (i+1)*BRS + (PuPartSize>>1))/PuPartSize with i = 0..PuPartSize

- 1

where the values of RC pixel array are specified as

RC(i) = ((PuPartSize – (i+1))*TR[ PuPartSize-1 ] + (i+1)*BRS + (PuPartSize>>1))/PuPartSize with i =

0..PuPartSize

where the BRS value is specified as

BRS = DC + planar_delta_y

where DC is calculated identically to the DC value in DC prediction as specified in subclause 5.1.1.3.3.3

where the value of Dither [x, y] is specified as

– If | planar_delta_y | is greater than or equal to PuPartSize

Dither [x, y] = PuPartSize

– Otherwise if the sample p[ x, y ] is an inner sample with x,y = 1..PuPartSize – 2

Dither [x, y] = LUT [PuPartSize*y + x]

– Otherwise

Dither [x, y] = PuPartSize