| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D217_r2 WG11 Number: m18977_r2 |
| --- | --- |

| Title: | CE 8: Results for adaptive loop filter using prediction and residual (3-Input-ALF) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report and Proposal |  |  |
| Author(s) or Contact(s): | Semih Esenlik, Matthias Narroschke, Thomas Wedi Panasonic R&D Center Germany Monzastr. 4c, 63225 Langen, Germany | Tel: Email: | +49 6103 766 1306 Semih.Esenlik@eu.panasonic.com +49 6103 766 204 Matthias.Narroschke@eu.panasonic.com +49 6103 766 1194 Thomas.Wedi@eu.panasonic.com |
| Source: | Panasonic Corporation |  |  |

_____________________________

# Abstract

This contribution is a part of the CE8: It presents the results of the Adaptive loop filter using the prediction and residual (3-Input-ALF). An average Y-BD-rate gain of 0.9% for Intra High Efficiency, 0.8% for Random Access High Efficiency, 0.4% for Low Delay High Efficiency, 4.6 % for Intra Low Complexity, 5.9 % for Random Access Low Complexity, and 5.5 % for Low Delay Low Complexity compared to the official anchors can be achieved. The additional complexity of the decoder is only marginal since only one additional sample of the prediction and of the quantized prediction error are used in addition to reference, which uses 5x5,…, 9x9 samples of the deblocked signal. It is proposed to include it in the HM.

# Introduction

The Adaptive loop filter using the prediction and residual (3-Input-ALF) was first presented as part of the proposal JCTVC-A114 [1] in Dresden. At the Dresden meeting, it was decided to include it into the TMuC [5]. Subsequently, it has been evaluated in the context of the TE12 at the Guangzhou meeting in October, 2010, see [2] and [3].

# Adaptive loop filter using the prediction and residual

The adaptive loop filter using the prediction and residual aims at reducing quantization noise. Based on the prediction signals, the quantized prediction errore', and the deblocked signals'', the following linear filter operations are performed:

s'''=i=1M−2wi⋅si''+wM−1⋅s+wM⋅e'+c

All filter coefficientswi, with i=1,…,M as well as the offset c are estimated once per frame by minimizing the mean squared error between the original input signalsand the filtered signals''':

D=Es−s'''2→min

It is noted that the size of the filters that are designed for prediction and quantized prediction error signals are1×1 in size. In other words, the prediction and quantized prediction error signals are merely weighted during the calculation of the output signal. The estimated filter coefficients are coded and transmitted to the decoder. The filter size M is determined by minimization of the Lagrangian cost of overall data rate R and mean squared errorD:

R⋅λ+D→min

# Software integration

The adaptive loop filter using the prediction and residual was integrated into the 0.9 version of the HM [6].

The software integration includes a harmonized version with the filter of Qualcomm, which is the one enabled in the common test conditions [4] by setting the macro QC_ALF to 1. In the following, the loop filter of Qualcomm is also denoted as QC_ALF filter. Our integration is based on our former software  implementation, see [7]. It allows the following macro configurations:

- Macro QC_ALF=1  and macro WIENER_3_INPUT=1 :
Adaptive loop filter using the prediction and residual within the QC_ALF filter

- Macro QC_ALF=1  and macro WIENER_3_INPUT=0:
Only the QC_ALF filter is used

During the harmonization of the 3-Input-ALF and the QC_ALF, special care is taken not to change the original QC_ALF implementation. In other words, all of the encoder control operations as well as the implementation aspects in encoder and decoder are kept exactly the same as in the reference software. Therefore the goal of the CE8 to clearly understand the advantages and disadvantages of this particular individual loop filter tool is thought to be achieved.

# Experiments

Software and configuration

The settings for the experiment are:

- Macro QC_ALF=1 and macro WIENER_3_INPUT=1 :
Adaptive loop filter using the prediction and residual within the QC_ALF filter

- Configuration parameter ALF=1

Tested are all six test cases, which are Intra, Random access, and Low delay, each in high efficiency and low complexity operation mode.

The configuration files of the SVN repository, which equal to those defined in [4], were used for the experiments. For all run time measurements, computers of the same configuration are used for both, reference and proposal.

Experimental results (comparison to the official anchors)

As a reference, the official tagged 0.9 version of the HM software [6] is used.

For the high efficiency operation mode the QC_ALF filter enabled in the reference software. The results reflect the gain of the adaptive loop filter using the prediction and residual compared to the loop filter QC_ALF.

For the low complexity operation mode, the anchors do not use any loop filter, disabled by setting the parameter ALF=0. The results reflect the gain of the Adaptive loop filter using the prediction and residual within the QC_ALF filter compared to no loop filter.

The following tables show the results of the experiment.

The attached excel file “JCTVC-D217_r2_CE8_3-Input-ALF_PANASONIC.xls” includes all further details.

Sequence dependent Y-BD-rate gains

Sequence dependent Y-BD-rate gains calculated for 3-Input-ALF are presented below. The reference simulations are carried out according to the Common Test Conditions [4]. The results for the six test cases are shown in the following Figures. All negative numbers show increased coding efficiency compared to the reference.

From the experimental results shown above, it can be seen that the adaptive loop filter using the prediction and residual integrated into the QC_ALF filter increases the coding efficiency for almost all of the test sequences. Only for two test sequences in Low Delay High Complexity configuration, no coding efficiency increase is achieved. In the High Efficiency operation mode, the harmonized loop filter shows an average Y-BD-rate gain of 0.9% for Intra, 0.8% for Random Access, and 0.4% for Low Delay compared to TMuC 0.9 anchor [6]. In the Low Complexity operation mode, it shows an average Y-BD-rate gain of 4.6 % for Intra, 5.9 % for Random Access, and 5.5 % for Low Delay compared to the HM 0.9 anchor.

Complexity Analysis

In 3-Input-ALF, 1×1 filters are used to filter residual and predictions signals. Keeping in mind that filter sizes up to 9×9 can be used in the anchor, the increase in computational complexity of the filter design and filtering operations increase only negligibly.

The reference QC_ALF implementation in TMuC 0.9 requires that original frame, post-deblocked frame, pixel-wise classification frame and correlation matrices are accessed in each encoder pass. The 3-Input-ALF requires two additional frames, prediction frame and quantized prediction error frame, in each during each encoding pass. If the operation of writing the filtered image is also counted, it can be seen that the frame memory access requirement is only moderately increased.

Improved Implementation

During the implementation of the 3-Input-ALF, special care has been taken not the change the 1-input reference part, which is the QC_ALF in 0.9 HM software. However it is known that implementation of the QC_ALF is not optimal, e.g. in terms of runtimes. In a second improved implementation, this issue is addressed to achieve lower runtimes.

In this second improved implementation, the parts of the encoder and decoder related to the adaptive loop filter are optimized. The following three aspects are optimized:

- Efficient pointer implementation of the filtering operation in the decoder.

- Reuse of already calculated correlation matrices during the filter size decision.

- Update of already calculated correlation matrices during the filtering region decision instead of calculation from scratch.

The goal of this second improved implementation is to reduce the encoder and decoder runtimes. Consequently, the same coding results are achieved as with the first implementation. Therefore, also the BD bitrates match exactly to the ones in the first implementation.

The results related to the improved implementation are as follows:

According to the results of the second implementation it is clear that a considerable amount of saving in terms of runtime is possible with small changes in the implementation. Especially in the case of intra only configurations, up to 16% reduction in the encoding time is achieved. The results of this experiment are considered as very valuable since they show that the runtimes can already be reduced significantly without any change in the number of encoding passes.

The results of the improved implementation are provided in the document “JCTVC-D217_r2_improved_implementation”.

# Conclusion

The Adaptive-loop filter using prediction and residual (3-Input-ALF) was first proposed in Dresden as part of the proposal JCTVC- A114 [1] and was adopted into the TMuC [5]. Subsequently, it has been continuously been further refined and evaluated in the context of the TE12 at the Guangzhou meeting [2] and [3]. The current implementation of the Adaptive loop filter using prediction and residual using the common test conditions [4] shows an average Y-BD-rate gain of 0.9% for Intra High Efficiency, 0.8% for Random Access High Efficiency, 0.4% for Low Delay High Efficiency, 4.6% for Intra Low Complexity, 5.9 % for Random Access Low Complexity, 5.5 % for Low Delay Low Complexity compared to the official anchors. The additional complexity of the decoder is only marginal since only one additional sample of the prediction and of the quantized prediction error are used in addition to reference, which uses 5x5,…, 9x9 samples of the deblocked signal.

Since the Adaptive loop filter using prediction and residual (3-Input-ALF) increases the coding efficiency substantially and consistently with increasing the decoder complexity only marginally, we see a value in this technique for the standardization and therefore propose to include it in the next version of the HM.

# References

- I. Amonou et. al., Description of video coding technology proposal by France Telecom, NTT, NTT DOCOMO, Panasonic and Technicolor, JVCVC-A114, Dresden, Germany, April 2010.

- M. Narroschke, “TE12.5: Results for adaptive loop filter using prediction and residual”, JCTVC-C214, Guangzhou, CN, October, 2010.

- K. McCann, “TE12: Summary of evaluation of TMuC tools in TE12”, JCTVC-C225, Guangzhou, CN, October, 2010.

- F. Bossen, “Common test conditions and software reference configurations,” JCTVC-C500, Guangzhou, CN, October 2010.

- JCTVC-B205, Test Model under Consideration, Geneva, Switzerland, July 2010.

- HM 0.9 Software, http://hevc.kw.bbc.co.uk/trac/browser/tags/0.9

- Software implementation of JCTVC-C214: http://hevc.kw.bbc.co.uk/trac/browser/branches/0.8-panasonic

# Patent rights declaration(s)

Panasonic Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).