| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D222 |
| --- | --- |

| Title: | Performance of Rotational Transform (ROT) in TMuC 0.9 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Xing Wen, Oscar C. Au, Lin Sun, Jiali Li, Department of Electronic and Computer Engineering Hong Kong University of Science and Technology Kowloon, Hong Kong | Tel: Email: | 852-66261845 wxxab@ust.hk |
| Source: | Hong Kong University of Science and Technology |  |  |

_____________________________

# Abstract

The 8x8 Rotational Transform (ROT) is a secondary transform after DCT and is implemented in the JCT-VC HEVC Test Model (HM). This is a contribution presenting the performance of Rotational Transform (ROT) tool integrated into the TMuC 0.9 software. Attached JCTVC-D222.xls is a copy of current results on ROT tests. In this report, we compare the results in which ROT is on and off in TMuC 0.9.

# Introduction

The secondary 8x8 Rotational Transform (ROT) is applied after DCT, the block diagram is shown in Fig. 1.

Figure 1: ROT is applied after the DCT

The ROT transfomed block mo can be obtained followed equations:

where mi is the input residue block, D is DCT matrix, Rh and Rv are horizontal ROT and vertical ROT matrices respectively.

The matrices Rh and Rv are defined as the following compund Given’s rotation matrix products:

which Rh and Rv can are in following forms:

In TMuC 0.9, there are four sets of rotation angles θ1i, θ2i, θ3i, θ4i, θ5i, θ6i (i = 1 to 4) which equals to four sets of Rh and Rv respectively.

# Experimental results

According to JCTVC-B312 [1], the simulation was conducted for all sequences of WQVGA, WVGA, 720p, 1080p and cropped 4kx2k based on the recommended simulation common conditions, JCTVC-B302[2]. The version of TMuC software is 0.9. The OS is Windows 7 Enterprise 64-bit, the CPU is Inter® Core (TM) i7, 2.8GHz with 4G and 8G memory. The test is conducted on the TMuC 0.9 with ROT enabled by setting “ROT :  1” in configuration files.

According to the Table 1, we can see that ROT improves coding performance about 2.6 % for “all intra” test condition and around 3.5% for “intra loco”. While for the random access ROT improves coding performance slightly larger than 1%.

Table 1  Y U V BD-rate (%), encoding and decoding time of ROT for intra

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BDR | U BDR | V BDR | Y BDR | U BDR | V BDR |
| Class A | -2.8 | -0.5 | 0.7 | -3.8 | -1.0 | 0.2 |
| Class B | -2.1 | -0.1 | 0.6 | -3.2 | -1.0 | -0.3 |
| Class C | -2.8 | -0.6 | -0.6 | -3.5 | -1.9 | -2.0 |
| Class D | -2.7 | -0.5 | -0.7 | -3.3 | -1.8 | -2.3 |
| Class E | -2.8 | 3.0 | 1.8 | -3.9 | 3.0 | 1.1 |
| All | -2.6 | 0.2 | 0.2 | -3.5 | -0.7 | -0.8 |
| Enc Time[%] | 404% |  |  | 557% |  |  |
| Dec Time[%] | 163% |  |  | 164% |  |  |

Table 2  Y U V BD-rate (%), encoding and decoding time of ROT for random access

|  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BDR | U BDR | V BDR | Y BDR | U BDR | V BDR |
| Class A | -1.2 | 0.0 | 0.8 | -1.5 | -0.3 | 0.4 |
| Class B | -1.1 | -0.1 | 0.5 | -1.3 | -0.4 | -0.2 |
| Class C | -1.3 | -0.6 | -0.6 | -1.4 | -0.9 | -1.0 |
| Class D | -1.1 | -0.1 | -0.2 | -1.1 | -0.6 | -0.9 |
| Class E |  |  |  |  |  |  |
| All | -1.2 | -0.2 | 0.1 | -1.3 | -0.6 | -0.5 |
| Enc Time[%] | 154% |  |  | 133% |  |  |
| Dec Time[%] | 113% |  |  | 165% |  |  |

The table 3 is the ROT performance summary based on TMuC 0.7 in condition of intra high efficiency and random access high efficiency [3]. The performance of ROT based on TMuC 0.7 is slightly worse than performance based on TMuC 0.9.

Table 3  Y U V BD-rate (%) for intra and random access based TMuC 0.7

|  | Intra ROT |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -2.1 | 1.1 | 2.0 |
| Class B | -1.9 | 0.6 | 1.2 |
| Class C | -2.3 | 0.0 | -0.1 |
| Class D | -2.1 | -0.1 | -0.2 |
| Class E | -2.4 | 3.6 | 2.8 |
| All | -2.1 | 0.9 | 0.9 |
| Enc Time[%] | 241% |  |  |
| Dec Time[%] | 102% |  |  |
|  |  | Random access ROT |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.9 | 0.5 | 0.8 |
| Class B | -0.9 | 0.0 | 0.4 |
| Class C | -1.0 | -0.5 | -0.5 |
| Class D | -0.8 | 0.0 | -0.2 |
| Class E |  |  |  |
| All | -0.9 | -0.1 | 0.1 |
| Enc Time[%] | 104% |  |  |
| Dec Time[%] | 101% |  |  |

# Conclusion

In this contribution, the experimental results of ROT on TMuC 0.9 were shown. The experimental results show that the coding gain of ROT is about 3.5% for all intra loco and 2.6% for all intra HE, and ROT can achieve 1.2% and 1.3% RA HE and RA loco. The gain for ROT in TMuC 0.9 is higher than the gain of ROT in TMuC 0.7 . It is suggested that ROT should be introduced into HM next version. However, the coding time increased by ROT should not be neglected, especially for all intra.

# Reference

[1]  Ken McCann, “Tool Experiment 12: Evaluation of TMuC Tools,” Doc. JCTVC-B312, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Geneva, CH, July 2010

[2]  Frank Bossen, “Common test conditions and software reference configurations,” Doc. JCTVC-B300, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Geneva, CH, July 2010

[3]  R. Cohen, A. Vetro, H. Sun , “Alternative performance measurement of MDDT and ROT in TMuC,”Doc. JCTVC-C036, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Guangzhou, October 2010

# Patent rights declaration(s)

The Hong Kong University of Science and Technology may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).