| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D223 |
| --- | --- |

| Title: | CE4: Interpolation filtering of chroma samples using two-stage averaging |  |  |
| --- | --- | --- | --- |
| Status: | JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | A. Fuldseth G. Bjøntegaard Phillip Pedersens vei 22 1366 Lysaker Norway | Tel: Email: | +47 92414082 arild.fuldseth@cisco.com |
| Source: | Cisco Systems Norway |  |  |

_____________________________

# Abstract

This contribution presents results for interpolation filtering for motion compensation of chroma samples based on JCTVC-A119 (TENTM). The interpolation is based on rounding chroma motion vectors to quarter sample accuracy and then using two-stage averaging for calculating sample values at fractional positions. The interpolation filtering does not require multiplications and require fewer additions than the AVC chroma interpolation filtering. Also, the averaging operation is well suited for SIMD-based implementations. The compression performance of the interpolation filtering method has been compared to the AVC chroma interpolation filter using the HM 0.9 software. Depending on the test condition, BDR (luma) penalties between 0.3% and 1.2% are observed while the average reduction of chroma PSNR varies between 0.12 dB and 0.26dB.

# Introduction

In the TENTM proposal to the HEVC CfP, JCTVC-A119, a low complexity interpolation method for motion compensation of chroma samples was described [1]. This interpolation method was also used for the low complexity configuration of TMuC 0.7, but was replaced with the AVC interpolation method in the HM 0.9 software. This document presents results for the TENTM chroma interpolation method compared to the anchor (AVC chroma interpolation method) using the HM 0.9 software. A complexity analysis is also provided.

# Description

The interpolation of chroma samples are performed in two steps:

- Rounding of motion vectors from one eight-sample accuracy to quarter-sample accuracy.

- Two-stage averaging

The two-stage averaging is described as follows. Consider the quarter sample positions below (A, B, C, and D are integer sample positions).

A   a   b   c   B
	d    e   f   g 
	h    i    j   k    h1
	l    m   n  o
	C        b1      D

The sample values at fractional positions are calculated as follows:

b 	= 	(A+B)>>1
h	= 	(A+C)>>1
j 	= 	(B+C)>>1

a 	= 	(A+b+1)>>1
c 	= 	(b+B+1)>>1
d	= 	(A+h+1)>>1
e 	= 	(b+h+1)>>1
f 	= 	(b+j+1)>>1
g	= 	(b+h1+1)>>1
i 	= 	(h+j+1)>>1
k 	= 	(j+h1+1)>>1
l 	= 	(h+C+1)>>1
m 	= 	(h+b1+1)>>1
n	= 	(j+b1+1)>>1
o 	= 	(h1+b1+1)>>1

Note that while the half sample positions b, h, and j are rounded downwards, the quarter sample positions are rounded upwards. This is to achieve a good balance between rounding and truncation.

# Complexity analysis

This section provides a complexity analysis using the AVC chroma interpolation filter as a reference. In AVC, a fractional chroma sample, x, is calculated using the following generic expression:

x = (w1*A + w2*B + w3*C + w4*D + 32)>>6,						(1)

where w1, w2, w3, and w4 are weights that depends on the fractional position. This operation requires 4 multiplications, 4 additions, and one shift.

In the TENTM method described above, any fractional sample value can be calculated with the following generic expression:

x = (((A+B)>>1)+((C+D)>>1)+1)>>1							(2)

which requires 3 additions and 3 shifts (excluding the rounding offset of +1).

Note however, that for horizontally and vertically aligned sample values; the AVC interpolation (1) can be expressed using the same form as in (2). Note also that for the half sample positions, the expression for both filters can be further simplified.

Also, the expression in (2) is particularly suitable for utilizing SIMD instructions since the expressions (X+Y)>>1 and (X+Y+1)>>1 can be easily vectorized so that multiple samples can be processed in parallel.

In practical implementations there are other contributions to the complexity, including memory read/write. Also, for small block sizes, calculation of the weights w1, w2, w3, and w4 of (1) might contribute significantly.

We have evaluated both interpolation methods in a real-time environment on an Intel processor with SIMD instructions. Typically, the proposed method uses half the CPU cycles of the AVC interpolation method.

# Results

Table 1 shows BD-rate results and encode- decode times (relative to the HM0.9 anchor) for four test cases. The common coding conditions [2] were used. In order to achieve consistent timing results, 9 frames for each sequence/QP combination were encoded and decoded serially on the same computer. The encode- and decode-times where then normalized by multiplying with N/9, where N is the number of frames in the full sequence. In addition to calculating BD-rate numbers for Y, U, and V separately, BD-rate using a combined PSNR measure was calculated:

;

Table 2 shows the average change in chroma PSNR for each test case. For each sequence/QP combination the difference between chroma PSNR for the anchor and the proposed method was calculated. The difference was averaged over all sequences and all QP values.

Table 1. BD-rate and encode- decode times for all test cases.

|  | Random access |  |  |  | Random access LoCo |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | YUV | Y | U | V | YUV |
| Class A | 0,4 | 7,4 | 6,7 | 2,7 | 0,3 | 6,0 | 5,9 | 2,1 |
| Class B | 0,2 | 11,6 | 11,0 | 3,1 | 0,2 | 8,0 | 10,6 | 2,4 |
| Class C | 0,4 | 7,3 | 8,3 | 3,0 | 0,3 | 4,7 | 4,6 | 1,7 |
| Class D | 0,3 | 9,3 | 11,4 | 2,9 | 0,3 | 6,8 | 6,9 | 1,8 |
| Class E |  |  |  |  |  |  |  |  |
| All | 0,3 | 9,3 | 9,8 | 3,0 | 0,3 | 6,5 | 7,4 | 2,0 |
| Enc Time[%] | 100 % |  |  |  | 99 % |  |  |  |
| Dec Time[%] | 100 % |  |  |  | 97 % |  |  |  |
|  | Low delay |  |  |  | Low delay LoCo |  |  |  |
|  | Y | U | V | YUV | Y | U | V | YUV |
| Class A |  |  |  |  |  |  |  |  |
| Class B | 1,0 | 12,2 | 15,0 | 4,5 | 0,7 | 8,4 | 10,8 | 3,2 |
| Class C | 1,4 | 8,1 | 8,9 | 4,3 | 0,8 | 5,1 | 5,0 | 2,4 |
| Class D | 1,6 | 12,6 | 12,0 | 4,2 | 0,9 | 6,7 | 5,8 | 2,4 |
| Class E | 0,6 | 17,1 | 20,9 | 6,4 | 0,4 | 16,2 | 22,2 | 5,0 |
| All | 1,2 | 12,2 | 13,8 | 4,8 | 0,7 | 8,6 | 10,3 | 3,1 |
| Enc Time[%] | 100 % |  |  |  | 100 % |  |  |  |
| Dec Time[%] | 98 % |  |  |  | 98 % |  |  |  |

Table 2. Delta PSNR for chroma.

| Test case | dPSNR-U | dPSNR-V |
| --- | --- | --- |
| Random access | -0,16 dB | -0,19 dB |
| Random access LoCo | -0,12 dB | -0,13 dB |
| Low delay | -0,22 dB | -0,26 dB |
| Low delay LoCo | -0,16 dB | -0,20 dB |

# Conclusions

Results for an MC interpolation method of chroma samples based on JCTVC-A119 have been presented. The method requires less arithmetic operations than the AVC interpolation method while resulting in 0.3% – 1.2% loss in BD-Rate and 0.12 dB – 0.26 dB reduction in chroma PSNR.

# References

- Kemal Ugur, Kenneth R. Andersson, Arild Fuldseth, “Description of video coding technology proposal by Tandberg, Nokia, Ericsson”, JCTVC-A119, JCT-VC Meeting, April 2010.

- F. Bossen, “Common Test Conditions and Software Reference Configurations,” JCT-VC, Doc JCTVC-C500, Guangzhou, CN, October, 2010.

# Patent rights declaration

Cisco may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

By accident, fractional positions f,i,k, and n are implemented differently in the TMuC/HM softare and in the  JCTVC-A119 software. However, simulations with the TMuC/HM software show that the effects of this are negligible. The results and formulas presented in this document are consistent with what is in the TMuC/HM software.