| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D224 |
| --- | --- |

| Title: | Unified transform design for HEVC with 16 bit intermediate data representation |  |  |
| --- | --- | --- | --- |
| Status: | JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | A. Fuldseth G. Bjøntegaard Phillip Pedersens vei 22 1366 Lysaker Norway | Tel: Email: | +47 92414082 arild.fuldseth@cisco.com |
| Source: | Cisco Systems Norway |  |  |

_____________________________

# Abstract

This contribution proposes a new set of unified transforms for HEVC, covering all transform sizes from 4x4 to 32x32. The proposed transforms are intended to replace the existing transforms specified in HM1.0. The proposed transforms have the following properties; 16 bit data representation before and after each transform stage (assuming 8-bit input data and IBDI disabled), no need for correction of different norms of basis vectors during quantization/dequantization, all transform sizes above 4x4 can reuse arithmetic operations for smaller transform sizes, and implementations using either pure matrix multiplication or a combination of matrix multiplication and butterfly structures are possible. The proposed transforms are implemented without transform precision extension (TPE) and compared with the existing transforms with TPE=4 (anchor conditions). The average BD-rate gains vary between 0.1% and 0.6% depending on the test condition (low complexity/high efficiency, intra/random access/low delay).

# Introduction

In HM1.0 [1], the transforms specified for TUs of size 4x4 and 8x8 are the same as in AVC. For TUs of size 16x16 and 32x32, an approximate implementation of the Chen algorithm for DCT is used. The intention of this contribution is to improve the existing transforms in several aspects.

- The current 16x16 and 32x32 transforms use a bit width of 20 or more before each stage of the inverse transform. With TPE, this number is increased correspondingly for the intermediate stage. The proposed transform design chooses intermediate scaling carefully to limit the bit width to 16 at all intermediate stages (assuming 8 bit input and IBDI disabled). The proposed transform implementations do not use TPE. Although TPE could potentially be added, internal experiments indicate only marginal gain (<0.1%) by adding TPE on top of the proposed transforms. The transform design works well with IBDI in which case the bit width would increase correspondingly.

- The current transforms have different norms for different basis vectors. This complicates quantization/dequantization since different scaling needs to be used for different transform coefficients. With the proposed transforms, the norms of the basis vectors are sufficiently close so that the same scaling can be used for all transform coefficients during quantization/dequantization. Also, the scaling is the same for all transform sizes, except for right shifts that depend on the transform size.

- Coefficients and basis vectors of the smaller transforms are a subset of coefficients and basis vectors of larger transforms. This allows for re-use of logic for multiply/adds in hardware design.

- As opposed to the current 16x16 and 32x32 transforms, the proposed transform design is specified in terms of an NxN matrix of transform coefficients and also the right shift to be used after each transform stage. This allows for more additional flexibility in implementations, such as direct matrix multiplications with SIMD instructions for the innermost loop, or multi-stage butterfly implementations utilizing symmetry properties of the basis vectors.

In addition, the proposed transforms have some properties that are also considered desirable:

- 8 bit representation of transform coefficients.

- Bit width of accumulators for matrix multiplication (before right shift and storage) does not exceed 32 bit.

- Same symmetry/anti-symmetry properties as the DCT transform.

- Scaled transform coefficients are close to the DCT transform.

- Basis vectors are almost orthogonal.

- Same quantization/dequantization scheme for all transform sizes where the multipliers depend on the value of QP and the shifts depends only on log2(N).

# Description

2.1 Basis vectors

The basis vectors of the 4x4 and 8x8 transforms are shown below. The basis vectors of the 16x16 and 32x32 transforms are shown in Appendix A. We have also designed a 64x64 transform that can be made available in the software for experimental purposes.

4x4 transform:

{64, 64, 64, 64}

{83, 36,-36,-83}

{64,-64,-64, 64}

{36,-83, 83,-36}

8x8 transform:

{64, 64, 64, 64, 64, 64, 64, 64}

{89, 75, 50, 18,-18,-50,-75,-89}

{83, 36,-36,-83,-83,-36, 36, 83}

{75,-18,-89,-50, 50, 89, 18,-75}

{64,-64,-64, 64, 64,-64,-64, 64}

{50,-89, 18, 75,-75,-18, 89,-50}

{36,-83, 83,-36,-36, 83,-83, 36}

{18,-50, 75,-89, 89,-75, 50,-18}

2.2 Intermediate scaling and quantization/dequantization.

In this subsection, intermediate scaling and quantization/dequantization are described in more detail. In addition to specifying the scaling in the dequantization and the inverse transform, we also specify the forward transform and quantizer that we have used for our simulation results.

Definitions

B	= input bit width

N 	= transform size

M	= log2(N)

Q	= f(QP%6), 	where f(x)   =   {52103,46346,41323,36792,32768,29229}, x=0,…,5

IQ 	= g(QP%6), 	where g(x)  =   {161,181,203,228,256,287}, x=0,…,5

Forward transform

The following applies to the implementation of the forward transform.

- The forward transform is specified as a matrix multiplication in two stages, horizontal and vertical.

- After the first stage, the resulting transform coefficients are shifted right with M-1 bits.

- After the second stage, the resulting transform coefficients, coeff, are shifted right with M+6 bits.

This ensures a bit width of B+8 after each transform stage.

Quantization

Quantization can be implemented as follows (RDOQ=off):

level 	= 	(coeff*Q + offset)>>(22 + QP/6 – M)

For RDOQ, the scaling and cost function calculations were adjusted accordingly.

Dequantization

The dequantizer is specified as follows:

coeffQ 	= 	(level*IQ << (QP/6) + 2N)>>(M+1)

The scaling ensures that the value of the quantized transform coefficients, coeffQ, can be represented with B+8 bits.

Inverse transform

The following applies to the implementation of the inverse transform.

- The inverse transform is specified as a matrix multiplication in two stages, horizontal and vertical.

- After the first stage, the resulting transform coefficients are shifted right with 6 bits.

- After the second stage, the resulting transform coefficients are shifted right with 13 bits.

This ensures a bit width of B+8 after each transform stage assuming the quantization scheme above. In the standard specification, it might be desirable to specify constraints on the value of level to ensure a maximum bit width after dequantization and after each stage of the inverse transform.

Quantization step size and QP

The quantizer multipliers, f(x) and dequantizer multipliers, g(x) are chosen so that

f(x) * g(x)  (1<<23), 	x = 0,1,…,5

Furthermore, assume that we have an N*N DC block with pixel values equal to X.  If this block is input to an orthonormal transformation containing a DC basis vector (e.g. the Discrete Cosine Transform), the value of the DC coefficient is equal to X*N.  With the proposed transforms and quantization/ dequantiztion scheme, the same relationship between input pixel values and the value of level applies when QP = 4. In other words, if all pixel values in an NxN block are equal to X, and QP=4, this will result in a DC coefficient with level = X*N after quantization. The range of QP values maps to approximately the same range of quantization step sizes as in HM1.0.

# Complexity analysis

The proposed transforms can be implemented in two ways; matrix multiplications, and combined butterfly/matrix multiplication.

3.1 Matrix multiplication

With straightforward matrix multiplication, the number of multiply/adds for each transform stage is equal to N3. Note however, that matrix multiplication is particularly suited for SIMD instructions. Assuming a SIMD width of N, the matrix multiplication can be implemented using N2 SIMD instructions.

3.2 Combined butterfly/matrix multiplications

The basis vectors shown in section 2 and in Appendix A have some nice symmetry properties that are inherited from the DCT transform:

- Even rows with index 0,2,4,… are symmetric with a symmetry point before coefficient N/2.

- Odd rows with index 1,3,5,7,…are anti-symmetric with a symmetry point before coefficient N/2.

- Even rows with index 0,4,8,… have additional symmetry point before elements N/4 and 3N/4.

- Even rows with index 2,6,10,…have additional anti-symmetry points before elements N/4 and 3N/4.

For larger transforms this pattern repeats itself with additional symmetry and anti-symmetry points before coefficients n*N/8, n*N/16 etc. These properties can be utilized by a “butterfly-like” implementation to reduce the number of multiply/adds.

In our software both methods are supported, matrix multiplications for readability and a butterfly-like implementation for speed.

3.3 Bit width

With 8-bit input and IBDI disabled, the bit width of all intermediate transform stages is 16. On a large number of architectures, 16 bit represents a “magic” threshold since 17 bit or more normally needs to be stored in units of 32 bits.

The multiply-accumulation of matrix implementation stays within 32 bits before the right shift. This is important as many registers are limited to 32 bit width.

# Results

Table 1 shows BD-rate results for the new transforms using HM0.9 as the anchor and using the common coding conditions [2]. As can be seen from the table, there is a small but consistent gain, ranging from 0.1% to 0.6%. Note however, that the main advantages of the proposed transforms are related to implementation benefits rather than compression gain. Also, note that the proposed transforms do not use TPE while TPE=4 us used for the anchor.

In order to achieve consistent timing results, 9 frames for each sequence/QP combination were encoded and decoded serially on the same computer. The encode- and decode-times where then normalized by multiplying with N/9, where N is the number of frames in the full sequence.

Table 1. BD-rate results for new transforms

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0,5 | -0,3 | -0,3 | -1,4 | -0,7 | -0,3 |
| Class B | -0,3 | 0,1 | 0,1 | -0,5 | -0,4 | -0,5 |
| Class C | -0,1 | 0,2 | 0,1 | -0,3 | -0,4 | -0,6 |
| Class D | -0,1 | 0,1 | 0,2 | -0,3 | -0,5 | -0,5 |
| Class E | -0,2 | 0,3 | -0,1 | -0,8 | 0,0 | -0,1 |
| All | -0,2 | 0,1 | 0,0 | -0,6 | -0,4 | -0,4 |
| Enc Time[%] | 107 % |  |  | 100 % |  |  |
| Dec Time[%] | 100 % |  |  | 100 % |  |  |
|  | Random access |  |  | Random access LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0,3 | 0,3 | 0,5 | -0,5 | 0,1 | 0,4 |
| Class B | -0,1 | 0,5 | 0,3 | -0,2 | 0,1 | 0,3 |
| Class C | -0,1 | 0,2 | 0,2 | -0,1 | 0,2 | 0,0 |
| Class D | 0,0 | 0,3 | 0,5 | 0,0 | 0,0 | 0,2 |
| Class E |  |  |  |  |  |  |
| All | -0,1 | 0,3 | 0,4 | -0,2 | 0,1 | 0,2 |
| Enc Time[%] | 105 % |  |  | 101 % |  |  |
| Dec Time[%] | 101 % |  |  | 99 % |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0,1 | 0,0 | 0,0 | -0,2 | -0,2 | -0,2 |
| Class C | -0,2 | 0,0 | 0,0 | -0,1 | -0,3 | -0,2 |
| Class D | 0,0 | -0,3 | 0,0 | -0,1 | 0,2 | -0,1 |
| Class E | -0,3 | 1,2 | -0,2 | -0,2 | -0,2 | -0,1 |
| All | -0,1 | 0,2 | 0,0 | -0,1 | -0,1 | -0,1 |
| Enc Time[%] | 105 % |  |  | 100 % |  |  |
| Dec Time[%] | 99 % |  |  | 99 % |  |  |

# Conclusions

New unified integer transforms for HEVC have been proposed. The new transforms have several implementation benefits, including:

- 16 bit data representation before and after each transform stage when IBDI is disabled.

- No need for correction of different norms of basis vectors during quantization/dequantization.

- Reusability of small transforms in larger transforms.

- Allows both matrix multiplication and butterfly structure.

BD-Rate gains between 0.1% and 0.6% compared to the anchor are observed.

# References

- T. Wiegand, W.-J. Han, J.-R. Ohm, G. J. Sullivan, “High Efficiency Video Coding (HEVC) text specification Working Draft 1.”, JCTVC-C403, Guangzhou, October, 2010.

- F. Bossen, “Common Test Conditions and Software Reference Configurations,” JCT-VC, Doc JCTVC-C500, Guangzhou, CN, October, 2010.

# Patent rights declaration

Cisco may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Appendix A

16x16 transform

{64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64}

{90 87 80 70 57 43 25  9 -9-25-43-57-70-80-87-90}

{89 75 50 18-18-50-75-89-89-75-50-18 18 50 75 89}

{87 57  9-43-80-90-70-25 25 70 90 80 43 -9-57-87}

{83 36-36-83-83-36 36 83 83 36-36-83-83-36 36 83}

{80  9-70-87-25 57 90 43-43-90-57 25 87 70 -9-80}

{75-18-89-50 50 89 18-75-75 18 89 50-50-89-18 75}

{70-43-87  9 90 25-80-57 57 80-25-90 -9 87 43-70}

{64-64-64 64 64-64-64 64 64-64-64 64 64-64-64 64}

{57-80-25 90 -9-87 43 70-70-43 87  9-90 25 80-57}

{50-89 18 75-75-18 89-50-50 89-18-75 75 18-89 50}

{43-90 57 25-87 70  9-80 80 -9-70 87-25-57 90-43}

{36-83 83-36-36 83-83 36 36-83 83-36-36 83-83 36}

{25-70 90-80 43  9-57 87-87 57 -9-43 80-90 70-25}

{18-50 75-89 89-75 50-18-18 50-75 89-89 75-50 18}

{ 9-25 43-57 70-80 87-90 90-87 80-70 57-43 25 -9}

32x32 transform

{64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64}

{90 90 88 85 82 78 73 67 61 54 46 38 31 22 13  4 -4-13-22-31-38-46-54-61-67-73-78-82-85-88-90-90}

{90 87 80 70 57 43 25  9 -9-25-43-57-70-80-87-90-90-87-80-70-57-43-25 -9  9 25 43 57 70 80 87 90}

{90 82 67 46 22 -4-31-54-73-85-90-88-78-61-38-13 13 38 61 78 88 90 85 73 54 31  4-22-46-67-82-90}

{89 75 50 18-18-50-75-89-89-75-50-18 18 50 75 89 89 75 50 18-18-50-75-89-89-75-50-18 18 50 75 89}

{88 67 31-13-54-82-90-78-46 -4 38 73 90 85 61 22-22-61-85-90-73-38  4 46 78 90 82 54 13-31-67-88}

{87 57  9-43-80-90-70-25 25 70 90 80 43 -9-57-87-87-57 -9 43 80 90 70 25-25-70-90-80-43  9 57 87}

{85 46-13-67-90-73-22 38 82 88 54 -4-61-90-78-31 31 78 90 61  4-54-88-82-38 22 73 90 67 13-46-85}

{83 36-36-83-83-36 36 83 83 36-36-83-83-36 36 83 83 36-36-83-83-36 36 83 83 36-36-83-83-36 36 83}

{82 22-54-90-61 13 78 85 31-46-90-67  4 73 88 38-38-88-73 -4 67 90 46-31-85-78-13 61 90 54-22-82}

{80  9-70-87-25 57 90 43-43-90-57 25 87 70 -9-80-80 -9 70 87 25-57-90-43 43 90 57-25-87-70  9 80}

{78 -4-82-73 13 85 67-22-88-61 31 90 54-38-90-46 46 90 38-54-90-31 61 88 22-67-85-13 73 82  4-78}

{75-18-89-50 50 89 18-75-75 18 89 50-50-89-18 75 75-18-89-50 50 89 18-75-75 18 89 50-50-89-18 75}

{73-31-90-22 78 67-38-90-13 82 61-46-88 -4 85 54-54-85  4 88 46-61-82 13 90 38-67-78 22 90 31-73}

{70-43-87  9 90 25-80-57 57 80-25-90 -9 87 43-70-70 43 87 -9-90-25 80 57-57-80 25 90  9-87-43 70}

{67-54-78 38 85-22-90  4 90 13-88-31 82 46-73-61 61 73-46-82 31 88-13-90 -4 90 22-85-38 78 54-67}

{64-64-64 64 64-64-64 64 64-64-64 64 64-64-64 64 64-64-64 64 64-64-64 64 64-64-64 64 64-64-64 64}

{61-73-46 82 31-88-13 90 -4-90 22 85-38-78 54 67-67-54 78 38-85-22 90  4-90 13 88-31-82 46 73-61}

{57-80-25 90 -9-87 43 70-70-43 87  9-90 25 80-57-57 80 25-90  9 87-43-70 70 43-87 -9 90-25-80 57}

{54-85 -4 88-46-61 82 13-90 38 67-78-22 90-31-73 73 31-90 22 78-67-38 90-13-82 61 46-88  4 85-54}

{50-89 18 75-75-18 89-50-50 89-18-75 75 18-89 50 50-89 18 75-75-18 89-50-50 89-18-75 75 18-89 50}

{46-90 38 54-90 31 61-88 22 67-85 13 73-82  4 78-78 -4 82-73-13 85-67-22 88-61-31 90-54-38 90-46}

{43-90 57 25-87 70  9-80 80 -9-70 87-25-57 90-43-43 90-57-25 87-70 -9 80-80  9 70-87 25 57-90 43}

{38-88 73 -4-67 90-46-31 85-78 13 61-90 54 22-82 82-22-54 90-61-13 78-85 31 46-90 67  4-73 88-38}

{36-83 83-36-36 83-83 36 36-83 83-36-36 83-83 36 36-83 83-36-36 83-83 36 36-83 83-36-36 83-83 36}

{31-78 90-61  4 54-88 82-38-22 73-90 67-13-46 85-85 46 13-67 90-73 22 38-82 88-54 -4 61-90 78-31}

{25-70 90-80 43  9-57 87-87 57 -9-43 80-90 70-25-25 70-90 80-43 -9 57-87 87-57  9 43-80 90-70 25}

{22-61 85-90 73-38 -4 46-78 90-82 54-13-31 67-88 88-67 31 13-54 82-90 78-46  4 38-73 90-85 61-22}

{18-50 75-89 89-75 50-18-18 50-75 89-89 75-50 18 18-50 75-89 89-75 50-18-18 50-75 89-89 75-50 18}

{13-38 61-78 88-90 85-73 54-31  4 22-46 67-82 90-90 82-67 46-22 -4 31-54 73-85 90-88 78-61 38-13}

{ 9-25 43-57 70-80 87-90 90-87 80-70 57-43 25 -9 -9 25-43 57-70 80-87 90-90 87-80 70-57 43-25  9}

{ 4-13 22-31 38-46 54-61 67-73 78-82 85-88 90-90 90-90 88-85 82-78 73-67 61-54 46-38 31-22 13 -4}