| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D225 |
| --- | --- |

| Title: | Directional interpolation filters for luma samples using 8 filter coefficients |  |  |
| --- | --- | --- | --- |
| Status: | JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | A. Fuldseth G. Bjøntegaard Phillip Pedersens vei 22 1366 Lysaker Norway | Tel: Email: | +47 92414082 arild.fuldseth@cisco.com |
| Source: | Cisco Systems Norway |  |  |

_____________________________

# Abstract

This contribution presents results for interpolation filtering of luma samples using directional filters with 8 coefficients. The directional interpolation filters presented here represent a straightforward extension of the interpolation filters in JCTVC-A119 (TENTM) where 6 filter coefficients were used. These interpolation filters are also used in the current low complexity configuration of HM 1.0. When using 8 filter coefficients instead of using 6 filter coefficients, BD-rate gains of 2.6% and 2.4% over the 6-tap filters are observed for low delay and random access configurations respectively.

# Introduction

In the TENTM proposal to the HEVC CfP, JCTVC-A119, a low complexity interpolation method for motion compensation of luma samples was used [1]. This interpolation method uses a 6-tap directional filter and was adopted for the low complexity operating point in HM1.0. This document presents results for an 8-tap extension of the 6-tap directional filters. BD-rate results for the low complexity operating point comparing 6-tap and 8-tap filters are provided together with a complexity analysis.

# Description

The 6-tap filters used in JCTVC-A119 and HM1.0 uses the following set of filter coefficients:

{3,-15,111, 37,-10, 2}/128

{3,-17, 78, 78,-17, 3}/128
    {2,-10, 37,111,-15, 3}/128

For the 8-tap version we use the following set of filter coefficients:

{-4,16,-40,228, 76,-28, 12, -4}/256

{-4,20,-48,160,160,-48, 20, -4}/256
    {-4,12,-28, 76,228,-40, 16, -4}/256

These filter coefficients are the same as those provided in the HM v0.9 software for 8-tap DCT-IF.

# Complexity analysis

On many platforms, SIMD instructions are supported. This allows for processing of multiple (typically 4, 8, or 16) bytes in parallel. This means that a 6-tap filter does not fully utilize the capabilities of typical SIMD architectures. On the other hand, an 8-tap filter has the potential to take full advantage of both 4- and 8-byte parallelism. This provides a useful trade-off between complexity and performance.

# Results

Table 1 shows BD-rate results for the two low complexity test cases. For simulations we used the common coding conditions [2] and HM0.9 but including bugfix 110. This is consistent with the simulation conditions used in CE3.

Table 1. BD-rate for low complexity test cases.

| Low delay |  |  |  |
| --- | --- | --- | --- |
| Sequence | BDR | BDR-low | BDR-high |
| BasketballPass | -1,1 | -1,0 | -1,0 |
| BQSquare | -12,9 | -15,5 | -9,5 |
| BlowingBubbles | -6,2 | -4,7 | -6,8 |
| RaceHorses | -0,8 | -0,3 | -1,2 |
| BasketBallDrill | -2,2 | -1,8 | -2,3 |
| BQMall | -3,0 | -2,4 | -3,2 |
| PartyScene | -7,8 | -6,7 | -7,8 |
| RaceHorses | -0,5 | -0,5 | -0,2 |
| Kimono | 0,0 | 0,0 | -0,1 |
| ParkScene | -1,2 | 0,0 | -2,3 |
| Cactus | -0,8 | -1,0 | -0,4 |
| BasketballDrive | -1,1 | -1,3 | -0,9 |
| BQTerrace | -3,0 | -4,8 | -1,0 |
| Vidyo1 | -0,7 | -0,3 | -0,8 |
| Vidyo3 | 0,9 | 1,1 | 0,9 |
| Vidyo4 | -1,0 | -0,3 | -1,4 |
| class D average | -5,2 | -5,4 | -4,6 |
| class C average | -3,3 | -2,9 | -3,4 |
| class B average | -1,2 | -1,4 | -0,9 |
| class E average | -0,3 | 0,2 | -0,5 |
| Average | -2,6 | -2,5 | -2,4 |

| Random Access |  |  |  |
| --- | --- | --- | --- |
| Sequence | BDR | BDR-low | BDR-high |
| BasketballPass | -0,9 | -0,5 | -1,1 |
| BQSquare | -15,3 | -12,2 | -16,1 |
| BlowingBubbles | -5,1 | -3,5 | -6,3 |
| RaceHorses | -0,2 | 0,1 | -0,5 |
| BasketBallDrill | -1,3 | -1,1 | -1,5 |
| BQMall | -2,6 | -2,0 | -3,1 |
| PartyScene | -8,2 | -6,5 | -9,2 |
| RaceHorses | -0,1 | -0,2 | 0,0 |
| Kimono | 0,0 | 0,1 | -0,1 |
| ParkScene | -0,8 | -0,3 | -1,4 |
| Cactus | -0,1 | -0,2 | 0,2 |
| BasketballDrive | -1,0 | -0,9 | -1,0 |
| BQTerrace | -1,4 | -2,8 | 0,3 |
| Traffic | 0,1 | 0,1 | 0,2 |
| PeopleOnStreet | 0,4 | 0,3 | 0,5 |
| class D average | -5,4 | -4,1 | -6,0 |
| class C average | -3,0 | -2,4 | -3,5 |
| class B average | -0,7 | -0,8 | -0,4 |
| class A average | 0,2 | 0,2 | 0,3 |
| Average | -2,4 | -2,0 | -2,6 |

# Conclusions

Results for increasing the number of filter coefficients for the directional interpolation filter have been presented. The average gain is 2.6% and 2.4% for the low delay and random access configurations respectively. Due to better complexity compression trade-offs for most SIMD architectures, we propose to use the 8-tap directional filter for the low complexity operating point in the HM.

# References

- Kemal Ugur, Kenneth R. Andersson, Arild Fuldseth, “Description of video coding technology proposal by Tandberg, Nokia, Ericsson”, JCTVC-A119, JCT-VC Meeting, April 2010.

- F. Bossen, “Common Test Conditions and Software Reference Configurations,” JCT-VC, Doc JCTVC-C500, Guangzhou, CN, October, 2010.

# Patent rights declaration

Cisco may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).