| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D226 |
| --- | --- |

| Title: | Reducing the table sizes for LCEC |  |  |
| --- | --- | --- | --- |
| Status: | JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | A. Fuldseth Phillip Pedersens vei 22 1366 Lysaker Norway | Tel: Email: | +47 92414082 arild.fuldseth@cisco.com |
| Source: | Cisco Systems Norway |  |  |

_____________________________

# Abstract

This contribution presents results for reduced number of tables in LCEC. In particular, the total table size for the last_pos_and_level syntax element is reduced from 1120 bytes to 352 bytes. The associated BD-Rate loss is 0.7%, 0.2%, and 0.1% for intra, random access, and low delay configurations respectively.

# Introduction

In LCEC, syntax elements are mapped to a binary codeword in two steps. First, a syntax element is mapped to an index value n by use of an adaptive ranking table. Second, n is mapped to a binary codeword. The second mapping can be implemented arithmetically since structured (UVLC-like) VLC codes are used. Also, the length of the binary codeword is non-decreasing with increasing values of n. This implies that it is desirable to use a ranking table where the most probable syntax elements are mapped to low values of n. In order to adapt to the local statistics of the video content, the ranking tables are adaptive by a swapping technique as described in [1]. This document presents results on reducing the number of ranking tables for LCEC.

# Description

For LCEC coefficient coding the last_pos_and_level syntax element contains the following information:

- Position of the last significant position in scan order

- Whether or not the value of the last significant coefficient is larger than 1 in magnitude

This implies that for an NxN block the last_pos_and_level syntax element can have 2*N*N possible values. Note also that in HM1.0, the number of coefficients to be coded is either 4x4 or 8x8, since partial transforms are used for larger transform sizes.

Furthermore, in HM1.0, different ranking tables are used for different block types:

4x4 blocks (table size = 32)

- Intra blocks

- Inter blocks in P slices

- Inter blocks in B slices

8x8 blocks (table size = 128)

- U

- V

- Y, intra blocks, 8x8 transform

- Y, inter blocks in P slices, 8x8 transform

- Y, inter blocks in B slices, 8x8 transform

- Y, intra blocks, >8x8 partial transform

- Y, inter blocks in P slices, >8x8 partial transform

- Y, inter blocks in B slices, >8x8 partial transform

Note that with the current common coding conditions the two tables corresponding to P slices is never used.

In the current contribution, the number of tables is reduced as follows:

4x4 blocks (table size = 32)

- U, V

- Y, intra blocks

- Y, inter blocks

8x8 blocks (table size = 128)

- U, V

- Y

This means that the number of ranking tables for 4x4 blocks is maintained, although used in a different way. The number of tables for 8x8 blocks, however, is reduced from 8 to 2.

Overall, the total table size for the last_pos_and_level syntax element is reduced from 3*32+8*128 = 1120 bytes to 3*32 + 2*128 = 352 bytes.

# Results

Table 1 shows BD-rate results for the three low complexity test cases using HM0.9 as the anchor and using the common coding conditions [2]. The average BD-Rate penalty is 0.7%, 0.2%, and 0.1% for intra, random access, and low delay configurations respectively.

Table 1. BD-rate results.

|  | Intra LoCo |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 1,3 | 1,4 | 1,8 |
| Class B | 1,1 | 1,2 | 2,0 |
| Class C | 0,2 | 1,5 | 1,7 |
| Class D | 0,2 | 1,6 | 1,6 |
| Class E | 0,7 | 1,6 | 1,1 |
| All | 0,7 | 1,4 | 1,6 |
|  | Random access LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0,5 | 0,7 | 0,9 |
| Class B | 0,5 | 0,7 | 1,6 |
| Class C | 0,1 | 0,6 | 1,0 |
| Class D | 0,0 | 0,3 | 0,6 |
| Class E |  |  |  |
| All | 0,2 | 0,6 | 1,1 |
|  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |
| Class B | 0,3 | 0,6 | 1,8 |
| Class C | 0,0 | 0,3 | 0,9 |
| Class D | -0,1 | 0,3 | 0,1 |
| Class E | 0,3 | 0,8 | 0,6 |
| All | 0,1 | 0,5 | 0,9 |

# Conclusions

The results presented in this contribution shows that the table sizes used for adaptive ranking tables in LCEC can be reduced significantly with only a marginal loss in compression efficiency. This encourages further study on the topic.

# References

- Kemal Ugur, Kenneth R. Andersson, Arild Fuldseth, “Description of video coding technology proposal by Tandberg, Nokia, Ericsson”, JCTVC-A119, JCT-VC Meeting, April 2010.

- F. Bossen, “Common Test Conditions and Software Reference Configurations,” JCT-VC, Doc JCTVC-C500, Guangzhou, CN, October, 2010.

# Patent rights declaration

Cisco may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

In the current draft HM specification (JCTVC-C403), the actual name of this syntax element is yet to be determined.