| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D227 |
| --- | --- |

| Title: | Replacing slices with tiles for high level parallelism |  |  |
| --- | --- | --- | --- |
| Status: | JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | A. Fuldseth Phillip Pedersens vei 22 1366 Lysaker Norway | Tel: Email: | +47 92414082 arild.fuldseth@cisco.com |
| Source: | Cisco Systems Norway |  |  |

_____________________________

# Abstract

This contribution introduces tiles as an alternative to slices for better support for high level parallelism in HEVC. While slices follows the raster scan order of LCUs, tiles have a fixed rectangular shape that is signaled in the sequence parameter set or in the picture parameter set. Furthermore, tiles come without the overhead associated with slice headers, and it is argued that it allows for better load balancing, lower delay, and more fine-grained parallelism than slices. Finally, tiles can co-exist with slices, are optional in the encoder and have a negligible impact on the decoder design. Experimental results using the low complexity low delay configuration with one tile/slice per LCU row, show that the BD-rate gain when using tiles instead of slices is 2.1% on average, and 6.3% for the class E sequences.

# Introduction

This contribution discusses high-level parallelism that is suitable for encoding and decoding on multi-core architectures with shared memory for e.g. storage of reference pictures. In the AVC standard this is typically achieved by use of slice, where a slice is defined as a number of consecutive macroblocks in raster-scan order. For implementations of the AVC standard, slices are used for at least two purposes:

- Packetization

- Parallel processing

Since slices are independently decodable, they are useful both for transport over a network that is subject to packet loss and to parallel processing.

However, slices in AVC also have several properties that might be useful for packetization but less desirable for parallel processing:

- Slice header overhead

- Dependency on the raster scan order

For better parallel processing in HEVC, it is proposed to use slices for packetization (as in AVC) while using tiles for parallel processing.

# Description

2.1 Definition

A tile can be defined as follows:

-	A tile has a rectangular shape.

- A tile contains a fixed number of LCUs that is constant for the entire frame (except possibly at frame boundaries).

- The width and height (M, N) of a tile (in units of LCUs) are signaled in the sequence parameters set or in the picture parameter set.

- The dependency breaks at tile boundaries are the same as those at slice boundaries.

- Whether or not to do deblocking across tile boundaries can be signaled in the slice header in the same way as in AVC. (It might be useful to have separate signaling for horizontal and vertical boundaries though.)

- The use of tiles does not alter the raster-scan transmission order of the bitstream. If the height of a tile is more than one LCU, this requires some reshuffling of bits on the encoder side.

- Tiles can co-exist with slices in the same frame.

- Tiles are optional on the encoder side.

- Since the use of tiles does not alter the transmission order of the compressed bits, the only impact on the decoder is additional dependency breaks at tile boundaries.

2.2 Advantages of tiles in parallel processing

The usage of tiles for high level parallel processing has several advantages as compared to slices:

- No overhead due to slice headers.

- More flexibility of frame partitioning independent of the raster-scan order.

- The mechanism for partition of a frame based on packetization needs and parallelization needs are decoupled.

In particular, the definition of tiles allows for dependency breaks at columns boundaries. This has several additional advantages:

- Dividing a frame vertically gives smaller penalty on compression performance since a vertical boundary is shorter than a horizontal boundary (assuming 16:9 aspect  ratio).

- Parallelization by columns reduces the delay in cases where pixel data arrives one LCU row at a time from the camera, and all cores can start to work immediately as a new LCU row arrives.

- Parallelization by columns provides finer-grained parallelism and better load balancing. This is because cores can work independently on chunks of size M x 1 (in units of LCUs).

Finally, dependency breaks across column boundaries have several advantages that are not necessarily associated with parallel processing:

- Flexibility in the encoder to do “stitching” of arbitrarily shaped rectangles, i.e. change the spatial position of sub-pictures by manipulations in the compressed domain.

- Encoding by smaller widths might reduce the memory bandwidth and internal memory. This is true even for single-core implementations.

- More efficient packetization into (almost) fixed-sized packets (e.g. 1400 bytes). The packetizer can assemble independent chunks of compressed data (one per column/row) into one packet without any backward dependency to the encoder process.  More specifically, the packetizer can form slices out of a complete integer number of concatenated encoded tiles (by appending a slice header). Therefore the process becomes

    - Encode frame using fairly fine grained tiles (giving parallelism / cache efficiency / etc)

    - Packetizer concatenates as many as fit into each 1400 byte packet and append a slice header to form an independently decodable packet.

2.3 Tiles and slice groups

The AVC standard also supports slice groups (i.e. FMO) that can be used to specify partitioning of a frame into arbitrarily shaped partitions. This means that slice groups have some similarities to tiles. However, there are several important differences between tiles and slice groups:

- Two slice groups can not belong to the same slice. This implies that for every slice group, there need to be at least one slice with the associated slice header overhead.

- Slice groups change the transmission order from raster-scan order within the frame to raster-scan order within the slice group. This places an undesirable burden on the decoder.

- Finally, the AVC standard specifies a maximum of 8 slice groups per frame.

Comparison between tiles, slices and slice groups

The table below compares tiles, slices and slice groups. The green cells indicate features that are considered desirable for parallelization and for decoder design.

Table 1. Comparison of tiles, slices and slice groups

| Feature | Tiles | Slices | Slice groups |
| --- | --- | --- | --- |
| Slice header | No | Yes | Yes |
| Raster scan order within the picture | Yes | Yes | No |
| Support of dependency breaks at column boundaries | Yes | No | Yes |
| Maximum umber of partitions supported | >8 | >8 | 8 |

# Results

For experimental purposes the software produced by the AHG on slices were used. In order to study the benefit of not having slice headers in tiles, the software was modified to send a slice header only for the first slice in each frame. This would emulate the situation where there is one slice but multiple tiles per picture. The encoder was then configured to produce one slice or tile per LCU row. For the sake of simplicity, no column breaks were used. However, we believe this experiment is sufficient to show the benefit of tiles when the number of tiles is significantly higher than the number of slices.

The software was run in two different modes:

- Slice mode: 	Slice header as usual for every LCU row (e.g. unmodified source code)

- Tile mode:	Slice header only for the first slice of each frame (e.g. modified source code).

Table 2 shows BD-rate results for the two modes, both using HM0.9 without slices as anchor. As can be seen from the table, there are significant bitrate saving associated with eliminating the slice headers.

Table 3 shows BD-rate results comparing slice mode and tile mode directly. On average, tiles result in bitrate savings of 2.1%. However, for the class E sequences the gain of using the tile mode is 6.3% over the whole QP range (BDR) and 10.3% at low bit rates (BDR-low). This indicates that eliminating the slice headers for high level parallelization is particularly useful for content with a lot of static background (e.g. video conferencing).

Table 2. BD-rate results for slice mode and tile mode.

|  | SliceMode |  |  | TileMode |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Sequence | BDR | BDR-low | BDR-high | BDR | BDR-low | BDR-high |
| BasketballPass | 3,9 | 6,0 | 2,5 | 2,7 | 3,8 | 2,0 |
| BQSquare | 4,2 | 6,6 | 2,7 | 3,0 | 3,8 | 2,4 |
| BlowingBubbles | 3,6 | 6,1 | 2,2 | 2,5 | 3,7 | 1,9 |
| RaceHorses | 3,7 | 5,7 | 2,4 | 2,4 | 3,2 | 1,9 |
| BasketBallDrill | 4,7 | 7,0 | 2,9 | 3,2 | 4,4 | 2,3 |
| BQMall | 5,7 | 8,5 | 3,4 | 3,9 | 5,3 | 2,7 |
| PartyScene | 3,2 | 4,7 | 2,2 | 2,7 | 3,5 | 2,0 |
| RaceHorses | 4,3 | 6,3 | 2,7 | 3,3 | 4,4 | 2,4 |
| Kimono | 5,4 | 7,3 | 3,4 | 3,9 | 4,9 | 2,8 |
| ParkScene | 4,5 | 6,5 | 3,0 | 3,3 | 4,2 | 2,6 |
| Cactus | 3,8 | 5,4 | 2,3 | 2,5 | 3,2 | 1,9 |
| BasketballDrive | 6,2 | 7,8 | 4,6 | 5,2 | 6,1 | 4,2 |
| BQTerrace | 6,5 | 11,0 | 4,5 | 5,0 | 7,3 | 4,3 |
| Vidyo1 | 13,7 | 21,9 | 6,2 | 6,1 | 8,9 | 3,4 |
| Vidyo3 | 11,8 | 18,4 | 5,7 | 5,7 | 7,8 | 3,5 |
| Vidyo4 | 14,7 | 24,5 | 6,5 | 7,1 | 10,6 | 4,0 |
| class D average | 3,9 | 6,1 | 2,4 | 2,7 | 3,6 | 2,0 |
| class C average | 4,5 | 6,6 | 2,8 | 3,3 | 4,4 | 2,4 |
| class B average | 5,3 | 7,6 | 3,5 | 4,0 | 5,1 | 3,2 |
| class E average | 13,4 | 21,6 | 6,1 | 6,3 | 9,1 | 3,6 |
| Average | 6,2 | 9,6 | 3,6 | 3,9 | 5,3 | 2,8 |

Table 3. Tile mode vs. slice mode

| Sequence | BDR | BDR-low | BDR-high |
| --- | --- | --- | --- |
| BasketballPass | -1,1 | -2,1 | -0,5 |
| BQSquare | -1,2 | -2,6 | -0,3 |
| BlowingBubbles | -1,1 | -2,2 | -0,3 |
| RaceHorses | -1,3 | -2,4 | -0,5 |
| BasketBallDrill | -1,4 | -2,5 | -0,6 |
| BQMall | -1,7 | -3,0 | -0,7 |
| PartyScene | -0,6 | -1,2 | -0,2 |
| RaceHorses | -0,9 | -1,8 | -0,3 |
| Kimono | -1,4 | -2,2 | -0,6 |
| ParkScene | -1,1 | -2,2 | -0,3 |
| Cactus | -1,2 | -2,1 | -0,4 |
| BasketballDrive | -1,0 | -1,6 | -0,3 |
| BQTerrace | -1,4 | -3,3 | -0,2 |
| Vidyo1 | -6,6 | -10,7 | -2,7 |
| Vidyo3 | -5,5 | -9,0 | -2,0 |
| Vidyo4 | -6,7 | -11,2 | -2,4 |
| class D average | -1,2 | -2,3 | -0,4 |
| class C average | -1,1 | -2,1 | -0,4 |
| class B average | -1,2 | -2,3 | -0,4 |
| class E average | -6,3 | -10,3 | -2,4 |
| Average | -2,1 | -3,8 | -0,8 |

# Conclusions

In this document, tiles have been proposed as an alternative to slices for high level parallel processing on multi-core systems with shared memory. Several benefits of using tiles have been identified and experimental results show significant BD-rate gains associated with not sending a slice header with every tile. It is proposed to continue the study of tiles in coordination with other proposals for frame-level parallel processing.

# Patent rights declaration

Cisco may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

With the current software, the following elements of each slice header are required for proper decoding: NAL unit header, picture order count, and CU address of each slice. These elements were included in every slice/tile. However, since these elements represent a fixed number of bits per frame, this can be compensated for when calculating the total bitrate.