| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D228 |
| --- | --- |

| Title: | Cross-check report on improved coefficient coding for LCEC |  |  |
| --- | --- | --- | --- |
| Status: | JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | A. Fuldseth Phillip Pedersens vei 22 1366 Lysaker Norway | Tel: Email: | +47 92414082 arild.fuldseth@cisco.com |
| Source: | Cisco Systems Norway |  |  |

_____________________________

# Abstract

This contribution presents results of cross-check of improved coefficient coding for LCEC proposed by Qualcomm (JCTVC-D374).

# Introduction

This document reports on cross-check of the improved coefficient coding for LCEC using two different settings of macros:

Setting 1:

#define QC_MOD_LCEC        1   // modified LCEC for coeff. coding

#define QC_COEFF_SCAN      1   // enable coefficient scan for LCEC

#define QC_MOD_LCEC_16x16  0   // extend modified LCEC to 16x16 coeff. Coding

Setting 2:

#define QC_MOD_LCEC        1   // modified LCEC for coeff. coding

#define QC_COEFF_SCAN      1   // enable coefficient scan for LCEC

#define QC_MOD_LCEC_16x16  1   // extend modified LCEC to 16x16 coeff. Coding

# Results

Table 1 and Table 2 show BD-rate results setting 1 and setting 2 respectively. The results match those of Qualcomm. The tables also show encode- and decode-times. In order to achieve consistent timing results, 9 frames for each sequence/QP combination were encoded and decoded serially on the same computer. The encode- and decode-times where then normalized by multiplying with N/9, where N is the number of frames in the full sequence.

Table 1. BD-rate results for setting 1.

|  | Intra LoCo |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -3,2 | -2,5 | -2,5 |
| Class B | -3,5 | -3,6 | -3,6 |
| Class C | -3,0 | -2,3 | -2,4 |
| Class D | -3,2 | -2,3 | -2,4 |
| Class E | -3,4 | -3,0 | -2,8 |
| All | -3,3 | -2,8 | -2,8 |
| Enc Time[%] | 105 % |  |  |
| Dec Time[%] | 101 % |  |  |
|  | Random access LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1,3 | -1,2 | -1,2 |
| Class B | -1,6 | -2,2 | -2,4 |
| Class C | -1,1 | -1,7 | -1,7 |
| Class D | -1,0 | -2,0 | -2,0 |
| Class E |  |  |  |
| All | -1,3 | -1,9 | -1,9 |
| Enc Time[%] | 101 % |  |  |
| Dec Time[%] | 97 % |  |  |
|  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |
| Class B | -0,4 | -1,9 | -2,2 |
| Class C | -0,2 | -2,2 | -2,0 |
| Class D | -0,3 | -2,7 | -2,7 |
| Class E | -0,4 | -1,1 | -0,9 |
| All | -0,3 | -2,0 | -2,0 |
| Enc Time[%] | 101 % |  |  |
| Dec Time[%] | 100 % |  |  |

Table 2. BD-rate results for setting 2.

|  | Intra LoCo |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -6,3 | -1,8 | -2,0 |
| Class B | -6,3 | -3,1 | -3,6 |
| Class C | -4,0 | -2,3 | -2,6 |
| Class D | -4,1 | -2,6 | -2,8 |
| Class E | -5,9 | -4,5 | -3,9 |
| All | -5,2 | -2,9 | -3,1 |
| Enc Time[%] | 109 % |  |  |
| Dec Time[%] | 99 % |  |  |
|  | Random access LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -2,4 | 0,1 | -0,4 |
| Class B | -3,1 | -1,0 | -1,6 |
| Class C | -1,7 | -1,0 | -1,3 |
| Class D | -1,5 | -2,1 | -2,3 |
| Class E |  |  |  |
| All | -2,2 | -1,2 | -1,5 |
| Enc Time[%] | 104 % |  |  |
| Dec Time[%] | 101 % |  |  |
|  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |
| Class B | -2,9 | 1,4 | 0,5 |
| Class C | -1,9 | -1,1 | -1,2 |
| Class D | -1,9 | -2,9 | -2,6 |
| Class E | -2,3 | 0,2 | 0,7 |
| All | -2,3 | -0,5 | -0,7 |
| Enc Time[%] | 103 % |  |  |
| Dec Time[%] | 99 % |  |  |