| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4rd Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D229r1 WG11 Number: m18989 |
| --- | --- |

| Title: | CE2: Summary of Core Experiment 2 on Flexible Motion Partitioning |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Edouard Francois (CE coordinator), Xiaozhen Zheng, Peisong Chen (vice-coordinators) | Tel: Email: | +33-2-99-27-32-62 edouard.francois@technicolor.com peisongc@qualcomm.com zhengxiaozhen@huawei.com |
| Source: | Technicolor, Qualcomm, Huawei & HiSilicon Technologies |  |  |

_____________________________

# Abstract

This document summarizes the activity of CE2 related to Flexible Motion Partitioning. The description of the experiment can be found in JCTVC-C502r1. In this CE, three tools for motion partitioning have been studied: Asymmetric Motion Partitioning, Motion compensation with adaptable block shapes and Geometry Motion Partitioning. An additional tool has also been evaluated: OBMC. 7 companies have been involved as proponent or cross-checker.

# CE Documents

The following 9 documents are submitted for CE2:

- Summary document (this document)

  - JCTVC-D229 [CE coordinators] CE2: Summary of Core Experiment 2 on Flexible Motion Partitioning

- Proposals and verification documents

Proposal 1: Asymmetric Motion Partitioning with OBMC  (original proposal: JCTVC-C251)

  - Proposal :	JCTVC-D367 [Samsung and Qualcomm] CE2: Asymmetric motion partition with overlapped block motion compensation

  - Cross-check: 	JCTVC-D328 [Technicolor]
JCTVC-D298 [Huawei & HiSilicon]

Proposal 2: Motion compensation with adaptable block shapes  (original proposal: JCTVC-C099)

  - Proposal :	JCTVC-D297 [Huawei & HiSilicon Technologies] CE2: Huawei & HiSilicon report on Flexible Motion Partitioning

  - Cross-check: 	JCTVC-D405 [ETRI]
JCTVC-D418 [Microsoft]

Proposal 3: Geometry Motion Partitioning (original proposal: JCTVC- C031 and JCTVC-C301)

  - Proposal :	JCTVC-D230 [Technicolor and Qualcomm] Simplified Geometry Motion Partitioning

  - Cross-check: 	JCTVC-D173 [INRIA]

Proposal 4: Geometry Motion Partitioning with OBMC  (Original proposal: JCTVC-C251)

  - Proposal :	JCTVC-D368 [Qualcomm]  CE2: Overlapped Block Motion Compensation for Geometry Partition Block

  - Cross-check: 	JCTVC-D427 [MediaTek]

# Proposed Tools

Two types of motion partitioning have been studied:

- Rectangular Motion Partitioning: the tool, known as Asymmetric Motion Partitioning (AMP) and proposed by Samsung, consists of partitioning a square block into two non-symmetric horizontal or vertical rectangles shapes.

- Non-rectangular Motion Partitioning: a square block is possibly divided into two non-rectangular partitions; two different implementations are proposed by  Huawei & HiSilicon Technologies on one side, and Technicolor and Qualcomm on the other side.

In addition, OBMC is proposed by Qualcomm as a solution to smooth the boundary between AMP PUs or between two non-rectangular partitions. OBMC was already used in the GEO implementation of JCTVC-C031 and JCTVC-C301. OBMC proposed in JCTVC-D367 and JCTVC-D368 is an improved version.

# Summary of Results

Test conditions as specified in JCTVC-C500 were used. Only High Efficiency case was considered.

## Asymmetric Motion Partitioning with OBMC (AMP OBMC)

Proposal :	JCTVC-D367 [Samsung and Qualcomm]

AMP with OBMC – Reference: TMuC 0.9

|  | Random access |  |  | Low Delay |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| BD-rate | Y | U | V | Y | U | V |
| Class A | -1.1 | -1.6 | -1.5 |  |  |  |
| Class B | -0.8 | -1.0 | -1.0 | -0.9 | -1.4 | -1.4 |
| Class C | -1.1 | -1.4 | -1.4 | -1.1 | -1.4 | -1.6 |
| Class D | -1.1 | -1.1 | -1.1 | -1.1 | -2.1 | -1.4 |
| Class E |  |  |  | -2.1 | -2.7 | -2.5 |
| All | -1.0 | -1.2 | -1.2 | -1.2 | -1.8 | -1.6 |
| Enc Time | 113% |  |  | 110% |  |  |
| Dec Time | 102% |  |  | 99% |  |  |

AMP with OBMC – Reference: AMP without OBMC

|  | Random access |  |  | Low Delay |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| BD-rate | Y | U | V | Y | U | V |
| Class A | -0.3 | -0.8 | -0.9 |  |  |  |
| Class B | -0.2 | -0.4 | -0.4 | -0.2 | -0.4 | -0.5 |
| Class C | -0.4 | -0.6 | -0.5 | -0.4 | -0.4 | -0.4 |
| Class D | -0.3 | -0.5 | -0.3 | -0.3 | -0.3 | -0.4 |
| Class E |  |  |  | -0.7 | -0.8 | -0.2 |
| All | -0.3 | -0.5 | -0.5 | -0.4 | -0.4 | -0.4 |
| Enc Time | 102% |  |  | 103% |  |  |
| Dec Time | 102% |  |  | 103% |  |  |

## Motion compensation with adaptable block shapes (ABS)

Proposal :	JCTVC-D297 [Huawei & HiSilicon Technologies]

ABS High-efficiency version – Reference: TMuC0.9

|  | Random access |  |  | Low Delay |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| BD-rate | Y | U | V | Y | U | V |
| Class A | -4.3 | -5.0 | -4.6 |  |  |  |
| Class B | -2.4 | -3.1 | -2.9 | -2.5 | -3.8 | -4.0 |
| Class C | -3.8 | -4.7 | -4.9 | -3.3 | -4.3 | -4.6 |
| Class D | -3.2 | -3.6 | -3.2 | -3.3 | -5.2 | -4.6 |
| Class E |  |  |  | -6.7 | -7.6 | -7.4 |
| All | -3.2 | -3.9 | -3.7 | -3.7 | -5.0 | -5.0 |
| Enc Time | 343% |  |  | 303% |  |  |
| Dec Time | 100% |  |  | 95% |  |  |

ABS Fast version – Reference: TMuC0.9

|  | Random access |  |  | Low Delay |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| BD-rate | Y | U | V | Y | U | V |
| Class A | -3.9 | -4.7 | -4.2 |  |  |  |
| Class B | -2.1 | -2.8 | -2.9 | -2.3 | -3.4 | -3.8 |
| Class C | -3.6 | -4.5 | -4.7 | -3.2 | -4.1 | -4.4 |
| Class D | -3.2 | -3.7 | -3.7 | -3.3 | -5.1 | -4.4 |
| Class E |  |  |  | -6.0 | -6.7 | -6.6 |
| All | -3.0 | -3.7 | -3.8 | -3.4 | -4.6 | -4.7 |
| Enc Time | 237% |  |  | 214% |  |  |
| Dec Time | 99% |  |  | 96% |  |  |

ABS Diagonal-only version – Reference: TMuC0.9

|  | Random access |  |  | Low Delay |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| BD-rate | Y | U | V | Y | U | V |
| Class A | -1.9 | -2.5 | -2.1 |  |  |  |
| Class B | -1.1 | -1.4 | -1.5 | -1.1 | -1.7 | -2.0 |
| Class C | -1.8 | -2.3 | -2.5 | -1.6 | -2.2 | -2.3 |
| Class D | -1.8 | -2.2 | -2.1 | -1.8 | -3.1 | -2.4 |
| Class E |  |  |  | -2.7 | -3.2 | -2.8 |
| All | -1.6 | -2.0 | -2.1 | -1.7 | -2.4 | -2.3 |
| Enc Time | 161% |  |  | 152% |  |  |
| Dec Time | 100% |  |  | 101% |  |  |

## Simplified Geometry Motion Partitioning (SGEO)

Proposal :	JCTVC-D230 [Technicolor and Qualcomm]

SGEO version1 - Reference: TMuC0.9

|  | Random access |  |  | Low Delay |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| BD-rate | Y | U | V | Y | U | V |
| Class A | -3.9 | -4.3 | -4.3 |  |  |  |
| Class B | -1.9 | -2.7 | -2.6 | -2.0 | -3.1 | -3.3 |
| Class C | -3.4 | -4.5 | -4.5 | -3.0 | -4.0 | -4.3 |
| Class D | -2.8 | -3.3 | -3.2 | -2.8 | -4.2 | -3.6 |
| Class E |  |  |  | -4.9 | -4.8 | -5.4 |
| All | -2.8 | -3.5 | -3.5 | -3.0 | -3.9 | -4.0 |
| Enc Time | 190% |  |  | 190% |  |  |
| Dec Time | 97% |  |  | 101% |  |  |

SGEO version2 - Reference: TMuC0.9

|  | Random access |  |  | Low Delay |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| BD-rate | Y | U | V | Y | U | V |
| Class A | -2.6 | -2.8 | -2.9 |  |  |  |
| Class B | -1.3 | -1.9 | -2.0 | -1.5 | -2.6 | -3.0 |
| Class C | -2.2 | -3.2 | -3.2 | -2.0 | -3.0 | -3.1 |
| Class D | -1.9 | -2.2 | -2.1 | -1.9 | -3.5 | -2.8 |
| Class E |  |  |  | -4.0 | -4.9 | -4.3 |
| All | -1.9 | -2.5 | -2.5 | -2.2 | -3.4 | -3.2 |
| Enc Time | 130% |  |  | 134% |  |  |
| Dec Time | 98% |  |  | 100% |  |  |

## Geometry Motion Partitioning with OBMC

Proposal :	JCTVC-D368[Qualcomm]

SGEO version2 with new OBMC – Reference: TMuC0.9

|  | Random access |  |  | Low Delay |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| BD-rate | Y | U | V | Y | U | V |
| Class A | -2.8 | -3.0 | -2.9 |  |  |  |
| Class B | -1.4 | -1.9 | -2 | -1.6 | -2.6 | -3.1 |
| Class C | -2.4 | -3.3 | -3.3 | -2.1 | -3.0 | -3.3 |
| Class D | -2.0 | -2.3 | -2.2 | -2.0 | -3.4 | -2.9 |
| Class E |  |  |  | -4.1 | -4.7 | -4.5 |
| All | -2.0 | -2.5 | -2.5 | -2.3 | -3.3 | -3.4 |
| Enc Time | 129% |  |  | 136% |  |  |
| Dec Time | 105% |  |  | 103% |  |  |

SGEO version2 with new OBMC – Reference: SGEO version2 without (old) OBMC

|  | Random access |  |  | Low Delay |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| BD-rate | Y | U | V | Y | U | V |
| Class A | -0.8 | -1.1 | -1.2 |  |  |  |
| Class B | -0.4 | -0.6 | -0.7 | -0.4 | -0.6 | -0.8 |
| Class C | -0.7 | -1.2 | -1.1 | -0.6 | -1.0 | -0.8 |
| Class D | -0.6 | -0.6 | -0.3 | -0.4 | -0.8 | -0.5 |
| Class E |  |  |  | -1.2 | -1.2 | -0.9 |
| All | -0.6 | -0.8 | -0.8 | -0.6 | -0.8 | -0.7 |
| Enc Time | 105% |  |  | 102% |  |  |
| Dec Time | 100% |  |  | 100% |  |  |

## Results analysis

The following figures depict the encoding time as a function of the average Y BD-rate gain, for the different proposals. It is observed that this function is of exponential type.

Figure 1: encoding time as a function of BD-rate gain.

# Conclusion

In this tool experiment, four proposals have been evaluated based on the common test conditions and successfully cross-verified.

The CE participants suggest considering the different proposals for potential introduction into HM.

# Annex: list of CE participants

| Participant | Person, Email address |
| --- | --- |
| Technicolor | Edouard Francois, edouard.francois@technicolor.com Philippe Bordes, philippe.bordes@technicolor.com |
| Qualcomm | Peisong Chen, peisongc@qualcomm.com Liwei Guo, liweig@qualcomm.com |
| Huawei & HiSilicon | Xiaozhen Zheng, zhengxiaozhen@huawei.com Haoping Yu, haopingyu@huawei.com |
| Samsung | Il-Koo Kim, ilkoo.kim@samsung.com |
| INRIA | Laurent Guillo, laurent.guillo@inria.fr Ronan Boitard, ronan.boitard@inria.fr |
| MediaTek | Shan Liu, shan.liu@mediatek.com Yu-Wen Huang, yuwen.huang@mediatek.com |
| ETRI | Hui Yong KIM, hykim5@etri.re.kr Jongho Kim, pooney@etri.re.kr |
| Microsoft | Jizheng Xu, jzxu@microsoft.com |