| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4rd Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D230 WG11 Number: m18yyy |
| --- | --- |

| Title: | CE2: Simplified Geometry Block Partitioning |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Edouard Francois, Philippe Bordes (Technicolor) Liwei Guo, Peisong Chen (Qualcomm) | Tel: Email: | +33-2-99-27-32-62 edouard.francois@technicolor.com philippe.bordes@technicolor.com liweig@qualcomm.com peisongc@qualcomm.com |
| Source: | Technicolor and Qualcomm |  |  |

_____________________________

# Abstract

This document relates to CE2  “flexible block partitioning”. It reports the results of Simplified Geometry adaptive block partitioning (SGEO) implemented into the TMuC0.9. Two levels of simplification of GEO (SGEO_1 and SGEO_2) are proposed. SGEO_1 mainly implements code optimizations with almost no impact on the coding performance compared to the initial GEO algorithm of JCTVC-A121. In addition SGEO_2 uses early termination strategies and a reduced number of GEO modes.

The reported BD-rate results, using TMuC0.9 as reference, are as follows.

- SGEO_1 average Y BD-rate gains are of respectively 2.8% for Random Access High Efficiency, 3.0% for Low Delay High Efficiency, with an encoding time increase of 90%.

- SGEO_2 average Y BD-rate gains are of respectively 1.9% for Random Access High Efficiency, 2.2% for Low Delay High Efficiency, with an encoding time increase around 30-35%.

The decoding time is not impacted.

- Introduction

This document relates Geometry adaptive block partitioning (GEO), previously proposed by Qualcomm and Technicolor in JCTVC-A121, JCTVC-C031 and JCTVC-C301. Further work has been made in order to reduce the encoding complexity while trying to save as much as possible the coding gains. Several optimization algorithms and corresponding results are presented.

The document is organized as follows. Section 2 shortly describes the GEO tool and its initial implementation. Next section presents two versions of algorithmic optimizations and simplifications. Section 4 provides a set of results corresponding to different versions of GEO. Last section concludes this proposal.

# GEO description

## Tool description

Geometry adaptive block partitioning (GEO) consists of dividing a block into 2 regions using a straight line. One motion vector is sent for each region. Two polar coordinates parameters are used to specify the separation line, as shown in Figure 1:  the angle subtended by the perpendicular line with the X axis () and the distance of the partition line from the origin ().

Figure 1: Parameters defining a geometry motion partition.

## First encoder implementation

In order to reduce the encoding complexity, a simplified search algorithm exploits the motion vectors previously computed when testing the square, rectangular and further splitting partitioning modes [5]. Prior to testing the GEO modes, the square 2Nx2N, NxN, rectangular 2NxN and Nx2N, and split CU partitioning modes are first tested, as illustrated in Figure 2.

Figure 2: square, rectangular partitioning modes and re-use of pre-computed MVs for GEO.

Then GEO modes are evaluated as follows. First, fast geometry mode decision pre-selects the K best modes. For each partition region, a motion vector (MV) is tested for each list and each reference index of the list. The tested MV comes from the largest rectangular block previously tested and entirely inside the GEO region. For each tested MV, the prediction error Sum of Absolute Difference (SAD) over this partition is computed. The K partitioning modes with the minimum SAD are selected with their associated motion data. Then the best mode among these K candidates is selected based on full RD computation. Practically K is set to 8.

This algorithm is the initial GEO implementation into TMuC0.9. GEO applies for 16x16 and 32x32 CUs. It is parameterized as follows:

- Block 16x16:  = ,  = 1  494 modes

- Block 32x32: = ,  = 1  238 modes

# Proposed GEO encoding simplifications

## Simplified GEO version 1 (SGEO_1)

The following code optimizations have been made.

In the pre-selection phase:

- Redundancy reduction in motion compensated prediction: N temporary arrays are used to store the reference blocks of the latest tested MVs. If the new MV to be tested is equal to one of these tested MVs, the corresponding Temporary array is used to generate the prediction instead of applying the full motion compensation interpolation process. In practice N is set to 8 or 16.

- Faster SAD computation: The SAD computation is simplified by not scanning the full block for each partition, but by scanning only the pixels belonging to the partition. For each GEO mode, 2 arrays are stored, indicating for each line of the block the first pixel and the last pixel of the partition.

Figure 3: square and rectangular partitioning modes.

- Early termination of SAD computation: Once SAD of partition 0 (part0) has been evaluated, partition 1 (part1) is not tested if the MV of part1 to be tested is the same as the best MV of part0 or if the SAD of part0 is greater than the worse of the already selected GEO candidates.

In the GEO RD optimization:

- Early termination of GEO RD optimization: For each mode m to evaluate, before the call to SAD computation function, a value SADmax is set as:

SADmax = RDcostBest – λ.cost(m)

where RDcostBest is the best RDcost found so far, and cost(m) is the encoding cost of this mode m. SADmax corresponds to the maximum SAD value allowed for this mode to be retained as the best one. Then, the SAD computation process is stopped as soon as the SAD value overcomes SADmax.

- Clipping in SAD computation: In the SAD computation of the GEO RD optimization, the clipping applied to the bi-predicted signal is not applied for GEO mode. This slightly impacts the SAD, with almost no impact on the final result.

## Simplified GEO version 2 (SGEO_2)

In addition to the optimizations made in SGEO_1, the following simplifications have been made.

Reduction of GEO modes number:

- GEO modes corresponding to unbalanced partitioning modes (one partition much larger than the other one) are removed; practically if the smallest partition size is below 3 pixels for 16x16, 11 pixels for 32x32 blocks, the mode is not considered. This leads to:

  - 122 modes for 32x32 blocks (instead of 494 modes)

  - 127 modes for 16x16 blocks (instead of 238 modes)

This also impacts the decoder syntax since GEO modes are now coded using 7 bits FLC.

Early termination:

- The  rectangular partitions (2NxN and Nx2N) are not tested if:

C2Nx2N* < CNxN

with CP corresponding to the RD cost of partitioning mode P. Practically  is set to 1.25

- Similarly, if the RD cost of the 2Nx2N partitioning (including the Skip mode) is low compared to the other partitioning modes, it can be expected that further block splitting is most often not necessary. Practically GEO is tested when the following condition is true:

R = min(CNxN , C2NxN , CNx2N , Csplit) / C2Nx2N  > 

To derive the ratio parameter , an analysis over different sequences and QPs has been performed, using a systematic test of GEO (i.e. test above disabled). For each picture, the maximum value max of the ratio R is computed over all the blocks for which GEO is the best mode. An optimal value of  is then computed as the average of max over the pictures of a same sequence class and same QP. Figure 3 depicts the parameter  per QP value and sequence class, for block size of 32x32. It can be observed that this parameter should depend more or less linearly on the QP value. Similar trends are observed for 16x16 block size. Therefore  is defined as  = a+b.QP with a and b being empirically defined per block size based on the observed data similar to those depicted in Error: Reference source not found. Practically, a is set to 0.98 and b to 0.004.

Figure 4: optimal ratio parameter   per QP value.

Search-range reduction:

- In the best GEO candidate selection stage, the motion search is initialized with the motion vector found in the pre-selection (rather than with the motion predictor).

- In order to speed-up the motion estimation stage, the default search range is divided according to the sequences class:

  - Class D: 	search range is divided by 8

  - Class C: 	search range is divided by 4

  - Class A,B,E: 	search range is divided by 2

# Results

## Evaluation of Full Complexity GEO

A first set of tests has been performed to evaluate the Maximum possible performance of GEO, without limitation on the encoding complexity. This version is highly complex and requires around 25 times the encoding time of the default TMuC0.9. The decoder is not impacted. Due to this long encoding time, the evaluation is performed over a reduced number of frames. For class A 40 frames are used; for the other classes 60 frames are used.

As depicted in Table 1 and Table 2, Full Complexity GEO gives 4.9% gain for Random Access, with a maximum gain of 9% on RaceHorses (class D), 4.3% gain for Low Delay scenarios, with a maximum gain of 7,7% on Vidyo3 (Class E).

Table 1: Full Complexity GEO - Reference: TMuC0.9

|  | Random access |  |  | Low Delay |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| BD-rate | Y | U | V | Y | U | V |
| Class A | -6.5 | -7.3 | -6.8 |  |  |  |
| Class B | -3.0 | -4.1 | -4.5 | -2.6 | -3.8 | -4.1 |
| Class C | -6.5 | -8.1 | -8.7 | -5.0 | -6.8 | -7.4 |
| Class D | -5.0 | -5.4 | -5.7 | -4.7 | -5.5 | -6.0 |
| Class E |  |  |  | -5.8 | -6.7 | -5.4 |
| All | -4.9 | -6.0 | -6.2 | -4.3 | -5.5 | -5.6 |

Table 2: Full complexity GEO, TMuC0.9 (Detailed results).

| Class | sequence | Random Access |  |  | Low Delay |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y(%) | U(%) | V(%) | Y(%) | U(%) | V(%) |
| A | PeopleOnStreet | -5,7 | -5,3 | -5,7 |  |  |  |
|  | Traffic | -7,2 | -9,3 | -8,0 |  |  |  |
| B | Kimono | -1,1 | -1,8 | -1,8 | -1,3 | -1,5 | -1,5 |
|  | ParkSene | -5,1 | -5,7 | -5,8 | -3,7 | -5,7 | -5,9 |
|  | BasketballDrive | -2,9 | -3,7 | -3,8 | -3,5 | -4,0 | -4,6 |
|  | BQTerrace | -2,9 | -4,4 | -5,0 | -2,1 | -3,2 | -3,2 |
|  | Cactus | -3,1 | -5,2 | -6,2 | -2,4 | -4,6 | -5,1 |
| C | RaceHorses | -5,0 | -7,3 | -7,8 | -5,0 | -6,9 | -8,7 |
|  | PartyScene | -8,7 | -9,8 | -10,5 | -6,4 | -9,0 | -9,3 |
|  | BQMall | -3,8 | -4,4 | -5,1 | -2,9 | -4,2 | -3,9 |
|  | BasketballDrill | -8,4 | -10,8 | -11,3 | -5,8 | -6,9 | -7,4 |
| D | BasketballPass | -3,7 | -4,0 | -4,5 | -4,6 | -5,1 | -5,7 |
|  | BQSquare | -2,8 | -3,0 | -3,8 | -3,3 | -3,4 | -5,9 |
|  | BlowingBubbles | -4,7 | -5,7 | -4,8 | -4,5 | -5,3 | -5,3 |
|  | RaceHorses | -9,0 | -9,0 | -9,8 | -6,5 | -8,1 | -7,1 |
| E | Vidyo1 |  |  |  | -5,3 | -5,8 | -5,3 |
|  | Vidyo3 |  |  |  | -7,7 | -9,0 | -6,1 |
|  | Vidyo4 |  |  |  | -4,4 | -5,4 | -4,9 |
| Total Average |  | -4.9 | -6.0 | -6.2 | -4,3 | -5,5 | -5,6 |

## Evaluation of Simplified GEO versions

Test conditions as specified in JCTVC-C500 were used. Only High Efficiency case was considered.

As our computers cluster in not homogeneous, our evaluated encoding times are not really reliable. We report in priority the encoding times provided by our cross-checker, INRIA. Our numbers are provided in grey font.

Table 3: SGEO version1 (Full) - Reference: TMuC0.9

|  | Random access |  |  | Low Delay |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| BD-rate | Y | U | V | Y | U | V |
| Class A | -3.9 | -4.3 | -4.3 |  |  |  |
| Class B | -1.9 | -2.7 | -2.6 | -2.0 | -3.1 | -3.3 |
| Class C | -3.4 | -4.5 | -4.5 | -3.0 | -4.0 | -4.3 |
| Class D | -2.8 | -3.3 | -3.2 | -2.8 | -4.2 | -3.6 |
| Class E |  |  |  | -4.9 | -4.8 | -5.4 |
| All | -2.8 | -3.5 | -3.5 | -3.0 | -3.9 | -4.0 |
| Enc Time | 190% (228%) |  |  | 190% (216%) |  |  |
| Dec Time | 97% (111%) |  |  | 101% (101%) |  |  |

Table 4: SGEO version2 (Light) - Reference: TMuC0.9

|  | Random access |  |  | Low Delay |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| BD-rate | Y | U | V | Y | U | V |
| Class A | -2.6 | -2.8 | -2.9 |  |  |  |
| Class B | -1.3 | -1.9 | -2.0 | -1.5 | -2.6 | -3.0 |
| Class C | -2.2 | -3.2 | -3.2 | -2.0 | -3.0 | -3.1 |
| Class D | -1.9 | -2.2 | -2.1 | -1.9 | -3.5 | -2.8 |
| Class E |  |  |  | -4.0 | -4.9 | -4.3 |
| All | -1.9 | -2.5 | -2.5 | -2.2 | -3.4 | -3.2 |
| Enc Time | 130% (145%) |  |  | 134%(147%) |  |  |
| Dec Time | 98%(100%) |  |  | 100%(102%) |  |  |

## Summarizing

We summarize in the following table the performance results of the different algorithms.

Table 5: comparison between the different algorithms.

|  | Random access |  |  | Low delay |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | rate % | Enc time | Dec time | rate % | Enc time | Dec time |
| Full Complexity | -4.9 |  |  | -4.3 |  |  |
| SGEO_1 | -2.8 | 190% | 97% | -3.0 | 190% | 101% |
| SGEO_2 | -1.9 | 130% | 98% | -2.2 | 134% | 100% |

# Conclusion

In its Full Complexity version, GEO performance (estimated over a reduced number of frames) is evaluated to around 5% gain for Random Access, over 4% gain for Low Delay, with peek gains around 8-9%.

Two levels of simplification of GEO are proposed. SGEO_1 mainly implements code optimizations with almost no impact on coding performance compared to the initial GEO algorithm of JCTVC-A121. In addition SGEO_2 uses early termination strategies and a reduced number of GEO modes.

Compared to the TMuC 0.9, SGEO_1 provides an average gain of around 3% with an average encoding time increased of 90%. SGEO_2 provides an average gain of around 2% with an average encoding time increased of around 30%. The decoder complexity remains similar to the reference TMuC0.9 version.

# Patent rights declaration(s)

Technicolor may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).