| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, Korea, 20-28 January, 2011 | Document: JCTVC-D231 WG11 Number: m18991 |
| --- | --- |

| Title: | CE9 3.2d Simplified Motion vector prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Akira Fujibayashi 3-6 Hikarinooka, Yokosuka-shi Kanagawa, 239 8536 Japan Frank Bossen 3240 Hillview Ave Palo Alto CA 94304 USA | Tel: Email: | +81-468-40-6833 fujibayashi@nttdocomo.co.jp +1 650 496 4742 bossen@docomolabs-usa.com |
| Source: | NTT DOCOMO, Inc and DOCOMO USA Labs |  |  |

_____________________________

# Abstract

This contribution reports the simulation results for section 3.2d of Core Experiment 9 as described in part of JCTVC-C509r3 [1]. Two modifications originated from JCTVC-B094 [2] and one additional modification are proposed to simply and improve the implementability of motion vector prediction method.

The first modification is removing median predictor from the candidates of predictive motion vector (PMV). The second modification is disabling the process that prunes list of PMV candidates according to the motion vector difference (MVD). Additional modification is changing selection manner of left/top predictor that introduces the removing independent corner predictor.

The results confirm our proposed simplified motion vector prediction method achieves almost no loss to conventional motion vector prediction method while providing reduced complexity as parsing of motion vector data and reduced number of PMV candidates.

- Introduction

The core experiment section 3.2d aims to verify the modification of the set of PMV candidates for motion vector (=MV) coding method. In this contribution, no modification to SKIP/DIRECT/MERGE parts is applied so that modifications for these parts are conducted in other section of CE9 (section3.1).

# Technical description

From the description of JCTVC-B094, two modifications are evaluated. Additionally, addition to two modifications, changing selection manner of left/top predictor is proposed.

## Removing median predictor from PMV candidates

In current HM0.9 software, AMVP (Advanced Motion Vector Competition) is used for motion vector prediction. In AMVP, median predictor calculated from left, top, corner predictor is listed to one of PMV candidates. However, given that there are already left, top and corner predictors, median predictor may not be effective any more. Therefore, the performance when removing median predictor is evaluated.

## Disabling the process that prunes list of PMV candidates according to the MVD

As a part of AMVP, there is a process that prunes the list of PMV candidate according to MVD. Thus a predictor construction process affecting parsing of the predictor index takes place while parsing motion vector data associated with the current partition. Such a dependency makes it difficult for parallelized process and has possibility of error distribution. Additionally, this dependency affects the complexity of the finite state machine typically used for the parsing of the bitstream in hardware implementations. It is thus desirable to break this dependency such as to simplify hardware implementations. Therefore, the effect when disabling this pruning process is evaluated.

## Changing the selection manner of left/top predictor

Addition to two modifications described above, the selection manner of PMV candidates is modified. The number of PMV candidates is changed to be formed from up to three motion vector predictors: a left, a top and a collocated predictor.

The process of getting a left and a top predictor is modified like Figure 1.

Figure 1: Search directions and range for left and top predictor

For the left predictor, the first available motion vector is searched from the group of neighboring blocks left of the current block. For this group, block “I” is also included addition to the conventional group of neighboring blocks (F, G, H). A motion vector (MV) is considered available if the vector exists and the reference frame index of the searched block is identical to the reference index of the current block. The search is performed from bottom to top that is inverse order to conventional manner, and only the first available predictor is added to the list of candidates.

The top predictor is derived in a similar fashion but block A,B,C,D and E are listed to neighboring blocks top of the current block . The search order to the group of top neighboring blocks is also reversed to be performed from right to left. If a MV is found and the MV is identical to the left predictor, it is not considered as available and the search is continued.

The third predictor is derived from the collocated block as conventional and the list of predictors is then decimated by removing duplicate predictors.

Unlike to conventional manner, block A,E and I are grouped as a one of left or top neighboring blocks and are not independently considered as a corner predictor. Therefore, maximum number of motion vector predictor is reduced to be three. Considering the case corner predictor being effective, for example RaceHorses sequences, the search order is changed to be reversed.

- Results

The summary results for the two modifications and combination of all modifications under investigation using coding conditions defined in JCTVC-C500 [3] are shown in the Table1, Table2 and Table3 below.  Table1 is result when removing the median predictor form PMV candidates and Table 2 is one when disabling the process that prunes the list of PMV candidates according to MVD. Additionally, combination of all modifications is shown in Table3 .The full data is available in the Excel sheets attached to this contribution.

Table 1: Summary results of BD-Rate comparing HM0.9_NoMedianPredictor to HM0.9

| HM0.9_NoMedianPredictor vs. HM0.9 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Random access |  |  | Random access LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.0 | 0.0 | 0.1 | 0.0 | 0.1 |
| Class B | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | -0.1 |
| Class C | 0.0 | -0.1 | -0.1 | 0.0 | 0.1 | 0.0 |
| Class D | -0.1 | 0.0 | 0.2 | 0.0 | -0.1 | 0.1 |
| Class E |  |  |  |  |  |  |
| All | 0.0 | -0.1 | 0.0 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |

| HM0.9_NoMedianPredictor vs. HM0.9 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Low Delay |  |  | Low Delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | 0.0 | 0.0 | 0.0 | -0.1 | -0.2 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | -0.1 | -0.1 |
| Class D | -0.1 | -0.6 | -0.1 | -0.1 | -0.1 | -0.1 |
| Class E | 0.0 | 0.7 | 0.6 | 0.1 | 0.2 | -0.1 |
| All | 0.0 | 0.0 | 0.1 | 0.0 | 0.0 | -0.1 |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 101% |  |  | 99% |  |  |

Table 2: Summary results of BD-Rate comparing HM0.9_NoPruningProcess to HM0.9

| HM0.9 NoPruningProcess vs. HM0.9 default |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Random access |  |  | Random access LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.2 | 0.2 | 0.3 | 0.2 | 0.2 | 0.2 |
| Class B | 0.2 | 0.1 | 0.1 | 0.2 | 0.1 | 0.1 |
| Class C | 0.2 | 0.1 | 0.0 | 0.2 | 0.2 | 0.2 |
| Class D | 0.2 | 0.2 | 0.2 | 0.3 | 0.1 | 0.3 |
| Class E |  |  |  |  |  |  |
| All | 0.2 | 0.1 | 0.1 | 0.2 | 0.2 | 0.2 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |

| HM0.9_NoPruningProcess vs. HM0.9 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Low Delay |  |  | Low Delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.2 | 0.3 | -0.2 | 0.2 | 0.1 | 0.2 |
| Class C | 0.2 | 0.2 | 0.3 | 0.3 | 0.1 | 0.2 |
| Class D | 0.3 | -0.1 | 0.6 | 0.2 | 0.2 | 0.3 |
| Class E | 0.3 | 0.0 | 0.6 | 0.2 | 0.3 | 0.2 |
| All | 0.2 | 0.1 | 0.3 | 0.2 | 0.2 | 0.3 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |

The results in Table1 and Table2 confirm that the proposed two modifications do not result in a significant loss in coding efficiency compared to TMuC 0.9. As can be seen, removing median predictor results no difference in coding efficiency and disabling pruning process according to MVD only results 0.2% average loss while reducing the complexity to parse motion vector data. The effect of complexity reduction does not become obvious to encoding time and decoding time of software.

Table 3: Summary results of BD-Rate comparing Proposal to HM0.9

| HM0.9 Proposal vs. HM0.9 default |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Random access |  |  | Random access LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.1 | 0.0 | 0.0 | -0.1 | -0.1 | -0.1 |
| Class B | 0.0 | 0.1 | 0.0 | 0.0 | 0.1 | 0.0 |
| Class C | 0.0 | -0.1 | -0.2 | 0.1 | 0.1 | 0.0 |
| Class D | 0.0 | 0.1 | 0.1 | 0.1 | 0.1 | 0.3 |
| Class E |  |  |  |  |  |  |
| All | 0.0 | 0.0 | 0.0 | 0.0 | 0.1 | 0.1 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

| HM0.9 Proposal vs. HM0.9 default |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Low Delay |  |  | Low Delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | 0.1 | -0.1 | 0.0 | -0.2 | -0.1 |
| Class C | 0.0 | 0.1 | 0.0 | 0.1 | 0.0 | 0.1 |
| Class D | 0.1 | -0.4 | 0.5 | 0.0 | 0.2 | 0.1 |
| Class E | 0.0 | 0.8 | 0.2 | 0.3 | 0.2 | 0.1 |
| All | 0.0 | 0.1 | 0.1 | 0.1 | 0.0 | 0.0 |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |

The result in Table3 confirms that our proposed simplified motion vector prediction method achieves almost no loss in coding efficiency compared to TMuC 0.9. Though disabling the process that prunes the list of PMV candidates according to MVD brings 0.2% coding loss as shown in Table2, changing the selection manner of left/top predictor recovers this loss while the number of PMV candidates is further reduced.

- Conclusions and Recommendations

The results of the core experiment have verified that there is no significant difference in coding efficiency due to the 2 modifications. Additionally, changing selection manner of left/top predictor recovers the coding loss caused by other modifications while reducing the number of PMV candidates further more.

The proposed simplified motion vector prediction method consisting of 3 modifications brings almost no loss to current motion vector prediction. Though the complexity reduction by those modifications does not become obvious to encoding or decoding time of software, these modifications reduce the number of PMV candidates and reduce the complexity to parse the motion vector data. It is therefore recommended that proposed simplified motion vector prediction method is adopted into the HEVC WD.

# Patent rights declaration(s)

NTT DOCOMO, Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] J.Jung,B.Bross, “ Core Experiment 9: Motion Vecter Coding”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11,JCTVC-C509r3,3rd Meeting: Guangzhou, China, 7-15 October, 2010.

[2] Frank Bossen, Philipp Kosse, “ Simplified motion vector coding method”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11,JCTVC-C094,2nd Meeting: Geneva, CH, 21-28 July, 2010

[3] F. Bossen., “Common conditions and software reference configurations”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-C500, 3rd Meeting: Guangzhou, China, 7-15 October, 2010.