| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, Korea, 20-28 January, 2011 | Document: JCTVC-D232 WG11 Number: m18992 |
| --- | --- |

| Title: | CE9: Cross verification for subtest 3.3 temporal motion vector memory compression (JCTVC-D072) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Akira Fujibayashi 3-6 Hikarinooka, Yokosuka-shi Kanagawa, 239 8536 Japan | Tel: Email: | +81-468-40-6833 fujibayashi@nttdocomo.co.jp |
| Source: | NTT DOCOMO, Inc. |  |  |

_____________________________

# Abstract

This contribution reports the simulation results and cross verification of a Core Experiment 9 section 3.3 as described in JCTVC-C305r3 [1].  The verification task has been done successfully and the results match exactly provided by SHARP (JCTVC-D072 [2]).

- Verification

Simulation results have been generated by setting AMVP_BUFFERCOMPRESS being 1 in the four inter-coding configurations specified for TMuC 0.9 in JCTVC-C500 [3]. We have confirmed provided code can be compiled successfully, and simulation has been done by using Linux x86_64 workstations typically having Intel Xeon processor 3.3GHz clock. Encoding/decoding time is measured by using “time” command of Linux system.

- Results

We evaluated PSNR as well as encoding/decoding time for each sequences, and the summary of the results are shown in table 1 below. The full data is available in the Excel sheets attached to this contribution.

Table 1: Summary results of BD-Rate comparing SHARP’s proposal to HM0.9

| HM0.9_ AMVP_BUFFERCOMPRESS vs. HM0.9 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Random access |  |  | Random access LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.5 | 0.5 | 0.5 | 0.5 | 0.4 | 0.4 |
| Class B | 0.2 | 0.1 | 0.2 | 0.2 | 0.1 | 0.1 |
| Class C | 0.3 | 0.2 | 0.2 | 0.3 | 0.3 | 0.2 |
| Class D | 0.6 | 0.5 | 0.6 | 0.6 | 0.4 | 0.5 |
| Class E |  |  |  |  |  |  |
| All | 0.4 | 0.3 | 0.3 | 0.4 | 0.3 | 0.3 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 102% |  |  | 101% |  |  |

| HM0.9_ AMVP_BUFFERCOMPRESS vs. HM0.9 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Low Delay |  |  | Low Delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.4 | 0.2 | 0.2 | 0.4 | 0.1 | 0.2 |
| Class C | 0.8 | 0.8 | 0.7 | 0.9 | 0.7 | 0.8 |
| Class D | 0.9 | 0.4 | 0.8 | 0.9 | 0.6 | 0.6 |
| Class E | 0.5 | 0.3 | 0.3 | 0.6 | 0.6 | 0.7 |
| All | 0.6 | 0.4 | 0.5 | 0.7 | 0.5 | 0.5 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 102% |  |  | 101% |  |  |

The results in Table1 confirmed that AMVP_BUFFERCOMPRESS performs 0.4% coding loss on average for Random-access High Efficiency (HE), and 0.6% on average for Low-delay High Efficiency conditions. For Low Complexity (LoCo) settings, the coding loss is 0.6% and 0.7% on average for Random-access and Low-delay conditions.

The encoding time with AMVP_BUFFERCOMPRESS is not changed for each condition. The decoding time is slightly increased (2% for HE setting, 1% for LoCo setting) for all conditions.

- Conclusions and Recommendations

The results presented by SHARP in JCTVC-D072 are confirmed.

# Patent rights declaration(s)

NTT DOCOMO, Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] J.Jung,B.Bross, “ Core Experiment 9: Motion Vecter Coding”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11,JCTVC-C509r3,3rd Meeting: Guangzhou, China, 7-15 October, 2010.

[2] Yeping Su , Andrew Segall, “CE9: Reduced resolution storage of motion vector data”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-C072, Daegu, Korea, 20-28 January, 2011.

[3] F. Bossen., “Common conditions and software reference configurations”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-C500, 3rd Meeting: Guangzhou, China, 7-15 October, 2010.