| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, Korea, 20-28 January, 2011 | Document: JCTVC-D233r1 WG11 Number: m18993 |
| --- | --- |

| Title: | CE9: 3.1 PU Merge & Skip tools and proposed improvements |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information/Proposal |  |  |
| Author(s) or Contact(s): | Yoshinori Suzuki TK Tan 3-6 Hikarinooka, Yokosuka-shi Kanagawa, 239 8536 Japan | Tel: Email: | +81 46 840 3517 suzukiyos@rd.nttdocomo.co.jp +65 9799 3861 tktan.p@rd.nttdocomo.co.jp |
| Source: | NTT DOCOMO, Inc. |  |  |

_____________________________

# Abstract

In this contribution the verification results of CU and PU merging modes for Core Experiment (CE) 9 as described in JCTVC-C509 [1] are provided, and the new merging mode to improve the performance-complexity trade-off is proposed.

- Introduction

The HM utilizes three tools: skip mode, direct mode and CU merging which have similar functionalities.  These tools reduce the motion information bits, which consist of delta motion vectors, prediction mode and reference picture index, by inheriting these information from neighboring blocks to produce the prediction signal in a CU.

In section 3.1 of CE9, three items were evaluated, 1) disabling of direct mode, 2) unifying skip mode with the merging method and 3) enabling merging at the PU level.

In addition to evaluating the tools as described in section 3.1 of CE 9, an improvement in the performance-complexity trade-off is also proposed in this contribution. The redundancy between PU and CU splits was reported by JCTVC-C306 [2]. However, the removal of PU split results in coding loss. Furthermore, the encoder complexity of PU split is high.

To cope with the redundancies of PU split, the new merging mode which can handle rectangular splits (2NxN, Nx2N) and merges better than the PU merging is proposed. The proposed method reduces the complexity of motion search at the encoder as well as the amount of side information. And it also removes the overlapping functionality between PU and CU splits.

- Results of section 3.1 in CE 9

In section 3.1 of CE 9, 18 experiments are established. NTT DOCOMO participated in 10 experiments. The full set of data is available in the Excel sheets attached to this contribution (3.1.f, 3.1.i, 3.1.k, 3.1.l, 3.1.m, 3.1.n, 3.1.o, 3.1.p, 3.1.q, and 3.1.r)

In these experiments, the redundancies among direct, skip and merging modes for both CUs and PUs are studied.

- Disabling direct

  - 3.1.m for CU merge (cf. 3.1.a)

  - 3.1.n for PU merge (cf. 3.1.b)

- Unifying skip mode with the merging method

  - MV candidates for prediction of target CU/PU

    - 3.1.m  3.1.p  3.1.q  3.1.l for CU merge

    - 3.1.n  3.1.o  3.1.r  3.1.e for PU merge

  - Signaling of mrg_cbf (merge and no residual data is coded)

    - 3.1.k for CU merge (cf. 3.1.l)

    - 3.1.e for PU merge (cf. 3.1.f)

According to experiments 3.1.m and 3.1.n, the influence on disabling direct mode can be evaluated for CU and PU merging modes, respectively. Table 1 and Table 2 explain each experiment to evaluate the influence on unifying skip mode with the merge mode. Table 1 shows MV candidates for skip and merge modes. In the four experiments, 3.1.e, 3.1.f, 3.1.k and 3.1.l, skip mode is completely unified with the merge mode since there is no MV candidates for skip mode any more. Table 2 show whether the flag to identify inheriting motion information from neighbor CU/PU and skipping a residual coding is used for each experiment. From the comparison between experiments 3.1.e and 3.1.f for PU merge and between experiments 3.1.k and 3.1.l for CU merge, the effectiveness of using mrg_cbf is known.

Table 1 MV candidates for prediction of target CU/PU

|  | MV candidates | (0,0) | left | above | median | corner | collocated |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 3.1.a, 3.1.m (CU Merge) 3.1.o (PU Merge) | skip *1 | skip (w/ scan) merge (w/o scan) | skip (w/ scan) merge (w/o scan) | skip | skip (w/ scan) *2 | skip |
| 2 | 3.1.p (CU Merge) 3.1.b, 3.1.n (PU Merge) | skip *1 | merge (w/o scan) | merge (w/o scan) | 0 | skip (w/ scan) *2 | skip |
| 3 | 3.1.q (CU Merge) 3.1.r (PU Merge) | 0 | merge (w/o scan) | merge (w/o scan) | 0 | skip (w/ scan) *2 | skip |
| 4 | 3.1.k (CU Merge) 3.1.e (PU Merge) | 0 | merge (w/o scan) | merge (w/o scan) | 0 | merge (w/o scan) *3 | merge |
| 5 | 3.1.l (CU Merge) 3.1.f (PU Merge) | 0 | merge (w/o scan) | merge (w/o scan) | 0 | merge (w/o scan) *3 | merge |

*1; zero vector is used when no other candidates is available

*2; corner MV is above-right or below-left or above-left as in AMVP

*3; corner MVs are both above-right and below-left if they are available

Table 2 Signaling of mrg_cfb

|  | MV candidates | PU merge | CU merge |
| --- | --- | --- | --- |
| 1 | 3.1.a, 3.1.m (CU Merge) 3.1.o (PU Merge) | P (for 2Nx2N), 0 (others) | 0 |
| 2 | 3.1.p (CU Merge) 3.1.b, 3.1.n (PU Merge) | P (for 2Nx2N), 0 (others) | 0 |
| 3 | 3.1.q (CU Merge) 3.1.r (PU Merge) | P (for 2Nx2N), 0 (others) | 0 |
| 4 | 3.1.k (CU Merge) 3.1.e (PU Merge) | P (for 2Nx2N), 0 (others) | P |
| 5 | 3.1.l (CU Merge) 3.1.f (PU Merge) | 0 | 0 |

Simulations were conducted for the above experiments using the common conditions defined in [3]. Table 3 and Table 4 show the summary results of section 3.1 in CE9 for CU and PU merging modes, respectively. From these tables, it is confirmed that experiments 3.1.k and 3.1.e achieve the best coding performance for CU merge and PU merge, respectively.

The results of experiments 3.1.m, 3.1.b and 3.1.n indicate that disabling of direct mode can reduce encoding time with a little coding gain. The result of experiments 3.1.l is better than that of experiments 3.1.m and 3.1.p and is equal to that of experiment 3.1.q. for CU merging, and the result of experiment 3.1.e is better than that of experiments 3.1.n, 3.1.o and 3.1.r for PU merging. These results show that skip mode is unified with the merging mode without any coding loss. From experiments 3.1.k, 3.1.l, 3.1.e and 3.1.f, it is confirmed that the use of mrg_cbf is effective.

Table 3 Comparison experiments in BD-rate (Y) and Encoding time for CU merging (-ve; gain)

|  | Random access |  | Random access LoCo |  | Low delay |  | Low delay LoCo |  | Average |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate | Enc time | BD-rate | BD-rate | Enc time | Enc time | BD-rate | Enc time | BD-rate | Enc time |
| 3.1.m | 0.3% | 73% | -0.2% | 85% | 0.2% | 79% | -0.4% | 89% | -0.03% | 82% |
| 3.1.p | 0.1% | 72% | -0.2% | 84% | 0.0% | 78% | -0.6% | 88% | -0.18% | 81% |
| 3.1.q | 0.1% | 73% | -0.6% | 84% | 0.0% | 78% | -0.8% | 88% | -0.33% | 81% |
| 3.1.k | -0.5% | 75% | -1.1% | 85% | -0.1% | 81% | -1.2% | 89% | -0.73% | 83% |
| 3.1.l | -0.4% | 75% | -0.7% | 86% | 0.0% | 81% | -0.2% | 90% | -0.33% | 83% |

Table 4 Comparison experiments in BD-rate (Y) and Encoding time for PU merging

|  | Random access |  | Random access LoCo |  | Low delay |  | Low delay LoCo |  | Average |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate | Enc time | BD-rate | BD-rate | Enc time | Enc time | BD-rate | Enc time | BD-rate | Enc time |
| 3.1.b | -0.6% | 91% | -0.2% | 96% | -0.5% | 94% | 0.2% | 97% | -0.28% | 95% |
| 3.1.n | -0.3% | 74% | -0.9% | 86% | -0.3% | 80% | -0.6% | 90% | -0.53% | 83% |
| 3.1.o | -0.2% | 75% | -0.9% | 87% | -0.3% | 80% | -0.3% | 90% | -0.43% | 83% |
| 3.1.r | -0.3% | 74% | -1.3% | 86% | -0.3% | 80% | -0.8% | 90% | -0.68% | 83% |
| 3.1.e | -0.9% | 78% | -1.8% | 88% | -0.4% | 84% | -0.9% | 92% | -1.00% | 86% |
| 3.1.f | -0.9% | 78% | -1.2% | 88% | -0.4% | 84% | 0.1% | 92% | -0.60% | 86% |

# Improvement of merge mode

## Overlapping functionality between CU and PU splits

In the current HM, the coding frame is divided into the several CUs using the quad tree structure and each CU is predicted and the residual coded. The size of CU can be selected from 64x64, 32x32, 16x16 and 8x8. In addition, the CU can be divided into 1, 2, or 4 PUs (2Nx2N, 2NxN, Nx2N, NxN) for inter prediction.

The CU merging can reduce the motion information bits to produce the prediction signal in the CU by inheriting it from a neighbor CU or PU. In CE 9, the PU merging is also tested. In this method, the motion information of each PU can also be inherited from the neighbor CU or PU.

As reported in [2], CU split and PU split are redundant, except for the PU split at the smallest CU (SCU) level. CU split does not support 8x4, 4x8 and 4x4 partitions. However, PU splits modes (2NxN, Nx2N and NxN) for CUs larger than SCU can be represented by utilizing the combination of CU split and merging . It was also noted that PU split requires a lot of encoding time for motion search, however, the coding performance is degraded when the PU split is removed.

## CU partial merging

As described above, the removal of PU split decrease the coding performance although CU split and PU split are redundant, except for the PU split at SCU level.

If we look into PU split for CUs larger than SCU with and without merging separately,

- the square PU split (NxN) without merging can be represented using CU split,

- the square PU split with merging is tested at the smaller CU level

- to realize the rectangular PU split (2NxN, Nx2N) without merging using CU split and CU merging, overhead bits is required to produce a rectangular block,

- to realize the rectangular PU split with merging using CU split and CU merging, more overhead bits to produce a rectangular block and merge it with a neighboring CU. It also requires additional motion searches at the encoder though detected motion vectors are not used to produce the prediction signal in this PU when merging is selected.

In this contribution, a new merging mode instead of PU split and merges for CUs larger than SCU is proposed. This proposal consist of the following feature

- Disabled direct (same as 3.1.e)

- Unified skip/merge (same as 3.1.e)

- No PU splits modes (2NxN, Nx2N, NxN) for CUs larger than SCU

- CU merging (same as 3.1.k) and new merging mode, CU Partial merging, for CUs larger than SCU

The CU partial merging handles the rectangular splits (2NxN, Nx2N) and merges better than the PU merging. In this merging mode, the motion information for 1st area of the CU is inherited from a neighbor block and the motion information for 2nd area of the CU is predicted using the conventional motion compensation prediction with the motion vector coding as shown in Figure 1. The method reduces the complexity of motion search at encoder and the coding bits for merge_flag and merge_info. And it also removes the redundancy between PU and CU splits.

Three parameters are signaled to produce the prediction signal in the CU when this new merging mode is employed.

- The size and shape of the merge area (1st area of the CU) is signaled

  - shape; vertical or horizontal

  - size; a) 1/2 of CU (not signaled), b) 1/4, 1/2, or 3/4 of CU (signaled to select one)

- Index to select motion information for the merge area of CU from 5 candidates in the same manner as CU and PU merging modes

- The set of motion information for the remaining area (2nd area of the CU) is signaled (MV of 1st area is not used for MV prediction of 2nd area if one or more MV candidate(s) for MV prediction except for MV of 1st area is valid)

(a) Vertical partial merging                   (b) Horizontal partial merging

Figure 1 CU Partial merging

## Simulation results

Simulations were conducted for the above proposal using the common conditions defined in [3].Table 5 and Table 6 below show the results for random access and low delay conditions in the case that the size of merge area is 1/2 of CU (proposal A). Table 7 and Table 8 below show the results for random access and low delay conditions in the case that the size of merge area is 1/4, 1/2 or 3/4 of CU (proposal B).

Table 5: Simulation result for default random access condition (size of merge area is 1/2 of CU)

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.6 | 1.1 | 1.2 | -1.1 | 0.8 | 0.9 |
| Class B | -0.4 | 0.8 | 1.0 | -1.3 | 0.0 | 0.1 |
| Class C | 0.0 | 0.8 | 0.8 | -0.8 | 0.5 | 0.5 |
| Class D | -0.4 | 0.8 | 0.8 | -1.1 | 0.4 | 0.6 |
| Class E |  |  |  |  |  |  |
| All | -0.3 | 0.8 | 0.9 | -1.1 | 0.3 | 0.4 |
| Enc Time[%] | 60% |  |  | 64% |  |  |
| Dec Time[%] | 104% |  |  | 108% |  |  |

Table 6: Simulation result for default low delay condition (size of merge area is 1/2 of CU)

|  | Low delay |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.1 | 0.4 | 0.3 | -1.4 | -0.2 | -0.3 |
| Class C | -0.3 | 0.0 | 0.2 | -1.4 | -0.5 | -0.4 |
| Class D | -0.5 | -0.6 | -0.1 | -1.7 | -0.3 | -0.6 |
| Class E | -1.0 | -3.5 | -4.4 | -1.8 | 0.3 | -0.2 |
| All | -0.4 | -0.7 | -0.7 | -1.6 | -0.2 | -0.4 |
| Enc Time[%] | 65% |  |  | 67% |  |  |
| Dec Time[%] | 105% |  |  | 108% |  |  |

Table 7: Simulation result for default random access condition (size of merge area is 1/4, 1/2 or 3/4 of CU)

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.6 | 0.0 | 0.2 | -2.0 | -0.1 | 0.0 |
| Class B | -1.2 | -0.1 | 0.2 | -2.1 | -0.7 | -0.6 |
| Class C | -0.9 | -0.2 | -0.2 | -1.5 | -0.4 | -0.5 |
| Class D | -1.1 | -0.2 | -0.1 | -1.7 | -0.4 | -0.2 |
| Class E |  |  |  |  |  |  |
| All | -1.1 | -0.1 | 0.0 | -1.8 | -0.5 | -0.4 |
| Enc Time[%] | 86% |  |  | 96% |  |  |
| Dec Time[%] | 104% |  |  | 107% |  |  |

Table 8: Simulation result for default low delay condition (size of merge area is 1/4, 1/2 or 3/4 of CU)

|  | Low delay |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -1.1 | -1.1 | -1.6 | -2.3 | -1.0 | -1.2 |
| Class C | -1.2 | -0.9 | -1.0 | -2.0 | -1.3 | -1.4 |
| Class D | -1.4 | -1.8 | -1.5 | -2.3 | -1.2 | -1.5 |
| Class E | -3.0 | -5.8 | -6.9 | -3.7 | -1.8 | -1.7 |
| All | -1.5 | -2.1 | -2.4 | -2.5 | -1.3 | -1.4 |
| Enc Time[%] | 92% |  |  | 100% |  |  |
| Dec Time[%] | 105% |  |  | 108% |  |  |

As shown in Table 9, compared with 3.1.k (best case of CU merge) and 3.1.e (best case of PU merge), the proposed method offers the better performance/complexity tradeoff.

Table 9 Comparison experiments in BD-rate (Y) and Encoding time between three coding schemes

|  | Random access |  | Random access LoCo |  | Low delay |  | Low delay LoCo |  | Average |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate | Enc time | BD-rate | BD-rate | Enc time | Enc time | BD-rate | Enc time | BD-rate | Enc time |
| 3.1.k | -0.5% | 75% | -1.1% | 85% | -0.1% | 81% | -1.2% | 89% | -0.73% | 83% |
| 3.1.e | -0.9% | 78% | -1.8% | 88% | -0.4% | 84% | -0.9% | 92% | -1.00% | 86% |
| proposal A | -0.3% | 60% | -1.1% | 64% | -0.4% | 65% | -1.6% | 67% | -0.85% | 64% |
| proposal B | -1.1% | 86% | -1.8% | 96% | -1.5% | 92% | -2.5% | 100% | -1.73% | 94% |

# Conclusions

This contribution provided the results of section 3.1 in CE9 and proposed a new proposal which offers better performance/complexity trade-off. According to the CE9 results comparing the skip, direct and merging prediction modes, the direct mode can be removed with no loss and skip mode can be unified with the merging mode with a coding gain.

The proposed merging mode, the CU Partial merging, handles the rectangular splits and merges better than the PU merging. The method reduces the complexity of motion search at encoder and the coding bits for merge_flag and merge_info. And it also removes the redundancy between PU and CU splits. It is confirmed that our proposal offers the best performance-complexity tradeoff among three merging modes.

The proposed CU Partial merging method is recommended for adoption into the HM.

# Patent rights declaration(s)

NTT DOCOMO, Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- J. Jung et. al., “Core experiment 9: Motion Vector Coding”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-C509, 3rd Meeting: Guangzhou, China, 7-15 October, 2010.

- H. Schwarz et. al., “Investigations for representing rectangular blocks using the merging concept”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-C509, 3rd Meeting: Guangzhou, China, 7-15 October, 2010.

- F. Bossen., “Common conditions and software reference configurations”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-C500, 3rd Meeting: Guangzhou, China, 7-15 October, 2010.