| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, Korea, 20-28 January, 2011 | Document: JCTVC-D234 WG11 Number: m18994 |
| --- | --- |

| Title: | Random access support for HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposed Draft |  |  |
| Author(s) or Contact(s): | Akira Fujibayashi TK Tan 3-6 Hikarinooka, Yokosuka-shi Kanagawa, 239 8536 Japan | Tel: Email: | +81-468-40-6833 fujibayashi@nttdocomo.co.jp tktan.p@rd.nttdocomo.co.jp |
| Source: | NTT DOCOMO, Inc. |  |  |

_____________________________

# Abstract

Random access is an essential feature in any video coding schemes.  This contribution reports that random access is not supported in the HM0.9 software and therefore the common test conditions currently does not simulate true random access capabilities as the bit streams that are created cannot be decoded from the middle of the bit stream without causing error propagation.

Two different methods that output random accessible bit streams were added to the HM0.9 software and simulation results are reported.  The first method implements the instantaneous decoding refresh which is useful when bit stream editing and splicing are needed.  The second method implements a deferred decoding refresh which gives better coding efficiency than the first method.  An average coding gain of 4.4% in BD-rate for both high efficiency and low complexity test was obtained with the deferred decoding refresh method compared to the instantaneous decoding refresh method.

Both random access methods show merit and it is proposed that both these methods be added to the HEVC specification and future HM software releases.  When performing core experiments under the random access coding conditions, it is recommended that the deferred decoding refresh method be used.

Finally, the ability to skip frames and random access into the middle of a bit stream was also implemented in the HM0.9 software decoder.  It is proposed that this feature be added to the future releases of the HM software as well.

- Introduction

At the Geneva meeting in July, 2010, the JCT-VC decided to adopt the higher-level syntax mechanism of the AVC for HEVC.  As a result, the random access capability of AVC is now part of the HEVC specification.

However, the HM0.9 software as it is implemented now, does not provide any true random access capabilities.  Reference pictures that are decoded prior to the random access point are not marked as “unused for reference”.  This means that during a random access, these reference pictures will not be available and therefore causes decoding mismatch that perpetuates beyond the first few B pictures.  The following paragraph describes the current implementation in further detail.

The software configuration for random access case inserts an I picture at regular frame intervals of approximately one second.  The sample output in Figure 1, shows that an I picture is inserted at POC 48.  This is followed by a series of B pictures with POC between 41 and 47.  In an opened_gop structure, it is natural for these pictures to reference the I picture and other references that were decoded prior to the I picture.  However, notice that the next picture with POC 56 which is greater than the POC of the I picture has 2 pictures in each of the list and one of them is a picture with POC 46.  This picture should not be referenced as it will not be available in a random access situation.  Therefore, prediction error due to the missing picture with POC 46 will propagate via the picture with POC 56 as it is used as a reference for subsequent pictures.

Figure 1: Output of the HM encoder using the random access configuration.

Figure 2 shows the above in the output order of the sequences.  It clearly shows the propagation path of the prediction from B40 to B56 as described earlier in red.  Since B56 is used for subsequent prediction, error due to the reference to the unavailable B40 will persist in the decoded pictures following the random access.

Figure 2: Illustration of the frames and prediction structure in output order for HM0.9

It is also noted that the IDR pictures are also not supported in the HM0.9 software.  In the next section, the implementation and investigation into random access using IDR pictures is reported.

- Random access with IDR

The HM0.9 was modified so that it will produce bit streams containing IDR pictures that can be randomly accessed.  Figure 3 shows the pictures and prediction structure in output order for this implementation of the IDR picture random access.  The prediction structure is kept the same, however, the reference pictures are instantaneously marked as “unused for reference” when the IDR picture is decoded.  Hence the prediction from B40 is removed.

Figure 3: Illustration of the frames and prediction structure in output order with the IDR

The following results were obtained for the above implementation and compared to the HM0.9 using the random access configuration as defined in [1].  The results show that by disabling the prediction from the previous GOP, the coding efficiency is reduced by 6.7% and 5.8% in BD-rate for the high efficiency and low complexity cases, respectively.

Table 1: Summary results of BD-Rate comparing HM0.9 IDR to HM0.9 default

| HM0.9 IDR vs. HM0.9 default |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Random access |  |  | Random access LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 7.8 | 10.3 | 9.8 | 7.1 | 7.4 | 7.4 |
| Class B | 8.6 | 13.0 | 13.0 | 7.4 | 8.1 | 7.3 |
| Class C | 5.4 | 6.1 | 6.5 | 4.6 | 4.9 | 5.0 |
| Class D | 5.0 | 5.9 | 6.3 | 4.2 | 4.5 | 4.7 |
| Class E |  |  |  |  |  |  |
| All | 6.7 | 8.9 | 9.0 | 5.8 | 6.2 | 6.0 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |

This experiment showed that using the IDR picture to enable random access causes a significant penalty in coding efficiency.  The next section describes the method for achieving random access without IDR pictures.

- Random access without IDR

The IDR picture random access is useful for bit stream editing and splicing.  However as shown in the previous section the coding efficiency is not as good.  Using an IDR picture as the random access point forces all reference pictures to be instantaneously marked as “unused for reference”.  This prevents any past decoded pictures from being used as reference, even by the B pictures with output order smaller than the output order of the IDR picture.  Therefore an IDR picture strictly implements a closed_gop which results in lower coding efficiency.  In order to achieve random access with better coding efficiency, it is proposed to defer the marking of the reference pictures as “unused for reference” to a later time, so that the above mentioned B pictures may refer to the decoded pictures prior to the random access point.

This alternative can be supported by a combination of 3 mechanisms in AVC:

- Using I pictures instead of IDR pictures as the random access points.

- Sending a recovery point SEI message to indicate the I picture is a random access point and indicating the recovery_frame_cnt for the first picture with an output time greater than the I picture.

- Sending a series of MMCO commands to mark all decoded picture except for the I picture in the DPB as “unused for reference”, prior to the decoding of the first picture with an output time greater than the I picture.

However with the method described above, a very fundamental feature of random access with better quality has to be supported in a complicated manner, requiring combinations of different unrelated mechanisms.  It is also not guaranteed that the SEI message will always be present, since it is not mandatory to carry SEI messages in-band in the video bit stream.

Therefore, it is proposed that in addition to the capabilities that are now supported, a deferred decoding refresh (DDR) process should be supported in HEVC in a manner similar to the IDR picture.  Indicating the DDR picture as a NAL_unit_type as in the IDR picture case would make the signaling of this random access point explicit and in-band.  The next section describes the proposed syntax and semantics associated with this random access support.

- Syntax and semantics for deferred decoding refresh (DDR)

In the DDR process described in the previous section, the deferred reference picture marking must be done just prior to the decoding of the first non-reordered picture following the DDR picture.  This ensures that pictures with output orders greater than the output order of the DDR picture can be decoded without inter prediction from any of the decoder pictures that precedes the DDR picture in output order.

Similar to the IDR picture, the DDR picture can be defined as follows.

3.xxx	deferred decoding refresh (DDR) access unit: An access unit in which the primary coded picture is a DDR picture.

3.xxx	deferred decoding refresh (DDR) picture: A coded picture containing only slices with I slice types that causes the decoding process to mark all reference pictures except the current DDR picture as "unused for reference" immediately before the decoding of the first picture following the current DDR with an output order greater than the current DDR picture. All coded pictures that follow a DDR picture in output order can be decoded without inter prediction from any picture that precedes the DDR picture in output order.  All coded pictures with output order smaller than the current DDR are not affected by the deferred marking process.

The DDR is signalled as a NAL unit type.  An example is given in Table 7-2 below.  In this example, the nal_unit_type 4 is used for the DDR.  However, any other number may also be used.

- Table 7-2 – NAL unit type codes, syntax element categories, and NAL unit type classes

| nal_unit_type | Content of NAL unit and RBSP syntax structure | C | Annex Error: Reference source not found NAL unit type class | Annex Error: Reference source not found and Annex Error: Reference source not found NAL unit type class |
| --- | --- | --- | --- | --- |
| 0 | Unspecified |  | non-VCL | non-VCL |
| 1 | Coded slice of a non-IDR and non-DDR picture slice_layer_without_partitioning_rbsp( ) | 2, 3, 4 | VCL | VCL |
| 2 - 3 | reserved |  |  |  |
| 4 | Coded slice of a DDR picture slice_layer_without_partitioning_rbsp( ) | 2, 3 | VCL | VCL |
| 5 | Coded slice of an IDR picture slice_layer_without_partitioning_rbsp( ) | 2, 3 | VCL | VCL |
| ….. | ……. |  |  |  |

The RefreshPendingFlag (RP) is a variable that stores the state of the decoder.  It is set to “1” when the decoder has decoded a DDR picture and reset to “0” when the decoder has decoded the first picture with a temporal reference greater that the temporal reference in display order of the DDR picture (TD_DDR) and has performed the deferred reference picture marking process.  The temporal reference may be the output order or the output timing of the picture.

A flow chart of the DDR process is shown in Figure 4.  And overview of the decoding process is given as follows.

- The process begins with the initialization of the variable TR_DDR and RefreshPendingFlag to 0.

- If the RefreshPendingFlag is “1” and the temporal reference of the current picture (TR_CUR) is greater than TD_DDR, the deferred reference picture marking process is invoked and the RefreshPendingFlag is reset to “0”.  The deferred reference picture marking process marks all reference pictures except the latest DDR picture as “unused for reference”.

- If the nal_unit_type is “DDR”, the RefreshPendingFlag is set to “1” and the TD_DDR is set to the temporal reference of the current picture.

- Perform the decoding process for the NAL units.

- If the nal_ref_idc is not “0”, perform the decoded reference picture marking process.

- Proceed with the next picture.

Figure 4:  Flow chart for DDR processing

- Random access with DDR

The HM0.9 was further modified so that it will produce bit streams containing DDR pictures that can be randomly accessed.  Figure 5 shows the pictures and prediction structure in output order for this implementation of the DDR picture random access.  The prediction structure is kept the same; however, the reference picture marking process is deferred to the instance when the first picture with an output order greater that the output order of the DDR picture is decoded.  Hence the prediction from B40 is not removed immediately and is available for the pictures B41 to 47 that are decoded after the DDR picture but have output order before it.  This improves the coding efficiency for these pictures.  However during random access situations these pictures contain prediction error due B40 being unavailable and are usually not decoded or displayed.

The DDR process marks the reference pictures except for the latest DDR picture as “unused for reference” just before picture B56 is decoded.  Hence the propagation of prediction error from the unavailable reference picture B40 in a random access situation is prevented.

Thus the DDR process enables a random accessible bit stream that does not propagate prediction error perpetually while maintaining a good coding efficiency.

Figure 5: Illustration of the frames and prediction structure in output order with the DDR

The results in Table 3 were obtained for the above implementation and compared to the HM0.9 with IDR using the random access configuration as defined in [1].  The results show that deferring the decoding refresh, by using DDR pictures instead of IDR pictures for the random access points achieves an average BD-rate of 4.4% better in both the high efficiency and low complexity cases.

Table 3: Summary results of BD-Rate comparing HM0.9 DDR to HM0.9 IDR

| HM0.9 DDR vs. HM0.9 IDR |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Random access |  |  | Random access LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -4.4 | -5.4 | -5.1 | -4.5 | -5.1 | -4.9 |
| Class B | -5.4 | -6.5 | -6.0 | -5.3 | -5.8 | -5.4 |
| Class C | -3.8 | -4.3 | -4.3 | -3.8 | -4.2 | -4.3 |
| Class D | -3.8 | -3.9 | -4.0 | -3.7 | -4.2 | -4.2 |
| Class E |  |  |  |  |  |  |
| All | -4.4 | -5.1 | -4.9 | -4.4 | -4.8 | -4.7 |
| Enc Time[%] | 98% |  |  | 98% |  |  |
| Dec Time[%] | 99% |  |  | 101% |  |  |

Table 4 shows the coding efficiency of using DDR pictures compared to HM0.9 default. The penalty for enabling random accessibility was reduced to only 1.9% and 1.1% in BD-rate for the high efficiency and low complexity cases, respectively.  Note that this is not a true loss but a trade off in coding efficiency for enabling the random access capability.  Comparison to the HM0.9 default is not a good indication of coding performance in the random access configuration as HM0.9 default does not support random access.  In fact, the bit streams generated could not be decoded from the middle of the bit stream as illustrated in the next section.

Table 4: Summary results of BD-Rate comparing HM0.9 DDR to HM0.9 default

| HM0.9 DDR vs. HM0.9 default |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Random access |  |  | Random access LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 2.9 | 4.2 | 4.1 | 2.3 | 1.9 | 2.1 |
| Class B | 2.7 | 5.6 | 6.2 | 1.7 | 1.8 | 1.5 |
| Class C | 1.4 | 1.6 | 1.9 | 0.7 | 0.5 | 0.5 |
| Class D | 1.0 | 1.7 | 2.0 | 0.3 | 0.2 | 0.4 |
| Class E |  |  |  |  |  |  |
| All | 1.9 | 3.3 | 3.6 | 1.1 | 1.0 | 1.0 |
| Enc Time[%] | 98% |  |  | 98% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |

- Random access decoding

Random access decoding capability has been implemented in the HM0.9 as a proof of concept.  All pictures prior to the random access point are skipped.  The decoder performs the following actions depending on the nal_unit_type of the picture at the random access point.

- If the nal_unit_type is a coded slice of a DDR picture, pictures with an output order equal to or greater than the output order of the random access point are decoded.

- If the nal_unit_type is a coded slice of an IDR picture, pictures in decoding order starting from and including the random access point are decoded.

- If the nal_unit_type is a coded slice of a non-IDR and non-DDR picture, a warning is issued and decoding of pictures with an output order equal to or greater than the output order of the random access point is attempted.  When starting a random access at a non-IDR/non-DDR coded slice, it is not guaranteed that the decoder will behave in a well defined way and not crash.

Figure 6 shows the 23rd pictures after the random access into the 2nd group of pictures (GOP) for the HM0.9 default and DDR bit streams, respectively.  Note that the decoder was unable to decode beyond the 25th picture of the HM0.9 default bit stream as the unavailable information lead to an undefined state (which causes a segmentation fault).  The propagation of prediction error is obvious in decoded picture of the HM0.9 default bit stream.  The decoder was able to decode the HM0.9 with DDR bit stream completely and the output matched the corresponding decoded pictures of the full bit stream.

(a) HM0.9 default				(b) HM0.9 with DDR

Figure 6: Decoded pictures from random accessing into the 2nd GOP

- Conclusions and recommendations

It was identified that the HM0.9 software does not produce bit streams that can be decoded from random access points within it.

Two methods for creating bit streams with random access support were implemented into the HM0.9 software.  The first method inserts IDR pictures while the second method inserts DDR pictures as the random access points.  Inserting DDR pictures as the random access points results in a 4.4% BD-rate improvement compared to inserting IDR pictures, for both the high efficiency and low complexity test conducted.

This contribution proposes that both IDR and DDR NAL unit types be supported in the HEVC.  It is also recommended that this capability be implemented in the HM software so that bit streams that are generated using the random access configuration, can truly be decoded from any of the random access points in the bit streams.  The preferred default random access configuration for core experiments should be the DDR method.

# Patent rights declaration(s)

NTT DOCOMO, Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- F. Bossen., “Common conditions and software reference configurations”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-C500, 3rd Meeting: Guangzhou, China, 7-15 October, 2010.