| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D235 |
| --- | --- |

| Title: | Enhancements to Intra Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sandeep Kanumuri1 TK Tan2 Frank Bossen1 1, 3240 Hillview Ave Palo Alto CA 94304 USA 2, 3-6 Hikarinooka, Yokosuka-shi Kanagawa, 239 8536 Japan | Tel: Email: | +1-650-496-4723 skanumuri@docomolabs-usa.com +65 9799 3861 tktan.p@rd.nttdocomo.co.jp +1-650-496-4742 bossen@docomolabs-usa.com |
| Source: | DOCOMO Communications Laboratories USA, Inc. & NTT DOCOMO, Inc. |  |  |

_____________________________

# Abstract

This contribution proposes two modifications to intra coding. The first modification is the introduction of a modified Planar mode called Planar+DST which codes the residual. The second modification improves the intra mode coding (IMC) by using up to two candidate modes derived from neighboring partitions. The proposed modifications improve the coding efficiency under all 6 common test conditions. For the High-Efficiency setting, the average reduction in luma BD-rate is 2.1%, 1.1% and 0.3% with Intra, Random-Access and Low-Delay conditions respectively. For the Low-Complexity setting, the corresponding reduction is 2.4%, 1.0% and 0.3%.

- Introduction

The current contribution proposes two modifications to enhance intra coding. The first modification (Planar+DST) is described in Section 2 and the second modification (IMC) is described in Section 3. Section 4 reports the performance of the proposed modifications in addition to the impact of an encoder bug fix related to CABAC. Section 5 presents our recommendation.

# Planar+DST mode

The first modification proposed in this contribution is the addition of a new intra mode called Planar+DST for both luma and chroma. Planar+DST is an intra mode similar to the Planar mode in [1]. The conceptual differences between the Planar mode and Planar+DST mode are illustrated in Figure 1 and also listed below:

- Planar mode codes the bottom-right pixel value (Z) while Planar+DST does not. Planar+DST estimates the bottom-right pixel value and derives the prediction using this estimated value. The estimated value is equal to the average of the reconstructed values of top-right (T) and bottom-left (L) pixels.

- Planar mode does not code the residual. Planar+DST codes the residual using a DST.

Value of Z is coded

Residual is NOT coded

T

L

Z

Interpolation indicated by dashed arrow

Value of Z is NOT coded

Z = (L + T) >> 1

Residual is coded using DST

Pixels being predicted

Pixels already reconstructed

Planar

Planar+DST

Figure 1: Planar and Planar+DST modes

From an implementation perspective, the computation of the prediction signal can be simplified for Planar+DST. When Z = (L + T) >> 1, the bilinear interpolation can be replaced with the average of two linear interpolations (horizontal and vertical) as shown in Figure 2. Furthermore, the two linear interpolations can be done with additions and shifts and hence there is no need for multiplications or divisions.

+

>> 1

T

L

T

T

L

L

Interpolation indicated by dashed arrow

Replication indicated by dotted arrow

Figure 2: Simpler computation of prediction signal in Planar+DST mode

## Signaling the Planar+DST mode

In the current HM specification, mode 2 represents the DC mode which is the only non-directional mode. Planar+DST is also a non-directional mode. In the current contribution, we generalize mode 2 to represent both the non-directional modes (DC and Planar+DST). When mode 2 is signaled, an additional flag is sent to resolve between DC and Planar+DST modes. A flag value of ‘0’ indicates the DC mode and a value of ‘1’ indicates the Planar+DST mode.

## Transform for residual

The residual from Planar+DST mode has a non-uniform energy distribution. Because of the nature of the prediction, the energy of the residual typically increases from left to right and from top to bottom within a block. A DST achieves a better compaction than a DCT in such a case. However, the need to implement additional transforms is not desirable from a complexity viewpoint.

The Planar+DST mode uses a variant of DST (type-4*) derived from the butterfly implementation of a DCT (type-2) at the next dyadic level. For example, the DST of size 4x4 is derived from the DCT of size 8x8. Since the largest transform size supported is 32x32, the support for DST exists only up to 16x16. For the case of 32x32, DCT is used. This approach reuses the implementation of existing transforms and hence new transforms need not be implemented.

The type-2 DCT matrix (size 2Nx2N) is of the form Hi,j=cosi−12j−1π22N,∀1≤i,j≤2N. The type-4* DST (size NxN) is derived as G∗i,j=H2i,N+1−j=−H2i,j+N,∀1≤i,j≤N.

The type-4 DST (size NxN) matrix is of the form Gi,j=sin2i−12j−1π4N,∀1≤i,j≤N and the type-4* DST differs from the type-4 DST in the sign value of some of the basis functions as indicated by the equation G∗i,j=−1i−1×Gi,j,∀1≤i,j≤N.

The quantization tables used for the DST are also derived from the quantization tables used for the DCT. Hence there is no need to store additional quantization tables.

# Improved intra mode coding

The second modification proposed in this contribution is the addition of a second candidate for the most probable mode.  In the current HM specification, a single most probable mode is derived in the encoder and decoder.  The most probable mode is the smaller of the prediction modes of the partition above and to the left of the current partition.  The encoder signals to the decoder if the prediction mode of the current partition is the same as the most probable mode.  If it is, no further information is sent; otherwise, the remainder mode (prediction mode “excluding” the most probable mode) is sent to the decoder.

With the increase in the number of prediction modes, the probability of the most probable mode matching the current prediction mode is reduced.  The cost of sending the remainder mode is also increased.

To improve the coding efficiency of sending the intra prediction mode information, up to two candidate modes are used instead of using just the most probable mode.  The following are the details of the proposal.

The encoder and decoder derive the candidate modes list as in Section 3.1.  The encoder then signals to the decoder if the prediction mode is in the candidate modes list.  If it is and if there is only one entry in the candidate modes list, then no further information is sent.  Otherwise, the index of the matching candidate prediction mode is sent.  If the prediction mode is not in the candidate modes list, then the remainder mode derived as in Section 3.2, is sent to the decoder.  Section 3.3 describes the derivation process of the prediction mode from the remainder mode decoded from the bitstream.

## Candidate modes list derivation.

The inputs to this process are the prediction modes predModeA and predModeB of the partition above and to the left of the current partition, respectively.

The output of this process is a candidate mode list candModeList[x] containing up to two possible candidate prediction modes.

The candidate prediction modes are predModeA and predModeB.  The candModeList[x] is derived as follows

- If both candidate prediction modes are not available, then the DC_pred mode is added to candModeList[0].

- Otherwise, if only one candidate prediction mode is available or if both candidate prediction modes are the same, then this candidate prediction mode is added to candModeList[0].

- Otherwise, both candidate prediction modes are added to the candidate modes list with the smaller of the two candidates at candModeList[0] and the larger at candModeList[1].

## Remainder mode derivation. (encoder)

The inputs to this process are the candidate modes list candModeList[x], the number of candidate modes in the list numCand and the prediction mode of the current partition predModeC.

The output of this process is the remainder mode remMode.

The remMode is derived by applying the following procedure

remMode = predModeC

for (cIdx = numCand-1; cIdx >= 0; cIdx--)

if (remMode > candModeList[cIdx]) remMode--

## Prediction mode derivation from remainder mode. (decoder)

The inputs to this process are the candidate modes list candModeList[x], the number of candidate modes in the list numCand and the remainder mode of the current partition remMode.

The output of this process is the prediction mode of the current partition predModeC.

The predModeC is derived by applying the following procedure

predModeC = remMode

for (cIdx = 0; cIdx < numCand; cIdx++)

if (predModeC >= candModeList[cIdx]) predModeC++

# Experimental results

The experiments were conducted in accordance with the 6 common test conditions described in [2].

The proposed modifications were implemented in V0.9 of the common software. During the implementation, an encoder bug related to CABAC was found. When different intra modes are tested in the encoder, the bug causes the encoder to use different probability estimates for different modes. This occurs because the probability estimates are modified when a mode is tested and are not reset to their original values before another mode is tested. When this bug is fixed, there is a small improvement in coding efficiency as shown in Table 1 below. The excel sheet corresponding to this result is attached as part of the contribution.

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.1 | -0.2 | -0.2 | 0.0 | 0.0 | 0.0 |
| Class B | -0.1 | -0.1 | -0.1 | 0.0 | 0.0 | 0.0 |
| Class C | -0.2 | -0.2 | -0.2 | 0.0 | 0.0 | 0.0 |
| Class D | -0.2 | -0.2 | -0.2 | 0.0 | 0.0 | 0.0 |
| Class E | -0.1 | -0.1 | -0.2 | 0.0 | 0.0 | 0.0 |
| All | -0.2 | -0.2 | -0.2 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Random access |  |  | Random access LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.1 | 0.0 | 0.2 | 0.0 | 0.0 | 0.0 |
| Class B | -0.1 | -0.1 | -0.1 | 0.0 | 0.0 | 0.0 |
| Class C | -0.1 | 0.0 | -0.1 | 0.0 | 0.0 | 0.0 |
| Class D | -0.1 | -0.1 | 0.2 | 0.0 | 0.0 | 0.0 |
| Class E |  |  |  |  |  |  |
| All | -0.1 | -0.1 | 0.0 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | 0.0 | 0.1 | 0.0 | 0.0 | 0.0 |
| Class C | -0.1 | 0.0 | -0.1 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | -0.4 | 0.2 | 0.0 | 0.0 | 0.0 |
| Class E | -0.2 | -0.2 | -0.3 | 0.0 | 0.0 | 0.0 |
| All | -0.1 | -0.1 | 0.0 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table 1: Performance of bug fix (Reference: V0.9)

The proposed modifications were tested with the bug fix enabled. The improvement in coding efficiency due to the proposed modifications and the bug fix can be found in Table 2. The excel sheet corresponding to this result is attached as part of the contribution.

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -2.4 | -1.7 | -1.0 | -2.7 | -2.8 | -2.3 |
| Class B | -2.5 | -1.9 | -1.5 | -2.7 | -2.7 | -2.2 |
| Class C | -2.1 | -1.4 | -1.3 | -2.1 | -2.2 | -2.2 |
| Class D | -1.9 | -1.4 | -1.3 | -2.0 | -2.0 | -2.1 |
| Class E | -2.6 | -0.7 | -0.5 | -2.9 | -2.3 | -2.7 |
| All | -2.3 | -1.5 | -1.2 | -2.4 | -2.4 | -2.3 |
| Enc Time[%] | 107% |  |  | 111% |  |  |
| Dec Time[%] | 102% |  |  | 105% |  |  |
|  | Random access |  |  | Random access LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.2 | -0.7 | 0.0 | -1.1 | -0.7 | -0.3 |
| Class B | -1.4 | -1.6 | -1.3 | -1.1 | -0.9 | -0.6 |
| Class C | -1.2 | -1.3 | -1.1 | -0.9 | -0.5 | -0.4 |
| Class D | -0.9 | -0.8 | -0.4 | -0.7 | 0.1 | 0.2 |
| Class E |  |  |  |  |  |  |
| All | -1.2 | -1.2 | -0.8 | -1.0 | -0.5 | -0.3 |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.5 | -0.7 | -0.4 | -0.3 | -0.2 | -0.4 |
| Class C | -0.5 | -0.7 | -0.7 | -0.3 | -0.3 | -0.4 |
| Class D | -0.3 | -0.7 | -0.3 | -0.2 | 0.0 | -0.3 |
| Class E | -0.1 | 1.4 | 0.2 | -0.2 | -0.5 | 0.5 |
| All | -0.4 | -0.3 | -0.3 | -0.3 | -0.2 | -0.2 |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 102% |  |  |

Table 2: Performance of proposed modifications + bug fix (Reference: V0.9)

The improvement in coding efficiency coming from the proposed modifications can be inferred by subtracting the result of Table 1 from Table 2. For Intra coding, the proposed modifications bring a 2.1% reduction in luma BD-rate with High-Efficiency (HE) setting and a 2.4% reduction with Low-Complexity (LC) setting. For Random-Access coding, the reduction is 1.1% with HE and 1.0% with LC. For Low-Delay coding, the reduction is 0.3% with HE and 0.3% with LC.

For Intra coding, the increase in encoding and decoding time is a good trade-off considering the substantial improvement in coding efficiency. For the practical scenarios of Random-Access and Low-Delay coding, there is minimal impact on encoding and decoding times while obtaining a significant improvement in coding efficiency.

# Conclusion

The current contribution proposes two modifications (Planar+DST and IMC) to enhance intra coding in HM. The Planar+DST mode has a simpler prediction than the Planar mode and codes the residual using a DST instead of coding the bottom-right pixel value. An added advantage is that the Planar+DST mode does not need the implementation of new transforms or new quantization tables. IMC enhances coding efficiency by allowing up to two candidates in the intra mode prediction.

This contribution leads to a significant reduction in both luma and chroma BD-rates with minimal impact on complexity. We recommend the adoption of this contribution into the HM.

# References

- K. Ugur, K. R. Andersson, A. Fuldseth, “Description of video coding technology proposal by Tandberg, Nokia, Ericsson”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-A119, 1st Meeting: Dresden, Germany, 15-23 April, 2010.

- F. Bossen., “Common conditions and software reference configurations”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-C500, 3rd Meeting: Guangzhou, China, 7-15 October, 2010.

# Patent rights declaration(s)

NTT DOCOMO, Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).