| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D237 WG11 Number: m18997 |
| --- | --- |

| Title: | CE8 Subset2: Verification results of Panasonic’s Proposal JCTVC-D217 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Tomoo Yamakage1 In Suk Chong2 Yu-Wen Huang3 | Email: | tomoo.yamakage@toshiba.co.jp ichong@qualcomm.com yuwen.huang@mediatek.com |
| Source: | TOSHIBA1, Qualcomm2 and MediaTek Inc.3 |  |  |

_____________________________

# Abstract

This contribution shows verification results of JCTVC-D217 (CE 8: Results for adaptive loop filter using prediction and residual (3-Input-ALF)) for CE8 Subset2.  Coding efficiency results are confirmed.  No mismatch between encoder and decoder is observed.  Relative encoding and decoding time have some degree of deviation.  Considering the difference of operating the environments, results are basically confirmed.

# Introduction

This contribution is a report of verification results of JCTVC-D217, which is a contribution to CE8 Subset2.  Verification is performed under the CE8 test conditions that refer to JCTVC-C500.  In addition, source code is reviewed.

# Experimental Results

Detailed results in Excel sheet are attached.

The following table is the summary of experimental results:

| Proponent | Panasonic (Proponent) |  |  | Toshiba (Cross-checker) |  |  | Qualcomm (Cross-checker) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test case | IO | LD | RA | IO | LD | RA | IO | LD | RA |
| Low Complexity | -4.6 | -5.9 | -5.5 | -4.6 | -5.9 | -5.5 | -4.6 | -5.9 | -5.5 |
| Encoding time | 195 | 118 | 119 | 168 | 109 | 109 | 188 | 113 | 114 |
| Decoding time | 216 | 196 | 204 | 235 | 244 | 255 | 255 | 250 | 262 |
| High Efficiency | -0.9 | -0.8 | -0.4 | -0.9 | -0.8 | -0.4 | -0.9 | -0.8 | -0.4 |
| Encoding time | 110 | 104 | 104 | 106 | 99 | 99 | 112 | 101 | 110 |
| Decoding time | 103 | 106 | 107 | 108 | 111 | 114 | 104 | 106 | 108 |

There is no mismatch between encoder and decoder.

There is no mismatch of overall BD-rate between the verification results and the proponent’s results. However, there are slight mismatches of coding results (bitrate and/or PSNR) for a few cases.

Relative encoding and decoding time have some degree of deviation, in particular on decoding time of Low Complexity cases. Note that Toshiba’s results were obtained by identical multicore PCs whose turbo boost mode was disabled by BIOS settings, and decoding image was not output to HDD when measuring decoding time.

A potential reason of these differences is the difference of the operating environments.

# Source Code Reviewing

Source code is reviewed by each company.  It is confirmed that the proposal uses an inverse DCT signal and a prediction signal with the collocated pixel position in addition to the QC_ALF anchor code.

MediaTek has studied the provided source codes. The sources of three-input ALF are the reconstructed pixels after deblocking filter, the predictors from motion compensation or intra prediction, and the residues which are the differences between the predictors and the reconstructed pixels before deblocking filter. In the provided software, only one tap is added for the predictors, and only one tap is added for residues, respectively. Therefore, from the viewpoint of filter operations, only additional two taps are added, compared to QC_ALF, which means the software run time should not be significantly increased. However, for the encoder, the number of required frame buffers is increased from one to three in order to store these variant sources. These will require three times of external memory storage as well as three times of increase in external memory access. Note that memory BW increase is very critical factor HW encoder design. In the decoder side, two more buffers are required to store predictors and residues of a leaf CU for three-input ALF. But further investigations will be needed to consider the implication of having three input instead of one input.

# Conclusion

This contribution shows verification results of JCTVC-D217. Coding efficiency results are confirmed.  No mismatch between encoder and decoder is observed.  Relative encoding and decoding time are basically confirmed.

# Patent rights declaration(s)

MediaTek Inc., Qualcomm, and TOSHIBA Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).