| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D239 WG11 Number: m19000 |
| --- | --- |

| Title: | CE11: Report on Zigzag scan performance for CABAC on TMuC0.9 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Jian Lou Krit Panusopone Limin Wang 6450 Sequence Drive San Diego, CA 92121 USA | Tel: Email: | +1-858-404-3427 lou@motorola.com +1-858-404-3618 krit@motorola.com +1-858-404-3893 limin.wang@motorola.com |
| Source: | Motorola Mobility Inc. |  |  |

_____________________________

# Abstract

This contribution document reports the results of applying zigzag scan for CABAC [1]. The simulations were conducted using TMuC0.9 software [2] with Motorola Mobility’s modifications. The code modifications clean up the redundancies introduced by adaptive scan [3] in order to achieve accurate speed. Bit-exact rate-distortion performance as that provided in [4] is achieved. Cross-check will be provided by Huawei and Qualcomm. Attached spreadsheet contains detailed data of the results.

# Introduction

The target of this document is to verify the performance of zigzag scan in transform coding for high efficiency test conditions. The simulations were conducted using TMuC0.9 software with Motorola Mobility’s modifications. The code modifications clean up the redundancies introduced by adaptive scan [3] in order to achieve accurate speed. Bit-exact rate-distortion performance as that provided in [4] is achieved. The anchor results are obtained with adaptive scan and the tested results are obtained with zigzag scan.

# Simulation environment

## Software

The software used to run the simulations is TMuC 0.9. The binaries were compiled using Microsoft Visual Studio 2008 64bit. The coding conditions used for the simulations are specified in [5]. The macro “MOT_ZIGZAG_SCAN” was introduced and enabled in TypeDef.h. The macro “CE11_SIG_COUNT” provided by TI, was enabled when counting the significance count in another round of decoding, in order not to affect the true decoding time.

## Hardware

All the simulation experiments were conducted on the PCs with identical settings, i.e., Microsoft Windows XP 64bits, Intel Xeon 2.80GHz with 8G memory. Therefore, both the encoding time and decoding time comparisons are reliable.

# Simulation Results

The experimental results of the rate-distortion and speed performance comparison are summarized in Table 1. The experimental results of significance count comparison are summarized in Table 2. Detailed results are included in the attachment (JCTVC-D239.xls).

Table 1 Rate-distortion and speed performance comparison of zigzag scan and adaptive scan

|  | Intra |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.3 | 0.5 | 0.4 |
| Class B | 0.2 | 0.4 | 0.5 |
| Class C | 0.4 | 0.6 | 0.6 |
| Class D | 0.4 | 0.6 | 0.6 |
| Class E | 0.4 | 0.4 | 0.2 |
| All | 0.3 | 0.5 | 0.5 |
| Enc Time[%] | 91% |  |  |
| Dec Time[%] | 99% |  |  |
|  |  | Random access |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.2 | 0.6 | 0.6 |
| Class B | 0.2 | 0.2 | 0.5 |
| Class C | 0.3 | 0.3 | 0.3 |
| Class D | 0.2 | 0.4 | 0.6 |
| Class E |  |  |  |
| All | 0.2 | 0.3 | 0.5 |
| Enc Time[%] | 93% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |
| Class B | 0.2 | 0.3 | 0.3 |
| Class C | 0.2 | 0.2 | 0.2 |
| Class D | 0.2 | -0.4 | 0.6 |
| Class E | 0.1 | 0.6 | 0.6 |
| All | 0.2 | 0.2 | 0.4 |
| Enc Time[%] | 94% |  |  |
| Dec Time[%] | 98% |  |  |

Table 2 Significance count comparison of zigzag scan and adaptive scan

|  | Intra | Random access | Low delay |
| --- | --- | --- | --- |
| Significance count increase | 2.17% | 1.80% | 2.61% |

# References

[1] J. Lou, K. Panusopone, and L. Wang, “Zigzag Scan for CABAC/PIPE,” JCT-VC document, JCTVC-C114, October 2010.

[2] JCT-VC, “Test Model under Consideration Software 0.9”,

https://hevc.hhi.fraunhofer.de/svn/svn_TMuCSoftware/tags/0.9/

[3] Martin Winken, Sebastian Boße, Benjamin Bross, Philipp Helle, Tobias Hinz, Heiner Kirchhoffer, Haricharan Lakshman, Detlev Marpe, Simon Oudin, Matthias Preiß, Heiko Schwarz, Mischa Siekmann, Karsten Sühring, and Thomas Wiegand, “Description of video coding technology proposal by Fraunhofer HHI,” JCT-VC document, JCTVC-A116, April 2010.

[4] J. Lou, K. Panusopone, and L. Wang, “CE11: Cross-check report from Motorola Mobility for HHI's adaptive scan (JCTVC-A116),” JCT-VC document, JCTVC-D236, January 2010.

[5] JCT-VC, “Common test conditions and software reference configurations”, JCT-VC document, JCTVC-C500, October 2010.

# Patent rights declaration(s)

Motorola Mobility Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).