| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D241_r1 WG11 Number: m19002 |
| --- | --- |

| Title: | Parallel processing friendly context modeling for significance map coding in CABAC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jian Lou Krit Panusopone Limin Wang 6450 Sequence Drive San Diego, CA 92121 USA | Tel: Email: | +1-858-404-3427 lou@motorola.com +1-858-404-3618 krit@motorola.com +1-858-404-3893 limin.wang@motorola.com |
| Source: | Motorola Mobility Inc. |  |  |

_____________________________

# Abstract

The scheme used in the current TMuC0.9 [1] for CABAC significance map coding utilizes the nearest neighbors for context modeling [2] in order to achieve better estimation of the probability distribution. Significant dependencies are introduced which prohibit the parallelization of CABAC. This contribution document proposes parallel processing friendly context modeling for significance map coding in CABAC. The proposed scheme is implemented with zigzag scan [3] and it could be extended to other schemes. The experimental results show that there is 0.2% to 0.4% bitrate increase while saving 6% to 9% of the encoding time.

# Introduction

The scheme used in the current TMuC0.9 [1] for CABAC significance map coding utilizes the nearest neighbors for context modeling [2] in order to achieve better estimation of the probability distribution. Significant dependencies are introduced which prohibit the parallelization of CABAC. An example is given in Fig. 1. Assuming the scanning direction is from top right to bottom left along the line of (y-1, x+1), (y, x) and (y+1, x-1), the context of the element at position (y, x) is determined by 10 coded neighbors (y-2, x-1), (y-1, x-2), (y-2, x), (y-1, x-1), (y, x-2), (y-2, x+1), (y-1, x), (y, x-1), (y+1, x-2) and (y-1, x+1). On the decoder side, the element at position (y-1, x+1) should be decoded first, so that the context of (y, x) can be determined. Due to the dependencies between the elements along the same scanning lines, parallel processing for context determination could not be achieved.

Fig. 1 Context selection utilizing coded neighbor information in significance map coding

# Parallel processing friendly context modeling for significance map coding in CABAC

It is proposed to determine the context for one element in high frequency position in the transform domain from the element’s coded neighbors, while excluding the coded neighbors along the same scanning line. An example combining zigzag scan is given in Fig. 2. In this example, the context of the element at position (y, x) is determined by 9 coded neighbors (y-2, x-1), (y-1, x-2), (y-2, x), (y-1, x-1), (y, x-2), (y-2, x+1), (y-1, x), (y, x-1) and (y+1, x-2). The inter scanning line dependency is removed and the contexts for all the elements along the same scanning line could be computed at the same time.

Fig. 2 Parallel processing friendly context selection

# Simulation environment

## Software

The software used to run the simulations is TMuC 0.9. The binaries were compiled using Microsoft Visual Studio 2008 64bit. The coding conditions used for the simulations are specified in [4]. The macro “CE11_SIG_COUNT” provided by TI, was enabled when counting the significance count in another round of decoding, in order not to affect the true decoding time.

## Hardware

All the simulation experiments were conducted on the PCs with identical settings, i.e., Microsoft Windows XP 64bits, Intel Xeon 2.80GHz with 8G memory. Therefore, both the encoding time and decoding time comparisons are reliable.

# Simulation Results

The experimental results of the rate-distortion and speed performance comparison are summarized in Table 1. The experimental results of significance count comparison are summarized in Table 2. Detailed results are included in the attachment (JCTVC-D241.xls).

Table 1 Rate-distortion and speed performance comparison of zigzag scan with the proposed context modeling and adaptive scan

|  | Intra |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.4 | 0.5 | 0.5 |
| Class B | 0.4 | 0.5 | 0.6 |
| Class C | 0.5 | 0.7 | 0.6 |
| Class D | 0.5 | 0.6 | 0.6 |
| Class E | 0.5 | 0.5 | 0.3 |
| All | 0.4 | 0.6 | 0.6 |
| Enc Time[%] | 91% |  |  |
| Dec Time[%] | 99% |  |  |
|  |  | Random access |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.3 | 0.4 | 0.5 |
| Class B | 0.3 | 0.3 | 0.2 |
| Class C | 0.3 | 0.3 | 0.2 |
| Class D | 0.3 | 0.2 | 0.6 |
| Class E |  |  |  |
| All | 0.3 | 0.3 | 0.3 |
| Enc Time[%] | 93% |  |  |
| Dec Time[%] | 101% |  |  |
|  | Low delay |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |
| Class B | 0.2 | 0.2 | 0.3 |
| Class C | 0.2 | 0.2 | 0.3 |
| Class D | 0.3 | 0.0 | 0.5 |
| Class E | 0.1 | -0.3 | -0.2 |
| All | 0.2 | 0.0 | 0.3 |
| Enc Time[%] | 94% |  |  |
| Dec Time[%] | 101% |  |  |

Table 2 Significance count comparison of zigzag scan with the proposed context modeling and adaptive scan

|  | Intra | Random access | Low delay |
| --- | --- | --- | --- |
| Significance count increase | 2.17% | 1.80% | 2.61% |

# Summary

In this contribution document, we propose the parallel processing friendly context modeling for significance map coding in CABAC. It is proposed that the coded neighbors along the same scanning line is excluded from the calculation of the context for one element in high frequency position in the transform domain while utilizing its other coded neighbors. The proposed scheme is implemented with zigzag scan and it could be extended to other schemes. The experimental results show that there is 0.2% to 0.4% bitrate increase while saving 6% to 9% of the encoding time.

# References

[1] JCT-VC, “Test Model under Consideration Software 0.9”,

https://hevc.hhi.fraunhofer.de/svn/svn_TMuCSoftware/tags/0.9/

[2] Martin Winken, Sebastian Boße, Benjamin Bross, Philipp Helle, Tobias Hinz, Heiner Kirchhoffer, Haricharan Lakshman, Detlev Marpe, Simon Oudin, Matthias Preiß, Heiko Schwarz, Mischa Siekmann, Karsten Sühring, and Thomas Wiegand, “Description of video coding technology proposal by Fraunhofer HHI,” JCT-VC document, JCTVC-A116, April 2010.

[3] J. Lou, K. Panusopone, and L. Wang, “Zigzag Scan for CABAC/PIPE,” JCT-VC document, JCTVC-C114, October 2010.

[4] JCT-VC, “Common test conditions and software reference configurations”, JCT-VC document, JCTVC-C500, October 2010.

# Patent rights declaration(s)

Motorola Mobility Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).