| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 |  |  | Document: JCTVC-D242 WG11 Number: m19003 |  |  |
| --- | --- | --- | --- | --- | --- |
| Title: | Cross-check of Sony's simplified context selection (JCTVC-D260) |  |  |  |  |
| Status: | Input Document to JCT-VC |  |  |  |  |
| Purpose: | Core Experiment |  |  |  |  |
| Author(s) or Contact(s): | Vivienne Sze Texas Instruments Inc. | Email: |  | sze@ti.com |  |
| Source: | Texas Instruments Inc. |  |  |  |  |

_____________________________

# Abstract

HHI_TRANSFORM_CODING uses a highly adaptive context modeling approach for significance map, where context selection for significant_coeff_flag depends on coefficients in 10 neighboring positions.  While it provides coding gains between 1.4% to 2.8% Error: Reference source not found, it greatly increases the complexity of context selection when coding significant_coeff_flag. Sony proposes reducing the dependency on neighbors to 5. These modifications were implemented in TMuC-0.9 and provided coding gains up to 0.1%.  This simplified context selection had a negligible effect on number significant_coeff_flag bins.

# Introduction

This document reports the performance evaluation results of Sony’s proposal of simplified context selection for significant_coeff_flag in TMuC-0.9.

# Tool Description

For improved coding efficiency, HHI_TRANSFORM_CODING proposes a highly adaptive context selection approach for significant_coeff_flag in the significance map, where the context selection can depend on the value of the coefficients in 10 neighbouring positions (typically in the left, top-left and top directions) in the transform Error: Reference source not found.    For instance, in Fig. 1 below, the context for position X can depend on positions A to K (note: only either G or K will be required, depending on the direction of the scan). Sony’s simplified context selection reduces the dependency on neighbors from 10 to 5 as shown in Fig 2.  The number of contexts used remains the same.

A

B

C

D

E

F

G

H

I

X

J

K

Fig.1 –Dependencies of context selection for significant_coeff_flag in HHI_TRANFORM_CODING.

B

E

F

H

I

X

Fig.2 –Dependencies of context selection for significant_coeff_flag in Sony’s simplified context selection.

# Experiment Results

The TMuC-0.9 software is used for the evaluation, the simulation platforms is LSF equipped with Intel(R) Xeon(R) CPU X5570@2.93GHz  64 bits Linux machines, the common test conditions and reference configurations specified by the Software AhG are followed.  As part of CE11, the number of significant_coeff_flag was counted, and the change relative to TMuC-0.9 anchor is reported. The significant_coeff_flag % was computed using the formula

(# significant_coeff_flag under test - # significant_coeff_flag in anchor) / # significant_coeff_flag in anchor

A negative % means a reduction in bins.  Only the high efficiency configuration is tested.  With Sony’s simplified context selection has negligible effect on number of significant_coeff_flag bins.

Detailed results in JCTVC-D242xls

|  | Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | % sig count |
| Class A | 0.0 | -0.1 | -0.1 | -0.42% |
| Class B | 0.1 | 0.0 | 0.0 | -0.75% |
| Class C | 0.0 | 0.0 | -0.1 | -0.01% |
| Class D | 0.0 | 0.0 | 0.0 | 0.05% |
| Class E | -0.1 | -0.1 | -0.2 | 0.16% |
| All | 0.0 | 0.0 | -0.1 | -0.19% |
| Enc Time[%] | 103% |  |  |  |
| Dec Time[%] | 111% |  |  |  |
|  | Random access |  |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | % sig count |
| Class A | 0.0 | 0.1 | 0.2 | -0.01% |
| Class B | 0.0 | 0.0 | -0.1 | -0.56% |
| Class C | 0.0 | -0.1 | -0.1 | 0.03% |
| Class D | -0.1 | 0.0 | 0.3 | 0.75% |
| Class E |  |  |  |  |
| All | 0.0 | 0.0 | 0.0 | 0.05% |
| Enc Time[%] | 94% |  |  |  |
| Dec Time[%] | 96% |  |  |  |
|  | Low delay |  |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | % sig count |
| Class A |  |  |  |  |
| Class B | -0.1 | -0.1 | -0.3 | 0.45% |
| Class C | -0.1 | -0.1 | -0.2 | 1.47% |
| Class D | -0.1 | -0.6 | 0.1 | 2.61% |
| Class E | -0.2 | -0.1 | -0.4 | 1.81% |
| All | -0.1 | -0.2 | -0.2 | 1.58% |
| Enc Time[%] | 93% |  |  |  |
| Dec Time[%] | 96% |  |  |  |

Table 1  Average gain allows for three fixed scan patterns in Samsung’s proposal versus adaptive scan order in HHI_TRANSFORM_CODING. Negative numbers in the table means coding gain.

# Summary

Sony’s simplified context selection for significant_coeff_flag 0% to 0.1% coding gain over HM1 and has negligible effect on the number of significant_coeff_flag bins.

# References

- “CE11: Coefficient scanning and coding”, JCTVC-C511, 3rd JCT-VC Meeting, Guangzhou, CN, October 2010.