| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 |  |  | Document: JCTVC-D244 WG11 Number: m19005 |  |  |
| --- | --- | --- | --- | --- | --- |
| Title: | Context selection complexity in HEVC CABAC |  |  |  |  |
| Status: | Input Document to JCT-VC |  |  |  |  |
| Purpose: | Information |  |  |  |  |
| Author(s) or Contact(s): | Vivienne Sze Texas Instruments Inc. | Email: |  | sze@ti.com |  |
| Source: | Texas Instruments Inc. |  |  |  |  |

_____________________________

# Abstract

CABAC is a well known throughput bottleneck in the video codec implementations.  This is due to the many feedback loops in the CABAC, several of which are in the context selection.  With the introduction of HHI Transform Coding into HM1, the data dependencies in context selection have become even stronger.  The number of contexts for significance map in HEVC is almost 2x more than AVC, thus tightening this bottleneck in the context selection of CABAC. This also increases implementation area cost, as the context memory needs to be larger.

# Bottlenecks in CABAC

CABAC is a well known throughput bottleneck in the video codec implementations (particularly at the decoder-side).  This is due to the many feedback loops in the CABAC, several of which are in the context selection. Fig. 1 highlights the feedback loops within the CABAC in both the encoder and decoder.  First, the value of the context is updated based on the previously encoded/decoded bins.  Second, the context to be chosen depends on the previously encoded/decoded bins as well as various factors such as macroblock type, binIdx, transform size, location in transform, etc.  At the encoder, this loop is contained in the context selection; at the decoder, this second feedback loop passes through the bin decoder.  Thus parallelism is a particular challenge at the decoder.

These loops make the CABAC difficult to parallelize, which makes it difficult to achieve the high throughput necessary for high resolution and frame-rate videos.  Furthermore, since high throughput can be traded-off for power savings using voltage scaling, the serial nature of CABAC limits the battery life for video codecs that reside on mobile devices.  This limitation is a critical concern, as a significant portion of future video codecs will be battery-operated.

- encoder

- decoder

Fig.1. The CABAC is composed of three key blocks: Binarizer, Context Modeler and Bin Coder (i.e. Arithmetic Coder). Feedback loops are highlighted with dashed lines.

## Speculative Computations for multi-bin decoding

A common technique used in practice is speculative computation which aims to increase the throughput of CABAC.   However, this comes as a cost of increased number of computations per bin.    For a two bin per cycle example: in order to decode bin2 in parallel with bin1 requires 2x computations for bin2 to account for the case that bin1=0 and bin1=1.  In particular, the number of context selection computations increases.  The number of operations increases exponentially as we increase the number of bins that are to be processed in parallel. (e.g. for N bins to be decoded in parallel, we need up to 1+2+4+…+2(N-1) context selection computations)

To reduce the cycle time (i.e critical path), the CABAC needs to be pipelined. For instance, the 4-stage pipeline shown in Fig. 2 is commonly used for AVC CABAC implementations [2].  Due to the feedback loops, each inserted pipeline stage between context selection and bin decoder increases the number of context selection computations by 2x.  For the architecture in Fig. 2, the number of context selections operations grows to by a factor of 4x, which means a 2-bin per cycle pipelined solution requires 12x context selection computations.

Fig. 2 Common pipelined architecture for CABAC. Context modeling blocks in grey.  Feedback loops highlight with dashed lines.

Thus the total increase in computations for context selection for an M-stage pipelined architecture decoding N-bins per cycles is 2(M-1) *(1+2+4+…+2(N-1) ). Often optimizations are done in an attempt to reduce the total computations.

# Additional Challenges in HEVC CABAC

HHI transform coding [1] is a new tool in HM1. While it provides additional coding gains, it increases the dependencies within context selection of the CABAC as shown in Fig. 3.  Specifically, it introduces additional feedback loops in the context selection of significance_coeff_flag and coeff_abs_level_minus1, tightening this bottleneck in the context selection of CABAC.  Each context selection will require additional computations to account for new dependencies.  This will make it much more difficult to reduce the total number of computations for the multi-bin approach describe in the previous section.  It is important to note that the bins for significance_coeff_flag and coeff_abs_level_minus1 tend to dominate, making it much more difficult to increase the throughput of CABAC.  Furthermore, the number of contexts for significance map in HEVC is almost 2x more than AVC which increases implementation area cost, as the context memory needs to be larger.

Fig. 3. Components of AVC context modeler shown in black.  Additional feedback loops due to HHI Transform Coding in red.

# Summary

In this contribution, we highlight the context selection bottleneck in CABAC.  Furthermore, we describe how this bottleneck is tightened by the new HHI transform coding tool.

# References

- “Description of video coding technology proposal by Fraunhofer HHI”, JCTVC-A116, 1st JCT-VC Meeting, Dresden, DE, April 2010.

- Y. Yi,  I.-C. Park , "High-Speed H.264/AVC CABAC Decoding," IEEE Trans. CSVT April 2007