| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D245 WG11 Number: m19006 |
| --- | --- |

| Title: | Verification of TI's proposal on matrix multiplication architecture for DCT/IDCT |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Jian Lou Limin Wang 6450 Sequence Drive San Diego, CA 92121 USA | Tel: Email: | +1-858-404-3427 lou@motorola.com +1-858-404-3893 limin.wang@motorola.com |
| Source: | Motorola Mobility Inc. |  |  |

_____________________________

# Abstract

This contribution document contains cross-check results of TI’s proposal on matrix multiplication architecture for DCT/IDCT [1]. The simulations were conducted using TMuC0.9 software [2] with modifications provided by TI. The cross-check has been completed successfully and the results match those provided by TI with minor differences possibly due to different compiler and OS. Attached spreadsheet contains detailed data of the results.

# Introduction

The target of this document is to verify the performance of TI’s proposal on matrix multiplication architecture for DCT/IDCT [1]. The simulations were conducted using TMuC0.9 software with TI’s modifications. The anchor results are obtained with default DCT/IDCT architecture in TMuC0.9 and the tested results are obtained with the proposed matrix multiplication architecture for DCT/IDCT.

# Simulation environment

## Software

The software used to run the simulations is TMuC 0.9. The binaries were compiled using Microsoft Visual Studio 2008 64bit. The coding conditions used for the simulations are specified in [3]. The macros “DCT_MATRIX”, “DCT_MATRIX_FAST”, “DCT_MATRIX_EVENODD_LEVEL2_DECOMPOSITION”, and “IDCT_MATRIX_EVENODD_LEVEL2_DECOMPOSITION” provided by TI were enabled in TypeDef.h. YUV file output during the decoding was disabled.

## Hardware

All the simulation experiments were conducted on the PCs with identical settings, i.e., Microsoft Windows XP 64bits, Intel Xeon 2.80GHz with 8G memory. Therefore, both the encoding time and decoding time comparisons are reliable.

# Simulation Results

The experimental results of the rate-distortion and speed performance comparison are summarized in Table 1. Detailed results are included in the attachment (JCTVC-D245.xls).

Table 1 Rate-distortion and speed performance comparison of matrix multiplication DCT/IDCT

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | -0.1 | -0.2 | -0.1 | 0.5 | 0.5 |
| Class B | 0.0 | 0.0 | -0.1 | 0.0 | 0.0 | 0.1 |
| Class C | 0.0 | 0.0 | -0.1 | 0.0 | 0.1 | 0.1 |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | 0.1 | 0.1 |
| Class E | -0.1 | 0.0 | -0.2 | 0.0 | 0.3 | 0.3 |
| All | 0.0 | 0.0 | -0.1 | 0.0 | 0.1 | 0.2 |
| Enc Time[%] | 102% |  |  | 107% |  |  |
| Dec Time[%] | 102% |  |  | 104% |  |  |
|  |  | Random access |  |  | Random access LoCo |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.1 | 0.0 | -0.1 | 0.5 | 0.3 |
| Class B | -0.1 | -0.1 | 0.0 | -0.1 | 0.2 | 0.0 |
| Class C | 0.0 | -0.2 | -0.1 | 0.0 | 0.3 | 0.2 |
| Class D | 0.0 | 0.1 | 0.2 | 0.0 | -0.2 | 0.3 |
| Class E |  |  |  |  |  |  |
| All | 0.0 | 0.0 | 0.0 | -0.1 | 0.1 | 0.1 |
| Enc Time[%] | 101% |  |  | 103% |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | 0.1 | -0.3 | -0.1 | 0.2 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.2 |
| Class D | 0.1 | -0.3 | -0.1 | -0.1 | 0.1 | -0.1 |
| Class E | -0.2 | -0.1 | -0.6 | -0.3 | -0.6 | 0.1 |
| All | 0.0 | 0.0 | -0.3 | -0.1 | 0.0 | 0.0 |
| Enc Time[%] | 101% |  |  | 102% |  |  |
| Dec Time[%] | 102% |  |  | 103% |  |  |

# References

[1] M. Sadafale and M. Budagavi, “Low-complexity, configurable transform architecture for HEVC,” JCT-VC document, JCTVC-C226, October 2010.

[2] JCT-VC, “Test Model under Consideration Software 0.9”,

https://hevc.hhi.fraunhofer.de/svn/svn_TMuCSoftware/tags/0.9/

[3] JCT-VC, “Common test conditions and software reference configurations”, JCT-VC document, JCTVC-C500, October 2010.

# Patent rights declaration(s)

Motorola Mobility Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).