| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D246 WG11 Number: m19007 |
| --- | --- |

| Title: | New DCT-based interpolation filters |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jian Lou Limin Wang 6450 Sequence Drive San Diego, CA 92121 USA | Tel: Email: | +1-858-404-3427 lou@motorola.com +1-858-404-3893 limin.wang@motorola.com |
| Source: | Motorola Mobility Inc. |  |  |

_____________________________

# Abstract

DCT-based interpolation filters [1] and their low complexity optimization were introduced by Samsung in the current TMuC0.9 [2]. Each fractional pixel position is interpolated by one filter to save complexity. The 12tap DCT-IF is now used for the Low-Delay High Efficiency tests and the Random-Access High Efficiency tests. This contribution document proposes the new DCT-based interpolation filters using the method from [3]. Compared to the current interpolation filters in TMuC, better Rate-Distortion performance could be achieved with the same tap length, or lower complexity could be achieved with the same Rate-Distortion performance.

# Introduction

DCT-based interpolation filters [1] and their low complexity optimization were introduced by Samsung in the current TMuC0.9 [2]. Each fractional pixel position is interpolated by one filter to save complexity. The 12tap DCT-IF is now used for the Low-Delay High Efficiency tests and the Random-Access High Efficiency tests.

The DCT-based interpolation filters from [1], either the original forms or the low complexity forms, are different from those derived from the method proposed in [3], though the same types of DCT (DCT-II and DCT-III) are used. It could be possibly caused by lack of accuracy, different scaling or other reasons.

This contribution document proposes the new DCT-based interpolation filters using the method from [3]. Compared to the current interpolation filters in TMuC, better Rate-Distortion performance could be achieved with the same tap length, or lower complexity could be achieved with the same Rate-Distortion performance.

# DCT-based interpolation filter design

DCT is one of the most popular transforms used in video signal processing applications. This is because DCT exhibits properties similar to the optimal Karhunen-Loeve Transform (KLT) [4]. As DCT can reflect the characteristics of images, with a suitable modification of DCT multiplying coefficients, alias-free image zooming can be achieved [5].

The (i, j)th coefficient of m×m DCT, DCTm(i, j), is

DCTm(i,j)=1mi=0,0≤j≤m−1cos(2i+12mjπ)m21≤i≤m−1,0≤j≤m−1					(1)

The forward transform for an m×n image, fm×n, is

Fm×n=DCTm×fm×n×DCTnT									(2)

where Fm×n stands for the transformed signal. In order to change image size from m×n to k×l, transformed signal, Fk×l, should be obtained from Fm×n.

1) Coefficient discarding.

If k > m, then

Fk×l'(i,j)=Fm×n(i,j)0≤j≤m−1,0≤i≤k−10otherwise						(3)

where zero padding is used in order not to introduce extra high frequency components.

2) Coefficient scaling.

This step is needed for the sake of keeping the dynamic range for output image since the normalized m×m DCT matrix contains the factor of 1m.

Fk×l(i,j)=Fk×l'(i,j)×k×lm×n									(4)

The final output image with the size of k×l is obtained by

fk×l=DCTkT×Fk×l×DCTl									(5)

The above algorithm can achieve an accurate image zooming without aliasing or blocking.

Even though the algorithm can be used for interpolation while overcoming the defect of aliasing, it is too complicated for practical implementation. This can be modified by using two 1-D DCTs and two 1-D IDCTs separately with less input pixels [6]. However, DCT and IDCT operations still need considerable computational and memory resources. Interpolation using filters are more preferable for the consideration of complexity and performance. The DCT-based Fractional-pel Interpolation Filter design only considers the linear relation between the integer pixels and interpolated fractional pixels.

Fig.1 shows 1/k-pel interpolation based-on DCT. The original image is fm×n and k-1 fractional pixels are interpolated between every two integer pixels. Therefore, the interpolated image contains (mk−k+1)×(nk−k+1) pixels, denoted as f(mk−k+1)×(nk−k+1). The icon ※ in Fig.1 is used for the interpolated fractional pixel I in the central area of the image. The DCT-based Fractional-pel Interpolation Filter design only considers the contribution of those integer pixels for I, thus obtaining the taps of the interpolation filter.

Briefly, the general rule for The DCT-based Fractional-pel Interpolation Filter design is to transform the original image, fm×n, into transform domain by m×n DCT and obtain Fm×n first. Then the coefficients are padded and scaled to get F(mk−k+1)×(nk−k+1). After that, inverse transform F(mk−k+1)×(nk−k+1) into spatial domain by (mk−k+1)×(nk−k+1) IDCT. Finally, get the filter taps of those integer pixels to interpolate the specified fractional pixel. More detailed derivation and analysis could be found in [3].

- Fig.1 1/k-pel Interpolation based-on DCT

# New DCT-based interpolation filters

Following the same framework, but with the derivation in section 2, new DCT-based interpolation filters could be derived. Fig. 2 illustrates the quarter-pel positions and table 1 gives the DCT-based interpolation filter coefficients with the same bit depth as [1].

- Fig.2 Quarter-Pel Positions

- Table 1 New DCT-based interpolation filter coefficients

| Number of Filter Taps | Position | Coefficients |
| --- | --- | --- |
| 4 | ¼ | { -19, 236, 50, -11,} |
|  | ½ | { -27, 155, 155, -27,} |
|  | ¾ | { -11, 50, 236, -19,} |
| 6 | ¼ | { 8, -33, 235, 61, -21, 6,} |
|  | ½ | { 12, -43, 159, 159, -43, 12,} |
|  | ¾ | { 6, -21, 61, 235, -33, 8,} |
| 8 | ¼ | { -5, 16, -38, 235, 65, -25, 12, -4,} |
|  | ½ | { -6, 21, -48, 161, 161, -48, 21, -6,} |
|  | ¾ | { -4, 12, -25, 65, 235, -38, 16, -5,} |
| 10 | ¼ | { 3, -9, 19, -40, 234, 68, -28, 15, -8, 2,} |
|  | ½ | { 4, -13, 25, -50, 162, 162, -50, 25, -13, 4,} |
|  | ¾ | { 2, -8, 15, -28, 68, 234, -40, 19, -9, 3,} |
| 12 | ¼ | { -2, 6, -12, 21, -42, 234, 70, -29, 17, -10, 5, -2,} |
|  | ½ | { -3, 9, -16, 28, -52, 162, 162, -52, 28, -16, 9, -3,} |
|  | ¾ | { -2, 5, -10, 17, -29, 70, 234, -42, 21, -12, 6, -2,} |

As can be seen from the results in Section 4, compared to the current interpolation filters in TMuC, the proposed new DCT-IF can achieve better Rate-Distortion performance with the same tap length. On the other hand, lower complexity could be achieved with the same Rate-Distortion performance. The complexity reduction includes fewer arithmetic operations and fewer data fetching operations. Take the new 10tap filter for example, compared to the 12tap filter in TMuC, it has comparable same rate-distortion performance with shorter filter length, therefore it can reduce the data fetching complexity by 24.89% to 5.26% for different block sizes. Table 2 gives the numbers of integer pixels to be fetched for different block sizes with different filter taps.

|  | 4x4 |  | 8x8 |  | 16x16 |  | 32x32 |  | 64x64 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Integer Pel # | % reduction compared to 12tap | Integer Pel # | % reduction compared to 12tap | Integer Pel # | % reduction compared to 12tap | Integer Pel # | % reduction compared to 12tap | Integer Pel # | % reduction compared to 12tap |
| 6Tap | 9x9 = 81 | 64.00% | 13x13 = 169 | 53.19% | 21x21 = 441 | 39.51% | 37x37 = 1369 | 25.96% | 69x69 = 4761 | 15.36% |
| 8Tap | 11x11 = 121 | 46.22% | 15x15 = 225 | 37.67% | 23x23 = 529 | 27.43% | 39x39 = 1521 | 17.74% | 71x71 = 5041 | 10.38% |
| 10Tap | 13x13 = 169 | 24.89% | 17x17 = 289 | 19.94% | 25x25 = 625 | 14.27% | 41x41 = 1681 | 9.09% | 73x73 = 5329 | 5.26% |
| 12Tap | 15x15 = 225 | 0 | 19x19 = 361 | 0 | 27x27 = 729 | 0 | 43x43 = 1849 | 0 | 75x75 = 5625 | 0 |

- Table 3 Numbers of integer pixels to be fetched for different block sizes

# Simulation environment

## Software

The software used to run the simulations is TMuC 0.9. The binaries were compiled using Microsoft Visual Studio 2008 64bit. The coding conditions used for the simulations are specified in [4].

## Hardware

Not all the simulation experiments were conducted on the PCs with identical settings. However, for some tests (anchor and the proposed 10tap filter without YUV output), PCs with same identical settings, i.e., Microsoft Windows XP 64bits, Intel Xeon 2.80GHz with 8G memory, are used to get reliable encoding time and decoding time comparisons.

# Experimental results

The experimental results of the rate-distortion performance comparison are summarized in Table 3. The average rate-distortion performance comparison is given in Fig. 3. Detailed results are included in the attachment (JCTVC-D246.xls).

Compared to the current interpolation filters in TMuC, the proposed new DCT-IF can achieve better Rate-Distortion performance with the same tap length or lower complexity could be achieved with the same Rate-Distortion performance. The new 10 tap filter can achieve the same rate-distortion performance as the TMuC 12tap filter, but reduce the encoding time by 6% and the decoding time by 3% with Random Access High Efficiency test condition and reduce the encoding time by 3% and the decoding time by 3% with Low Delay High Efficiency test condition.

| TMuC6Tap |  |  |  |  |  |  |  |  |  | MMI6Tap |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Random access |  |  |  |  |  |  |  |  |  | Random access |  |  |  |
|  |  | Y BD-rate |  | U BD-rate |  | V BD-rate |  |  |  |  |  | Y BD-rate |  | U BD-rate |  | V BD-rate |  |
| Class A |  | 0.12 |  | 0.05 |  | 0.18 |  |  |  | Class A |  | -0.08 |  | 0.11 |  | 0.10 |  |
| Class B |  | 0.38 |  | 0.12 |  | 0.08 |  |  |  | Class B |  | 0.25 |  | 0.23 |  | 0.14 |  |
| Class C |  | 2.40 |  | 0.98 |  | 1.14 |  |  |  | Class C |  | 1.82 |  | 0.91 |  | 0.89 |  |
| Class D |  | 4.69 |  | 2.99 |  | 2.96 |  |  |  | Class D |  | 3.49 |  | 2.09 |  | 2.32 |  |
| Class E |  |  |  |  |  |  |  |  |  | Class E |  |  |  |  |  |  |  |
| All |  | 2.03 |  | 1.10 |  | 1.14 |  |  |  | All |  | 1.49 |  | 0.89 |  | 0.91 |  |
| Enc Time[%] |  |  |  |  |  |  |  |  | Enc Time[%] |  |  |  |  |  |  |  |  |
| Dec Time[%] |  |  |  |  |  |  |  |  | Dec Time[%] |  |  |  |  |  |  |  |  |
|  | Low delay |  |  |  |  |  |  |  |  |  | Low delay |  |  |  |  |  |  |
|  | Y BD-rate |  | U BD-rate |  | V BD-rate |  |  |  |  |  | Y BD-rate |  | U BD-rate |  | V BD-rate |  |  |
| Class A |  |  |  |  |  |  |  |  | Class A |  |  |  |  |  |  |  |  |
| Class B | 0.48 |  | 0.53 |  | 0.27 |  |  |  | Class B |  | 0.43 |  | 0.36 |  | 0.40 |  |  |
| Class C | 2.62 |  | 2.40 |  | 2.40 |  |  |  | Class C |  | 2.56 |  | 2.45 |  | 2.42 |  |  |
| Class D | 4.66 |  | 5.21 |  | 5.69 |  |  |  | Class D |  | 4.18 |  | 4.48 |  | 5.66 |  |  |
| Class E | 0.86 |  | 0.16 |  | 0.45 |  |  |  | Class E |  | 0.98 |  | 1.59 |  | 1.12 |  |  |
| All | 2.13 |  | 2.10 |  | 2.19 |  |  |  | All |  | 2.00 |  | 2.14 |  | 2.36 |  |  |
| Enc Time[%] |  |  |  |  |  |  |  |  | Enc Time[%] |  |  |  |  |  |  |  |  |
| Dec Time[%] |  |  |  |  |  |  |  |  | Dec Time[%] |  |  |  |  |  |  |  |  |
| TMuC8Tap |  |  |  |  |  |  |  |  | MMI8Tap |  |  |  |  |  |  |  |  |
|  |  |  | Random access |  |  |  |  |  |  |  |  |  | Random access |  |  |  |  |
|  | Y BD-rate |  | U BD-rate |  | V BD-rate |  |  |  |  |  | Y BD-rate |  | U BD-rate |  | V BD-rate |  |  |
| Class A | -0.01 |  | -0.04 |  | 0.02 |  |  |  | Class A |  | 0.05 |  | 0.19 |  | 0.28 |  |  |
| Class B | -0.03 |  | -0.13 |  | -0.15 |  |  |  | Class B |  | -0.12 |  | -0.06 |  | 0.14 |  |  |
| Class C | 0.84 |  | 0.35 |  | 0.34 |  |  |  | Class C |  | 0.49 |  | 0.33 |  | 0.25 |  |  |
| Class D | 1.38 |  | 0.88 |  | 0.93 |  |  |  | Class D |  | 0.64 |  | 0.67 |  | 0.83 |  |  |
| Class E |  |  |  |  |  |  |  |  | Class E |  |  |  |  |  |  |  |  |
| All | 0.58 |  | 0.28 |  | 0.29 |  |  |  | All |  | 0.27 |  | 0.27 |  | 0.37 |  |  |
| Enc Time[%] |  |  |  |  |  |  |  |  | Enc Time[%] |  |  |  |  |  |  |  |  |
| Dec Time[%] |  |  |  |  |  |  |  |  | Dec Time[%] |  |  |  |  |  |  |  |  |
|  | Low delay |  |  |  |  |  |  |  |  |  | Low delay |  |  |  |  |  |  |
|  | Y BD-rate |  | U BD-rate |  | V BD-rate |  |  |  |  |  | Y BD-rate |  | U BD-rate |  | V BD-rate |  |  |
| Class A |  |  |  |  |  |  |  |  | Class A |  |  |  |  |  |  |  |  |
| Class B | -0.20 |  | -0.01 |  | 0.03 |  |  |  | Class B |  | -0.15 |  | 0.19 |  | -0.01 |  |  |
| Class C | 0.97 |  | 0.95 |  | 1.00 |  |  |  | Class C |  | 0.66 |  | 0.52 |  | 0.58 |  |  |
| Class D | 0.87 |  | 1.32 |  | 1.75 |  |  |  | Class D |  | 0.19 |  | 0.86 |  | 1.31 |  |  |
| Class E | 0.21 |  | -0.47 |  | 0.07 |  |  |  | Class E |  | 0.39 |  | 0.72 |  | 0.36 |  |  |
| All | 0.44 |  | 0.48 |  | 0.71 |  |  |  | All |  | 0.24 |  | 0.54 |  | 0.54 |  |  |
| Enc Time[%] |  |  |  |  |  |  |  |  | Enc Time[%] |  |  |  |  |  |  |  |  |
| Dec Time[%] |  |  |  |  |  |  |  |  | Dec Time[%] |  |  |  |  |  |  |  |  |
| TMuC10Tap |  |  |  |  |  |  |  |  | MMI10Tap |  |  |  |  |  |  |  |  |
|  |  |  | Random access |  |  |  |  |  |  |  |  |  | Random access |  |  |  |  |
|  | Y BD-rate |  | U BD-rate |  | V BD-rate |  |  |  |  |  | Y BD-rate |  | U BD-rate |  | V BD-rate |  |  |
| Class A | 0.35 |  | 0.15 |  | 0.35 |  |  |  | Class A |  | -0.09 |  | 0.13 |  | 0.24 |  |  |
| Class B | 0.48 |  | 0.17 |  | -0.02 |  |  |  | Class B |  | -0.10 |  | 0.01 |  | 0.03 |  |  |
| Class C | 0.59 |  | 0.06 |  | 0.02 |  |  |  | Class C |  | -0.02 |  | 0.08 |  | 0.07 |  |  |
| Class D | 1.29 |  | 0.57 |  | 0.58 |  |  |  | Class D |  | 0.13 |  | 0.38 |  | 0.50 |  |  |
| Class E |  |  |  |  |  |  |  |  | Class E |  |  |  |  |  |  |  |  |
| All | 0.71 |  | 0.24 |  | 0.20 |  |  |  | All |  | -0.02 |  | 0.15 |  | 0.20 |  |  |
| Enc Time[%] |  |  |  |  |  |  |  |  | Enc Time[%] |  | 94% |  |  |  |  |  |  |
| Dec Time[%] |  |  |  |  |  |  |  |  | Dec Time[%] |  | 97% |  |  |  |  |  |  |
|  | Low delay |  |  |  |  |  |  |  |  |  | Low delay |  |  |  |  |  |  |
|  | Y BD-rate |  | U BD-rate |  | V BD-rate |  |  |  |  |  | Y BD-rate |  | U BD-rate |  | V BD-rate |  |  |
| Class A |  |  |  |  |  |  |  |  | Class A |  |  |  |  |  |  |  |  |
| Class B | 0.93 |  | 0.46 |  | 0.17 |  |  |  | Class B |  | 0.07 |  | 0.05 |  | 0.13 |  |  |
| Class C | 1.05 |  | 0.61 |  | 0.69 |  |  |  | Class C |  | 0.15 |  | 0.09 |  | 0.17 |  |  |
| Class D | 1.83 |  | 0.99 |  | 1.49 |  |  |  | Class D |  | 0.28 |  | -0.09 |  | 0.59 |  |  |
| Class E | 2.10 |  | 0.54 |  | 0.45 |  |  |  | Class E |  | 0.12 |  | 0.65 |  | 1.01 |  |  |
| All | 1.40 |  | 0.65 |  | 0.68 |  |  |  | All |  | 0.15 |  | 0.14 |  | 0.42 |  |  |
| Enc Time[%] |  |  |  |  |  |  |  |  | Enc Time[%] |  | 97% |  |  |  |  |  |  |
| Dec Time[%] |  |  |  |  |  |  |  |  | Dec Time[%] |  | 97% |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | MMI12Tap |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | Random access |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | Y BD-rate |  | U BD-rate |  | V BD-rate |  |  |
|  |  |  |  |  |  |  |  |  | Class A |  | 0.06 |  | 0.08 |  | 0.26 |  |  |
|  |  |  |  |  |  |  |  |  | Class B |  | -0.14 |  | -0.08 |  | -0.21 |  |  |
|  |  |  |  |  |  |  |  |  | Class C |  | -0.41 |  | -0.14 |  | -0.16 |  |  |
|  |  |  |  |  |  |  |  |  | Class D |  | -0.75 |  | -0.15 |  | -0.21 |  |  |
|  |  |  |  |  |  |  |  |  | Class E |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | All |  | -0.35 |  | -0.09 |  | -0.14 |  |  |
|  |  |  |  |  |  |  |  |  | Enc Time[%] |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | Dec Time[%] |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | Low delay |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | Y BD-rate |  | U BD-rate |  | V BD-rate |  |  |
|  |  |  |  |  |  |  |  |  | Class A |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | Class B |  | 0.17 |  | -0.19 |  | -0.33 |  |  |
|  |  |  |  |  |  |  |  |  | Class C |  | -0.61 |  | -0.61 |  | -0.55 |  |  |
|  |  |  |  |  |  |  |  |  | Class D |  | -1.58 |  | -1.38 |  | -1.20 |  |  |
|  |  |  |  |  |  |  |  |  | Class E |  | 1.45 |  | 0.36 |  | 0.15 |  |  |
|  |  |  |  |  |  |  |  |  | All |  | -0.22 |  | -0.49 |  | -0.51 |  |  |
|  |  |  |  |  |  |  |  |  | Enc Time[%] |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | Dec Time[%] |  |  |  |  |  |  |  |  |

- Table 3 Rate-distortion performance comparison of TMuC DCT-IF and the proposed DCT-IF

- Fig.3 Average rate-distortion performance comparison

# Summary

DCT-based interpolation filters and their low complexity optimization were introduced by Samsung in the current TMuC0.9. A 12tap DCT-IF is currently used for the Low-Delay High Efficiency tests and the Random-Access High Efficiency tests. This contribution document proposes a new DCT-based interpolation filters. Compared to the current interpolation filters in TMuC, better Rate-Distortion performance could be achieved with the same tap length, or lower complexity could be achieved with the same Rate-Distortion performance. The new 10 tap filter can achieve the same rate-distortion performance as the TMuC 12tap filter, but reduce the encoding time by 6% and the decoding time by 3% with Random Access High Efficiency test condition and reduce the encoding time by 3% and the decoding time by 3% with Low Delay High Efficiency test condition.

# References

[1] K. McCann, W.-J. Han, and I.-K. Kim, “Samsung’s Response to the Call for Proposals on Video Compression Technology,” JCT-VC document, JCTVC-A124, April 2010.

[2] JCT-VC, “Test Model under Consideration Software 0.9”,

https://hevc.hhi.fraunhofer.de/svn/svn_TMuCSoftware/tags/0.9/

[3] Jian Lou, “The theory, implementation and application of Discrete Cosine Transform and Integer Cosine Transform”, M.E. thesis, Zhejiang University, Hangzhou, China, 2004.

[4] N. Ahmed, T. Natarajan, K. R. Rao, “Discrete Cosine Transform,” IEEE Trans. Computers, vol. C-23, pp. 90-93, Jan 1974

[5] J. I. Agbinya, “Two dimensional interpolation of real sequences using the DCT”, IEEE Electronic Letters, vol. 29, pp. 204-205, 1993

[6] J. I. Agbinya, “Interpolation Using the Discrete Cosine Transform”, IEEE Electronic Letters, vol. 28, pp. 1927-1928, 1992

[7] JCT-VC, “Common test conditions and software reference configurations”, JCT-VC document, JCTVC-C500, October 2010.

# Patent rights declaration(s)

Motorola Mobility Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).