| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D247 |
| --- | --- |

| Title: | CE1: Cross check with JCTVC-C501(3.1.2) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Yue Yu Krit Panusopone Limin Wang 6450 Sequence Drive San Diego, CA 92121 USA | Tel: Email: | +1-858-404-2251 yyu@motorola.com +1-858-404-3618 krit@motorola.com +1-858-404-3893 limin.wang@motorola.com |
| Source: | Motorola Mobility |  |  |

_____________________________

# Abstract

Core Experiment 1 [1] is set up at the Guangzhou meeting related to Decoder-Side Motion Vector Derivation (DMVD). As part of CE1, template matching based DMVD is tested. The simulation results are cross checked with Huawei. From the simulation results, it is shown that the template matching based DMVD can provide 0.9% and 0.5% gains for low delay and low delay loco categories, respectively. However, both encoding and decoding time also increase a lot.

- Experimental Results

## Test conditions

Huawei provides the source code. In the simulations, some flags are set as follows. HW_LCDMVD

HW_LCDMVD		1

HW_DMVD		1

HW_STDM		0

Other test conditions will be same as those specified in the TMuC 0.9 version [2].

## Test Platforms

The executable is built with Windows 64 bits Visual Studio 9.

## Performance comparison

Table I

|  | Low delay |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-Rate | U BD-Rate | V BD-Rate | Y BD-Rate | U BD-Rate | V BD-Rate |
| Class A | -0.9 | -1.0 | -1.5 | -0.5 | -0.3 | -0.3 |
| Class B | -1.0 | -1.1 | -1.3 | -0.3 | -0.7 | -0.7 |
| Class C | -0.9 | -1.5 | -1.1 | -0.2 | -0.5 | -0.4 |
| Class D | -1.7 | -3.4 | -3.5 | -0.8 | -0.3 | -0.5 |
| Class E | -1.1 | -1.6 | -1.7 | -0.4 | -0.5 | -0.5 |
| All | -0.9 | -1.0 | -1.5 | -0.5 | -0.3 | -0.3 |
| Enc Time[%] | 178% |  |  | 180% |  |  |
| Dec Time[%] | 114% |  |  | 124% |  |  |

The negative numbers shown in the above table mean that the template matching based DMVD requires less bitrate to achieve the same PSNR. In contrast, the positive numbers mean that it requires more bitrate to achieve the same PSNR.

Regarding time measurement, because the simulations may run at different platforms due to lack of enough computation resources at very limited time frame, the numbers shown in the tables may not be accurate.

- Conclusion

As part of CE1, template matching based DMVD is tested. Compared with anchors, the tool can provide 0.9% and 0.5% gains for low delay and low delay loco categories, respectively. However, both encoding and decoding time also increase a lot to achieve this performance improvement.

# References

[1] Yi-Jen Chiu, Haoping Yu, Yu-Wen Huang, and Shun-ichi Sekiguchi, " Core Experiment 1: Decoder-Side Motion Vector Derivation ", JCT_VC Document JCTVC-C501, Oct. 2010.

[2] F. Bossen, “Common test conditions and software reference configurations,” JCT_VC Document JCTVC-C500, Oct. 2010.

Motorola Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).