| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D248 |
| --- | --- |

| Title: | CE9: Cross check with JCTVC-C509(3.1.b) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Yue Yu Krit Panusopone Limin Wang 6450 Sequence Drive San Diego, CA 92121 USA | Tel: Email: | +1-858-404-2251 yyu@motorola.com +1-858-404-3618 krit@motorola.com +1-858-404-3893 limin.wang@motorola.com |
| Source: | Motorola Mobility |  |  |

_____________________________

# Abstract

Core Experiment 9 [1] is set up at the Guangzhou meeting related to motion vector coding. As part of CE9, modified skip and direct are tested. The simulation results are cross checked with HHI. From the simulation results, it is shown that the modified skip and direct can provide gains ranging from 0.2% to 0.6% for random access, random access loco and low delay categories; however, there is average 0.2% loss for low delay loco category.

- Experimental Results

## Test conditions

In the simulations, some flags are set as follows.

SAMSUNG_MRG_SKIP_DIRECT			1

HHI_MRG_ONLY_ZERO_CORNER_COL_FOR_SKIP	1

HHI_AMVP_NO_CORNER_FOR_SKIP		0

HHI_AMVP_NO_COL_FOR_SKIP			0

HHI_AMVP_NO_ZERO_FOR_SKIP			0

#define HHI_MRG			1

#define HHI_MRG_PU			1

#define HHI_MRG_COLLOCATED 	0

#define HHI_MRG_CORNER 		0

Other test conditions will be same as those specified in the TMuC 0.9 version [2].

## Test Platforms

The executable is built with Windows 64 bits Visual Studio 9.

## Performance comparison

Table I

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-Rate | U BD-Rate | V BD-Rate | Y BD-Rate | U BD-Rate | V BD-Rate |
| Class A | -0.7 | 0.0 | 0.2 | -0.5 | -0.1 | -0.1 |
| Class B | -0.5 | 0.1 | 0.1 | -0.2 | -0.1 | -0.1 |
| Class C | -0.5 | -0.1 | -0.2 | -0.2 | 0.1 | 0.1 |
| Class D | -0.6 | -0.3 | -0.2 | -0.2 | -0.1 | 0.2 |
| Class E |  |  |  |  |  |  |
| All | -0.6 | -0.1 | -0.1 | -0.2 | 0.0 | 0.0 |
| Enc Time[%] | 84% |  |  | 85% |  |  |
| Dec Time[%] | 85% |  |  | 89% |  |  |

Table II

|  | Low delay |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-Rate | U BD-Rate | V BD-Rate | Y BD-Rate | U BD-Rate | V BD-Rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.2 | 0.2 | 0.1 | 0.6 | -0.1 | -0.2 |
| Class C | -0.4 | -0.1 | -0.2 | 0.4 | -0.1 | -0.1 |
| Class D | -0.5 | -0.5 | -0.2 | 0.4 | -0.2 | 0.2 |
| Class E | -1.3 | -2.6 | -2.3 | -0.8 | -0.5 | -0.8 |
| All | -0.5 | -0.6 | -0.5 | 0.2 | -0.2 | -0.2 |
| Enc Time[%] | 100% |  |  | 93% |  |  |
| Dec Time[%] | 97% |  |  | 116% |  |  |

The negative numbers shown in the above two tables mean that the modified skip and direct with PU based merge requires less bitrate to achieve the same PSNR. In contrast, the positive numbers mean that it requires more bitrate to achieve the same PSNR.

Regarding time measurement, because the simulations may run at different platforms due to lack of enough computation resources at very limited time frame, the numbers shown in the tables may not be accurate.

- Conclusion

As part of CE9, modified skip and direct with PU based merge is tested. Compared with anchors, the tool provides gains for random access, random access low complexity and low delay categories, respectively. However, there is some performance loss for low delay low complexity category.

# References

[1] J. Jung, B. Bross, “Core Experiment 9: Motion Vector Coding,” JCT_VC Document JCTVC-C509_r3, Oct. 2010.

[2] F. Bossen, “Common test conditions and software reference configurations,” JCT_VC Document JCTVC-C500, Oct. 2010.

Motorola Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).