| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D249_r1 |
| --- | --- |

| Title: | Super largest coding tree block |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Krit Panusopone Xue Fang Limin Wang | Email: | krit@motorola.com X.Fang@motorola.com limin.wang@motorola.com |
| Source: | Motorola Mobility |  |  |

_____________________________

# Abstract

This document introduces the concept of super largest coding tree block (SLCTB) to allow representation of largest coding tree block (LCTB) in an efficient way. Introduction of this new concept reduces the overall quadtree overhead for large resolution video as well as the complexity for small resolution video.

# Introduction

JCT-VC is currently developing a new joint video coding standard, HEVC. This new standard is still block based coding, but it tries to add more flexibility in the basic coding blocks by allowing the coding blocks to be of variable sizes and/or shape. Instead of using the traditional terms, such as macroblcok and partition as in AVC, HEVC introduces the new terminologies, including largest coding tree blcok (LCTB), coding unit (CU), partition unit (PU), and transform unit (TU). In the new standard, a picture is first divided into LCTB(s). Each LCTB can further be split into CU(s). A CU may contain one or more PU(s). HEVC is considering using a quadtree (QT) data structure to represent the partition of CU(s) within a LCTB, and the partition of PU(s) within a CU. The QT representation has to be encoded and transmitted as overhead. The overhead for CU pattern within a LCTB is per LCTB, and for TU pattern within a CU is per CU.

This document proposes an efficient way to code the QT overhead with the largest coding tree block (LCTB). With introduction of a super largest coding tree block (SLCTB), QT overhead for homogeneous area may be coded more efficiently and less complexly. The proposal also keeps the complexity low by not allowing actual encoding at the super largest coding unit level.

# Quadtree structure in HEVC

QT provides one way to represent a variable size block structure since it allows relatively flexible partitioning. QT structure is simple; a parent node (square block) can be split into four children nodes, each of four children nodes has a quarter size of their parent. Each node can then be split over and over again until it reaches its stop condition. In video compression context, QT represents a collection of non-overlapping coding blocks that provides the lowest coding cost over a given area. HEVC includes QT overhead; i.e., QT split flag, in its bitstream so that the decoder can reconstruct the same QT partitioning.

HEVC sets QT parameters, such as LCTB size and minimum CU size, at sequence level [1]. Moreover, the current common test condition fixes the QT parameters for all sequences. Our experimental results show that LCTB of 64x64 occurs very frequently for large resolution video encoding (Class B, 1920x1080). For example, up to 70% of LCTB(s) take 64x64 CU partitions for sequence BQTerrace at QP 37 under random access and high efficiency condition. On the other hand, LCTB of 64x64 is rarely chosen for small resolution video encoding (Class D, 416x240). This results show that the LCTB representation in the current QT parameters setting is inefficient.

# The proposed methods

This document proposes SLCTB as an efficient way to represent LCTB in HEVC. In this mode, the system starts with QT at SLCTB level consisting of four LCTB(s) together. The new QT always splits SLCTB into four LCTB and then proceeds. The main difference between starting at SLCTB and starting at LCTB is that the encoder fetches four LCTB(s) at a time to support SLCTB. Fig. 1 (a) and (b) show processing orders for SLCTB and LCTB, respectively. Inside SLCTB, normal processing order is maintained, that is, starting with top left LCTB, followed by top right LCTB and bottom left LCTB, and ended with bottom right LCTB. Neighbor definition of CU does not change in this system and hence its distortion should be similar to existing method.

In addition, new syntax is introduced to code QT overhead in an efficient manner. Since SLCTB is always split, there is no need to transmit split flag ‘1’ for SLCTB. However, additional overhead is used at LCTB level to signal whether any one of the four LCTB splits further or not. If none of the four LCTBs will be split, flag ‘0’ is transmitted. Otherwise, flag ‘1’ is transmitted.

In the case that flag ‘0’ is transmitted, there is no need to transmit any split flag bit for the four LCTBs.

In the case that flag ‘1’ is transmitted, four split flag bits are required to indicate further QT split for each CU. Additional saving in QT overhead is however possible. Specifically, the last split flag can be omitted if the first three split flag bits are ‘0’. This change assumes that the last LCTB uses split flag ‘1’. This saving does not have any impact on coding process since flag ‘1; signals that at least one of the four LCTBs split. If the first three LCTBs do not split, it has to be the last LCTB that splits.

Figure 2 and 5 show two examples of QT representation of SLCTB. In Fig. 2, SLCTB is coded using four non-split LCTBs only. In this example, the proposed method only transmits one split flag ‘0’ for SLCTB, as shown in Fig. 3, while the existing method requires one split flag ‘0’ for each LCTB, meaning 4 bits for a block of SLCTB size, as shown in Fig. 4. In Fig. 5, SLCTB is coded with three non-split LCTBs and one split LCTB. Figure 6 shows the QT partition and the associated overhead for the example in Fig. 5, using the proposed method. Figure 7 shows the QT partition and the overhead for the same example in Fig. 5 using the current HEVC definition.

Fig. 1 (a) Processing order with SLCTB

Fig. 1 (b) Processing order with LCTB

Fig. 2 An example of a SLCTB partitioning (LCTB size 64x64)

Fig. 3 QT overhead with the proposed method (SLCTB) for partitioning in Fig. 2

Fig. 4 QT overhead with the existing method for partitioning in Fig. 2

Fig. 5 Another example of a SLCTB partitioning (LCTB size 64x64)

Fig. 6 QT overhead with the proposed method (SLCTB) for partitioning in Fig. 5

Fig. 7 QT overhead with the existing method for partitioning in Fig. 5

For the best scenario, the proposed method can save 3 bits for a SLCTB, or 0.75 equivalent bits per LCTB, as shown in Fig. 2. But, in the worst case, the proposed method may use one more bit for a SLCTB, or 0.25 equivalent bits for a LCTB. The overall impact on coding efficiency of the proposed method depends on the number of LCTBs chosen. Our experiments indicated that SLCTB of 128x128 is favorable for large resolution video; class A, class B, and class E. On the other hand, SLCTB of 64x64 is good for small resolution video; class C and class D.

# Experimental results

SLCTB mode is implemented in TMuC0.9 to evaluate its performance. Table 1 shows experimental results of the proposed method against those of the anchor encoder using the common test condition agreed at the Guangzhou meeting [2]. The proposed method set SLCTB size to 128 for class A, B, E and 64 for class C, D.

Experimental results show the encoding complexity reduces to 88% and 93% for random access high efficiency case and low complexity case respectively. For low delay case, it reduces to 89% and 95% for high efficiency and low complexity respectively. The coding losses in BD-rate are 0.3% for random access high efficiency case and 0.4% for random access low complexity case. Same BD-rate is observed for both low delay high efficiency and low delay low complexity.

Table 1 Experimental Results

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.1 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class B | -0.2 | -0.2 | -0.2 | -0.1 | -0.2 | 0.0 |
| Class C | 0.8 | 0.5 | 0.5 | 0.9 | 1.1 | 0.9 |
| Class D | 0.5 | 0.1 | 0.5 | 0.6 | 0.6 | 0.7 |
| Class E |  |  |  |  |  |  |
| All | 0.3 | 0.1 | 0.2 | 0.4 | 0.4 | 0.4 |
| Enc Time[%] | 88% |  |  | 93% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.2 | -0.2 | -0.2 | 0.0 | 0.1 | -0.2 |
| Class C | 0.4 | 0.5 | 0.7 | 0.4 | 0.6 | 0.7 |
| Class D | 0.2 | -0.2 | 0.5 | 0.1 | 0.6 | 0.3 |
| Class E | -0.4 | 0.2 | -0.5 | -0.6 | -0.2 | -0.9 |
| All | 0.0 | 0.1 | 0.4 | 0.0 | 0.3 | 0.0 |
| Enc Time[%] | 89% |  |  | 95% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

# References

- “WD1: Working Draft 1 of High-Efficiency Video Coding”, JCTVC-C403, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Guangzhou, China, Oct. 2010

- Frank Bossen, “Common test conditions and software reference configurations”, JCTVC-C500, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Guangzhou, China, Oct. 2010

# Patent rights declaration(s)

Motorola may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).