| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4nd Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D250_r2 |
| --- | --- |

| Title: | Efficient Transform Unit Representation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Krit Panusopone Xue Fang Limin Wang | Email: | krit@motorola.com X.Fang@motorola.com limin.wang@motorola.com |
| Source: | Motorola Mobility |  |  |

_____________________________

# Abstract

This document proposes an efficient transform unit representation. The new approach modifies RQT based-TU partitioning scheme in the current TMuC. The proposed syntax change offers the optimal trade-off between coding efficiency improvement and additional complexity.

# Introduction

Video compression system employs block processing for most of its operations. A block is a group of neighboring pixels, and it is treated as one coding unit in terms of compression operations. Theoretically, large coding unit is preferred to take advantage of correlation among immediate neighboring pixels. Block sizes of 4x4, 8x8 and 16x16 are employed in the past video compression standards; e.g., MPEG-1, MPEG-2, and MPEG-4. Traditionally, encoder utilizes a fixed transform size (e.g., 4x4, 8x8, 16x16) in a MB. If more than one possible transforms are allowed at MB level, an MB level parameter may be required to indicate which transform size is used.

JCT-VC is a new joint High Efficient Video Coding standard (HEVC) that is currently under development. HEVC is also a block-based coding scheme. However, instead of using the block related terms, such as macroblock and partition as in AVC standard, HEVC introduces new block related terminologies, including Largest Coding Tree Block (LCTB), Coding Unit (CU), Partition Unit (PU), and Transform Unit (TU). LCTB, CU and TU are square blocks while PU can be either square or rectangular blocks. In HEVC, a picture is first divided into LCTB(s), and each LCTB can be further divided into CU(s). A CU may contain one or more PU. HEVC uses Quadtree (QT) data structure to represent various possible CU(s) within a LCTB. Given a CU, HEVC allows TU with various sizes. A separate QT is used to represent the possible TU(s) within a CU. Both QT representations need to be coded as overhead. The CU QT is per LCTB and TU QT (RQT, residual quadtree) is per CU.

In the current JCT-VC reference SW, TMuC, RD based approach is used to determine CU(s) within a LCTB and TU(s) within a CU. This RD based approach can be very costly in terms of additional complexity.

# Quadtree partitioning for transform unit

To keep the overhead and the complexity relatively low, certain constraints can be applied to QT structure. In particular, HEVC limits the maximum size of a TU and the depth of RQT. Specifically, maximum TU size is set equal to the CU size. Depth of RQT determines the minimum size of a TU relative to the maximum size. Currently, tree depth is set to 3 in high efficiency test condition and 2 for low complexity condition.

The current RQT is suboptimal since the current TMuC sets the maximum size of a TU to the CU size [1]. Within a CU, TMuC uses RD approach to determine TU(s) within a CU. The TU sizes and their positions within a CU can be represented by RQT. This constraint is not efficient since there is minimal correlation across PU boundaries, but there is strong blocking artifact.

# Efficient Quadtree partitioning for transform unit

To improve trade-off between complexity and coding efficiency, it is proposed that maximum TU size is linked to PU size, instead of CU size. Specifically, given a PU (either a square block or a rectangular block), the corresponding TU takes the same size as the PU. If PU is a square block, TU is a square block of the same size. If PU is a rectangular block, TU size is then set to the size of the largest possible square block that can fit into the PU. This proposal requires no overhead for TU, and it has the least complexity since the overhead and complexity associated with RQT are eliminated completely. Fig. 1 shows an example of the TU partitioning in relation to PU and CU for the proposed method.

Fig.1 an example of the TU partitioning in relation to PU and CU for the proposed method (a) an LCU and its CU, (b) PU choice for a CU, (c) maximum TU for a PU.

# Experimental results

The proposed TU partitioning is implemented in TMuC0.9 to evaluate its performance. Table 1 shows experimental results of the proposed method against those of the anchor encoder using the common test condition agreed at the Guangzhou meeting [2].

Experimental results show, for random access case, the encoding complexity reduces to 73% and 90% for high efficiency and low complexity respectively. For low delay case, it reduces to 77% and 92% for high efficiency and low complexity respectively. The coding losses in bdrate are less than 1.0% for high efficiency case and less than 1.8% for low complexity case. For low complexity case it shows small gains for chroma components with slightly increased PSNR. The software has been cross checked by RIM [3].

Table 1 Experimental Results

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.7 | 0.3 | 0.5 | 0.4 | 0.0 | 0.1 |
| Class B | 0.9 | 0.5 | 0.5 | 0.7 | -0.2 | -0.4 |
| Class C | 0.8 | 0.3 | 0.3 | 1.0 | 0.0 | 0.0 |
| Class D | 0.9 | 0.2 | 0.4 | 1.4 | -0.3 | 0.1 |
| Class E |  |  |  |  |  |  |
| All | 0.9 | 0.3 | 0.4 | 0.9 | -0.1 | -0.1 |
| Enc Time[%] | 73% |  |  | 90% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 1.0 | 1.6 | 1.6 | 1.4 | -0.4 | -0.8 |
| Class C | 1.2 | 0.8 | 0.7 | 2.1 | -0.3 | -0.5 |
| Class D | 1.2 | 0.9 | 0.9 | 2.5 | -0.5 | -0.6 |
| Class E | 0.3 | 0.5 | 0.4 | 0.9 | 0.0 | -0.4 |
| All | 1.0 | 1.0 | 1.0 | 1.8 | -0.3 | -0.6 |
| Enc Time[%] | 77% |  |  | 92% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |

# References

- “WD1: Working Draft 1 of High-Efficiency Video Coding”, JCTVC-C403, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Guangzhou, China, Oct. 2010

- Frank Bossen, “Common test conditions and software reference configurations”, JCTVC-C500, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Guangzhou, China, Oct. 2010

- Jinwen Zan, Dake He, “Cross-check for Motorola’s Proposal D250 on Efficient Transform Unit Representation by RIM”, JCTVC-D433, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Daegu, KR, Jan. 2011

# Patent rights declaration(s)

Motorola may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).