| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D251_r1 WG11 Number: m19014 |
| --- | --- |

| Title: | Intra prediction based on repetitive pixel replenishment with adaptive block size |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kenichi Iwata Seiji Mochizuki Ryoji Hashimoto 5-20-1 Josuihoncho, Kodaira, Tokyo 187-8588, Japan | Tel: Email: | +81-42-312-6608 kenichi.iwata.yb@renesas.com seiji.mochizuki.zc@renesas.com ryoji.hashimoto.te@renesas.com |
| Source: | Renesas Electronics Corporation |  |  |

_____________________________

# Abstract

This contribution presents evaluation results of intra prediction by template matching with adaptive block size based on repetitive pixel replenishment (Intra RPR). The simulation results show that this proposed technique has an average of -1.8 % (Y), -1.4 % (U), and -1.3 % (V) BD-rate improvement against TMuC 0.9 [1]. BD-rate gain of the proposed scheme is up to -5.0 % (Y), -4.1 % (U), and -4.1 % (V).

- Introduction

Number of bits derived from intra picture average 26 % in alpha anchor of CfP [2] and 53% at most. Thus, focus was on a much reduction of intra-bit and improvement quality of intra picture. It is also good effect on the following picture.

Whereas intra-frame prediction in AVC uses only neighboring pixel, more pixels are used to create predicted image in the proposal. Key idea is based on that two dimensional picture has own correlation. Similar texture often exists in a frame. Thus we reduced this overlapping redundancy using template matching based on intra vector prediction. In this scheme, we can use more pixels than the AVC intra prediction for generating predicted images.

- Algorithm

The intra prediction using repetitive pixel replenishment (Intra RPR) is added to AVC [3] and TMuC [4], and also can be replaced with some other intra prediction tools. The concept of Intra RPR is described in Figure 1. The Intra RPR is to use a displacement vector to select a previously-decoded area within the same picture to use to form a prediction block.

Intra prediction block is created by motion searches around current block. If reference block includes a not-coded area, intra vector (Vx, Vy) is multiplied as (2Vx, 2Vy), and adaptively padding such region by using new reference pixel as shown in Figure 1. A method was described for filling in areas of the picture that have not yet been coded in repeatedly. This scheme is especially effective to predict the periodical textures.

Intra vector prediction is conducted by each 8x8 block unit for both luminance and chrominance and searches 16 blocks range. The motion vectors are signaled by calculating PMV to neighboring 8x8 blocks. We apply the intra vector prediction only to I slice. Accuracy of motion vector is integer pixel for both luminance and chrominance in this submitted data. It could be extend to sub-pixel displacement search.

Figure 1 Intra prediction based on RPR

This technique greatly improves the predicted images compared to the one generated by the anchor of CfP [2] in the contribution of JCTVC-B033 [4]. Predicted image of the proposal reproduce texture of original image faithfully and it worked well in diagonal lines particularly.

In this proposal, adaptive block size with RD optimization and template matching with fractional pixel precision are adopted on the Intra RPR.

- Experimental results

Evaluation of BD-rate with and without Intra-RPR is carried out in following conditions.

[Intra-RPR parameter setting]

- Search range: depending on template block size, 
          typical range is horizontal [-16, 16], vertical [-16, 0] at 8x8 template block

- Template block size: 4x4 ~ 8x8 ~ 64x64 (adaptively)

- Search precision: integer pixel and fractional precision pixel

[TMuC condition]

- Version 0.9

- Intra, Intra LoCo, Random access, Random access LoCo

[Experiment condition]

- QP : 22, 27, 32, 37

- First 60 pictures of all sequence

Table 1 and Table 2 show BD-rate reduction by this proposed method against TMuC 0.9.

Table 1 BD-rate (Overall)

Table 2 Evaluation results for each sequence

- Intra

- Intra LoCo

- Random access

- Random access LoCo

- Conclusion

New intra prediction based on repetitive pixel replenishment (Intra-RPR) is proposed. This proposed method improves the quality of predicted image greatly. The average BD-rate reduction is -1.8 % (Y), -1.4 % (U), and -1.3 % (V) BD-rate improvement, and -5.0 % (Y), -4.1 % (U), and -4.2 % (V) at the most.

# Patent rights declaration(s)

Renesas Electronics Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Reference

[1] JCT-VC, “Test Model under Consideration (TMuC),” documents JCTVC-A205, Dresden, Apr. 2010.

[2] ISO/IEC JTC1/SC29/WG11 and ITU-T Q6/16, “Joint Call for Proposals on Video Compression Technology,” WG11 document N11113 and Q6/16 document VCEG-AM91, Kyoto, Jan. 2010.

[3] Seiji Mochizuki, Kenichi Iwata, “Response to Joint Call for Proposals on Video Compression Technology: Renesas,” document JCTVC-A126, Dresden, Apr. 2010.

[4] Kenichi Iwata, Seiji Mochizuki, Ryoji Hashimoto, “Intra prediction based on repetitive pixel replenishment,” document JCTVC-B033, Geneva, Jul. 2010.