| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D254 |
| --- | --- |

| Title: | Enhanced Context Modeling for Skip and Split Flag |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Name Xiaoyin Che Wenpeng Ding Yunhui Shi Baocai Yin | Email: | wpding@bjut.edu.cn syhzm@bjut.edu.cn ybc@bjut.edu.cn |
| Source: | Beijing University of Technology |  |  |

_____________________________

# Abstract

CABAC is adopted in TMuC with high efficiency configuration. When CABAC is adpoted, the coding efficiency is largely depends on the probability distribution of symbol. This contribution proposes a new context model for entropy coding of the SplitFlag and SkipFlag by using both the depth and neighboring flags as context.

# Introduction

In current TMuC version, the CU structure quadtree has 4 levels, and the SplitFlag is used to indicate whether this CU will be further divided. The SkipFlag is used to indicate whether this CU will adopt the skip mode in inter frame coding. This document proposes a new context model for the SplitFlag and SkipFlag by using both the depth and neighboring flags as context.

# Proposed context model

## SplitFlag

In TMuC version 0.9, the context  of SplitFlag is modeled as in Table 1. The context  of current SplitFlag is determined by the SplitFlag of the left CU and above CU . There is no doubt that the SplitFlag of left and above CU is quite relevant to the SplitFlag of current CU, but the CU Depth is also a important factor in this case, especially when the SplitFlag values of the left and the above is different. In such cases, the probability of the SplitFlag value of the current CU to be 0 or to be 1 is quite close.

Table 1. Origin ctx model assignment on SplitFlag

| Left CU | AboveCU | Ctx Model Number |
| --- | --- | --- |
| 0 | 0 | 0 |
| 0 | 1 | 1 |
| 1 | 0 | 1 |
| 1 | 1 | 2 |

Table 2, Table 3 and Table 4 shows the statistics collected from classD YUV sequence. Table 2 shows the relationship between the SplitFlag value of current CU and the left & above, and Table 3 shows the relationship between the SplitFlag value and the depth of current CU. Table 4 shows the relationship Table 3 shows when the SplitFlag value of the left and the above is different.

Table 2. The relationship between the SplitFlag value of current CU and the left & above

(*In the first line, "01->0" means left CU SplitFlag value=0, above=1 and current=0)

| Sequence | QP | 00->0 | 00->1 | 11->0 | 11->1 | 01->0 | 01->1 | 10->0 | 10->1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BasketballPass | 22 | 4216 | 958 | 339 | 1400 | 1297 | 713 | 704 | 1303 |
|  | 27 | 4451 | 871 | 248 | 944 | 1107 | 532 | 602 | 963 |
|  | 32 | 4442 | 670 | 158 | 559 | 802 | 381 | 445 | 629 |
|  | 37 | 4163 | 449 | 91 | 260 | 525 | 238 | 295 | 385 |
| BlowingBubbles | 22 | 5053 | 1811 | 901 | 2969 | 1685 | 1862 | 1362 | 1639 |
|  | 27 | 5245 | 1559 | 597 | 1817 | 1427 | 1238 | 1095 | 1176 |
|  | 32 | 5070 | 1131 | 339 | 1014 | 1035 | 776 | 764 | 713 |
|  | 37 | 4744 | 746 | 175 | 470 | 684 | 400 | 461 | 398 |
| BQSquares | 22 | 4283 | 1159 | 496 | 1867 | 986 | 1171 | 857 | 867 |
|  | 27 | 4063 | 748 | 292 | 1165 | 644 | 761 | 549 | 520 |
|  | 32 | 3708 | 521 | 186 | 752 | 452 | 544 | 401 | 342 |
|  | 37 | 3634 | 369 | 131 | 543 | 321 | 395 | 306 | 243 |
| RaceHorses | 22 | 4379 | 2067 | 1294 | 4663 | 2367 | 2370 | 1741 | 2641 |
|  | 27 | 5582 | 1993 | 924 | 3147 | 2222 | 1792 | 1668 | 2118 |
|  | 32 | 6633 | 1675 | 612 | 1911 | 1724 | 1204 | 1287 | 1488 |
|  | 37 | 6387 | 1080 | 306 | 934 | 1050 | 691 | 841 | 785 |
| Sum |  | 76053 | 17807 | 7089 | 24415 | 18328 | 15068 | 13378 | 16210 |
| Ratio |  | 81.03% | 18.97% | 22.50% | 77.50% | 54.88% | 45.12% | 45.21% | 54.79% |

Table 3. the relationship between the SplitFlag value and the depth of current CU

(*In the first line, "1->0" means current CU depth=1, and SkipFlag value=0)

| Sequence | QP | 0->0 | 0->1 | 1->0 | 1->1 | 2->0 | 2->1 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BasketballPass | 22 | 366 | 534 | 1513 | 1573 | 4303 | 3289 |
|  | 27 | 426 | 474 | 1589 | 1257 | 4212 | 2116 |
|  | 32 | 470 | 430 | 1721 | 949 | 3812 | 1284 |
|  | 37 | 558 | 342 | 1680 | 638 | 3205 | 647 |
| BlowingBubbles | 22 | 65 | 835 | 1314 | 2976 | 7051 | 6153 |
|  | 27 | 147 | 753 | 1673 | 2289 | 6582 | 3874 |
|  | 32 | 287 | 613 | 1827 | 1575 | 5448 | 2152 |
|  | 37 | 434 | 466 | 1818 | 996 | 4245 | 1039 |
| BQSquares | 22 | 261 | 639 | 1413 | 2093 | 4187 | 5485 |
|  | 27 | 476 | 424 | 1445 | 1201 | 3389 | 2715 |
|  | 32 | 608 | 292 | 1368 | 750 | 2626 | 1674 |
|  | 37 | 669 | 231 | 1291 | 583 | 2450 | 1182 |
| RaceHorses | 22 | 1 | 899 | 643 | 3903 | 7963 | 8949 |
|  | 27 | 11 | 889 | 1135 | 3371 | 8872 | 5912 |
|  | 32 | 42 | 858 | 1720 | 2662 | 8441 | 3507 |
|  | 37 | 142 | 758 | 2297 | 1685 | 6490 | 1550 |
| Sum |  | 4963 | 9437 | 24447 | 28501 | 83276 | 51528 |
| Ratio |  | 34.47% | 65.53% | 46.17% | 53.83% | 61.78% | 38.22% |

Table 4. the relationship between the SplitFlag value and the depth of current CU when the SplitFlag value of the left and the above is different

| Sequence | QP | 0->0 | 0->1 | 1->0 | 1->1 | 2->0 | 2->1 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BasketballPass | 22 | 259 | 277 | 405 | 699 | 1367 | 1439 |
|  | 27 | 274 | 248 | 391 | 576 | 1098 | 937 |
|  | 32 | 270 | 228 | 359 | 458 | 780 | 567 |
|  | 37 | 243 | 186 | 292 | 300 | 468 | 282 |
| BlowingBubbles | 22 | 44 | 331 | 577 | 1091 | 2414 | 2671 |
|  | 27 | 84 | 324 | 650 | 905 | 1878 | 1645 |
|  | 32 | 121 | 255 | 580 | 662 | 1226 | 889 |
|  | 37 | 136 | 204 | 474 | 429 | 678 | 412 |
| BQSquares | 22 | 127 | 290 | 427 | 765 | 1290 | 2110 |
|  | 27 | 160 | 214 | 332 | 452 | 826 | 1013 |
|  | 32 | 133 | 126 | 206 | 293 | 527 | 702 |
|  | 37 | 119 | 98 | 175 | 250 | 419 | 487 |
| RaceHorses | 22 | 1 | 350 | 314 | 1240 | 3477 | 4084 |
|  | 27 | 7 | 353 | 570 | 1259 | 3310 | 2714 |
|  | 32 | 22 | 354 | 757 | 1136 | 2347 | 1546 |
|  | 37 | 67 | 324 | 763 | 732 | 1117 | 675 |
| Sum |  | 2067 | 4162 | 7272 | 11247 | 23222 | 22173 |
| Ratio |  | 33.18% | 66.82% | 39.27% | 60.73% | 51.16% | 48.84% |

According to these statistics, separating the yellow area of Table 4 from the origin proposal as a independent situation and assign it into a new context model is the most reasonable solution. The new context model will be definite following the original 3 and the initial probability will be assigned as [0, 64].Table 5 shows the new ctx model assignment.

Table 5. New ctx model assignment on SplitFlag

| Left CU | Above CU | Depth | Ctx Model Number |
| --- | --- | --- | --- |
| 0 | 0 | any | 0 |
| 0 | 1 | 0 or 1 | 3 |
| 0 | 1 | 2 | 1 |
| 1 | 0 | 0 or 1 | 3 |
| 1 | 0 | 2 | 1 |
| 1 | 1 | any | 2 |

## SkipFlag

What to do on SkipFlag is quite similar to SplitFlag. Besides the left value and the above value used in origin proposal, CU depth is also included to dicide which ctx model should be adopted. Table 6 and Table 7 show the origin and new ctx model assignment on SkipFlag separately.

Table 6. Origin ctx model assignment on SkipFlag

| Left CU | AboveCU | Ctx Model Number |
| --- | --- | --- |
| 0 | 0 | 0 |
| 0 | 1 | 1 |
| 1 | 0 | 1 |
| 1 | 1 | 2 |

Table 7. New ctx model assignment on SkipFlag

| Left CU | Above CU | Depth | Ctx Model Number |
| --- | --- | --- | --- |
| 0 | 0 | any | 0 |
| 0 | 1 | 0 | 3 |
| 0 | 1 | 1, 2 or 3 | 1 |
| 1 | 0 | 0 | 3 |
| 1 | 0 | 1, 2 or 3 | 1 |
| 1 | 1 | any | 2 |

# Experimental results

For this contribution, 50 frames of all the sequences in classA, classB, classC and classD are under Experiment with cfg file randomaccess.cfg.

|  |  | BD-rate Y | BD-rate U | BD-rate V |
| --- | --- | --- | --- | --- |
| class A | Traffic | -0.1 | -0.1 | 0.1 |
|  | PeopleOnStreet | -0.1 | 0.0 | 0.0 |
| class B | Kimono | 0.1 | 0.5 | 0.1 |
|  | ParkScene | -0.1 | 0.5 | -0.3 |
|  | Cactus | -0.1 | 0.7 | 0.3 |
|  | BasketballDrive | -0.2 | -0.3 | -0.5 |
|  | BQTerrace | 0.0 | 0.8 | -0.4 |
| class C | BasketballDrill | 0.1 | 0.6 | 0.5 |
|  | BQMall | 0.1 | -1.3 | 0.2 |
|  | PartyScene | -0.1 | -0.1 | -0.3 |
|  | RaceHorses | 0.0 | 0.3 | 0.7 |
| class D | BasketballPass | -0.1 | 0.2 | 0.4 |
|  | BQSquare | -0.1 | -0.4 | -1.3 |
|  | BlowingBubbles | 0.2 | -0.2 | -0.1 |
|  | RaceHorses | -0.2 | 0.6 | -0.3 |
| Summary | Class A | -0.1 | -0.1 | 0 |
|  | Class B | -0.1 | 0.4 | -0.2 |
|  | Class C | 0.1 | -0.1 | 0.3 |
|  | Class D | -0.1 | 0.0 | -0.3 |
|  | All | 0.0 | 0.1 | -0.1 |

# Patent rights declaration(s)

Beijing University may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).