| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D256 WG11 Number: m19020 |
| --- | --- |

| Title: | Efficient 16 and 32-point transforms |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | R. Joshi, Y. Reznik, J. Sole, and M. Karczewicz | Email: rajanj@qualcomm.com Tel.: +1-858-658-4511 |  |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

This contribution proposes new 16 and 32-point transforms. The proposed transforms are scaled orthogonal integer transforms and support an elegant recursive factorization structure. The proposed 16 and 32-point transforms use 36 and 92 multiplications, respectively, compared to 44 and 116 multiplications for the corresponding transforms in the existing test model. A multiplication-free implementation for the proposed transforms is also provided. The BD-rate performance of the proposed transforms is nearly identical to the existing transforms.

- Introduction

The existing 16 and 32-point transforms used in the HEVC Test Model are based on the well-known Chen and Smith factorization [1]. Arguably the best known factorization for 8 and 16-point transforms is due to Loeffler et al [2]. Recently, this factorization was used to implement 32-point transforms [3]. The Loeffler factorization is known to be numerically stable. It also has low complexity in terms of numbers of additions and multiplications. Yet, it does not fully reuse lower block size DCT type II blocks which may already be available in the system. Reusing lower sized transforms can simplify hardware as well as software implementations. Therefore, we propose an alternative factorization for the 16 and 32-point transforms.

- Technical description

The DCT matrices for type II and type IV transforms are specified by:

|  | (1) |
| --- | --- |

where we have omitted the normalization factors ( and ).

It is well known that even-sized DCT-II matrix can be factored into a product containing direct sum of smaller DCT-II and DCT-IV matrices as follows [4], [5]:

|  | (2) |
| --- | --- |

where PN  is a permutation matrix producing reordering:

|  | (3) |
| --- | --- |

and where IN/2 and JN/2 denote N/2N/2 identity and order reversal matrices, respectively. We next apply the following decomposition to the DCT-IV block in (2):

|  | (4) |
| --- | --- |

where  denotes matrices of the remaining half-sized DCT-II transforms, PN  is a reordering matrix (2), EN/2 is the diagonal sign-alteration matrix and RN  is the matrix of Givens rotations:

| and | (5) |
| --- | --- |
| . | (6) |

Hence, if we have to design a 32-point (N=32) transform, this factorization will effectively split it into:

- A 16-point DCT-II residing in the even part of the transform,

- two 8-point DCT-IIs in the odd part of the transform,

- one 16-point butterfly implied by Givens rotation matrix R16.

Similarly, a 16-point transform can be split into

- An 8-point DCT-II residing in the even part of the transform,

- two 4-point DCT-IIs in the odd part of the transform,

- one 8-point butterfly implied by Givens rotation matrix R8.

In order to design an integer transform, the sine and cosine terms are replaced by dyadic rationals. Figure 1 shows the proposed 16-point transform. The factors on the right hand side are normalization factors (not including ). Thus, the transform is a scaled transform.

Figure 1: Proposed 16-point transform

In order for the transform to be orthogonal, the butterfly factors have to satisfy:

Z4=A42+B42, Z8=A82+B82=C82+D82, and

Z16=A162+B162=C162+D162=E162+F162=G162+H162.

Similar relationship holds for butterfly factors in the odd part of 32-point transform, that is:

Z32=A322+B322=C322+D322 and so on.

The choice of butterfly factors for the proposed 16 and 32 point transforms is shown in Tables 1-3.

| A4 | B4 | A8 | B8 | C8 | D8 |
| --- | --- | --- | --- | --- | --- |
| 17/64 | 41/64 | 111/128 | 22/128 | 94/128 | 63/128 |

| A16 | B16 | C16 | D16 | E16 | F16 | G16 | H16 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 232/256 | 29/256 | 224/256 | 67/256 | 203/256 | 116/256 | 181/256 | 148/256 |

| A32 | C32 | E32 | G32 | I32 | K32 | M32 | O32 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1013/1024 | 1003/1024 | 982/1024 | 958/1024 | 922/1024 | 859/1024 | 827/1024 | 757/1024 |
| B32 | D32 | F32 | H32 | J32 | L32 | N32 | P32 |
| 34/1024 | 146/1024 | 251/1024 | 331/1024 | 421/1024 | 538/1024 | 586/1024 | 674/1024 |

Table 1-3: Butterfly factors for the proposed 16 and 32 point transforms

It should be noted that Tables 1-3 specify the butterfly factors for a 32-point transform. Tables 1-2 specify the butterfly factors for a 16-point transform. Table 1 specifies the butterfly factors for a 8-point transform. Finally, A4 and B4 specify a 4-point transform. All the factors chosen are less than 1.

## Multiplierless implementation

The proposed 16 and 32-point transforms can be implemented using only shifts and additions. This is accomplished by replacing a pair of multiplies in a butterfly by shifts and additions. Table 4 specifies a multiplierless design for the proposed transforms.

| Pair of factors | 1st factor (A) | 2nd factor (B) | Algorithm: y = A*x; z = B*x; | Complexity |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Additions | Shifts |
| A4,B4 | 17 | 41 | x1 = x << 4; x2 = x << 3; x3 = x1 + x2; y = x + x1; z = x3 + x4; | 3 | 2 |
| A8,B8 | 111 | 22 | x1 = x << 3; x2 = x1 - x; x3 = x2 << 4; x4 = x2 << 1; y = x3 - x; z = x1 + x4; | 3 | 3 |
| C8,D8 | 94 | 63 | x1 = x << 5; x2 = x - x1; z = x1 - x2; y = z - x2; | 3 | 1 |
| A16,B16 | 232 | 29 | x1 = x << 2; x2 = x1 - x; x3 = x1 << 3; z = x3 - x2; y = z << 3; | 2 | 3 |
| C16,D16 | 224 | 67 | x1 = x << 5; x2 = x + x1; x3 = x2 << 1; z = x + x3; x5 = x1 << 3; y = x5 - x1; | 3 | 3 |
| E16,F16 | 203 | 116 | x2 = x + (x<<1); x3 = x << 5; x4 = x3 - x2; x5 = x4 << 3; z = x4 << 2; y = x5 - x4; | 3 | 4 |
| G16,H16 | 181 | 148 | x1 = x << 5; x2 = x << 2; x3 = x + x1; x4 = x2 + x3; z = x4 << 2; y = x3 + z; | 3 | 3 |
| A32,B32 | 1013 | 34 | x1 = x << 3; x2 = x << 1; x3 = x1 + x2; x4 = x << 10; x5 = x4 - x; x6 = x1 << 2; z = x2 + x6; y = x5 - x3; | 4 | 4 |
| C32,D32 | 1003 | 146 | x2 = x+(x<<6); x4 = x2+(x<<3); x6 = x+(x<<2); x8 = (x<<10)-x; x9 = x6<<2; y = x8-x9; z = x4<<1; | 5 | 6 |
| E32,F32 | 982 | 251 | x1 = x << 2; x2 = x << 4; x3 = x + x1; x4 = x2 << 4; z = x4 - x3; x6 = x2 - x4; x7 = z - x6; y = x7 << 1; | 4 | 4 |
| G32,H32 | 958 | 331 | x2 = x+(x<<5); x4 = (x<<9)-x2; x6 = x2+(x5<<2); z = x+(x6<<1); y = x4<<1; | 4 | 5 |
| I32,J32 | 922 | 421 | x2 = (x<<3)-x; x4 = (x2<<1)-x; x6 = x4+(x2<<6); x8 = (x2<<5)-x2; z = (x8<<1)-x4; y = x6<<1; | 5 | 6 |
| K32,L32 | 859 | 538 | x2 = x+(x<<5); x4 = (x<<8)-x2; y = (x4<<2)-x2; x7 = x+(x2<<3); x9 = x7+(x<<2); z = x9<<1); | 5 | 6 |
| M32,N32 | 827 | 586 | x2 = x+(x<<2); x4 = x2+(x<<8); x6 = x4+(x<<5); x8 = (x2<<6)-x2; y = x8+(x<<9); z = x6<<1; | 5 | 6 |
| O32,P32 | 757 | 674 | x2 = (x<<2)-x; x4 = x+(x2<<8); y = x4-(x2<<2); x7 = x+(x2<<7); x9 = x7-(x2<<4); z = x9<<1; | 5 | 6 |

Table 4: Multiplierless implementation of the proposed transform

- Complexity measurement

## Arithmetic operations

Table 5 compares the complexity of the proposed transform with the existing transforms in the HEVC Test Model in terms of multiplications and additions. Table 6 specifies the complexity of 4×4, 8×8, 16×16 and 32×32 multiplierless transforms in terms of shifts and additions

|  |  | Chen 32T | Proposed transform |
| --- | --- | --- | --- |
| 16-point | Additions | 74 | 72 |
|  | Multiplications | 44 | 36 |
| 32-point | Additions | 194 | 186 |
|  | Multiplications | 116 | 92 |

Table 5: Complexity comparison of the proposed transform with HEVC Test Model transforms

| Transform | Complexity of factorization | Complexity of multiplications | Complexity of full multiplierless transform |
| --- | --- | --- | --- |
| 4-point | 6 adds + 4 muls (2 pairs) | 2 * (3adds + 2shifts) | 12 adds + 4 shifts |
| 8-point | 26 adds + 12 muls (6 pairs) | 2 * (9adds + 6shifts) | 44 adds + 12 shifts |
| 16-point | 72 adds + 36 muls (18 pairs) | 2 * (26adds + 23shifts) | 124 adds + 46 shifts |
| 32-point | 186 adds + 92 muls (46 pairs) | 2 * (81adds + 78shifts) | 348 adds + 156 shifts |

Table 6: Transform complexity estimates in terms of shifts and additions.

## Storage

The proposed 16 and 32-point transforms are scaled transform. Each output coefficient has to be normalized by a scale factor. In addition, there is another scale factor of 2/N, where N is the size of the 1-D transform. So for 2 dimensional transforms, the additional scale factor is 2/N. For the JCTVC Test model, transform sizes of 4, 8, 16, and 32 are used. In these cases, this additional scale factor can be absorbed into the quantizer step-size. Thus, for the proposed 32 point transform, the remaining scale factors are:

| Index | scale factor | index | scale factor |
| --- | --- | --- | --- |
| 0, 16 | 1/2 | 1, 31 | 1/Z32 |
| 2, 30 | 1/Z16 | 3, 5, 27, 29 | 1/(2∗Z8∗Z32) |
| 4, 28 | 1/(2∗Z8) | 7, 9, 23, 25 | 1/(Z4∗Z32) |
| 6, 10, 22, 26 | 1/(Z4∗Z16) | 11, 13, 19, 21 | 1/(Z8∗Z32) |
| 8, 24 | 1/Z4 | 15 | 1/(2∗Z32) |
| 12, 20 | 1/Z8 |  |  |
| 14, 18 | 1/(2∗Z16) |  |  |

Table 7: Scale factors for the proposed 32-point transform

Looking at the table, there are only 12 distinct scale factors for the proposed 32-point transform. Let QP be the quantizer step-size parameter. Let QP=QP/6 and QP=QP%6. Then for each of QPrem=0,1,…,5, ordinarily, a 32×32 matrix would have to be stored. But since there are only 12 distinct entries, for each of QPrem=0,1,…,5, only a 12×12 matrix has to be stored. In addition, a look up table of size 32 is needed to convert the position index to an index into the scale factor table. Horizontal and vertical directions share the same LUT. Both the quantization and dequantization matrix entries can be stored in 16 bits. So the total storage for quantization matrix, dequantization matrix and LUT is 12×12×6×2×2+32=3488bytes.

Furthermore, the normalization factors for the 16-point, 8-point and 4-point transforms are already present in the even-indexed entries of the 32-point transform. Thus, only 3 additional LUTs are needed if all the transforms from 4×4 to 32×32 are replaced using this framework. In that case, the total storage requirement for the quantization and dequantization matrices is 3516 bytes.

## Bit-width analysis

We now discuss the bit-depths at various stages of the dequantization and inverse transform process. The dequantization matrix we used for the proposed 32×32 has 8 bit precision. Assuming that the input to the dequantizer stage has bit-depth of B bits, after multiplication by the dequantization matrix entry, the bit-depth is B+8. Please note that no “transform precision extension” is performed on the decoder side. In the case of the proposed 16 and 32-point transforms, all the butterfly factors are in less than 1. Thus, the worst case scenario is for the DC coefficient, where the bit-depth can increase by 5 bits. So in the transpose buffer, the bit-depth needed is B+13 bits. The transform in the column direction adds another 5 bits. So at this stage, the bit-depth needed is B+18 bits

- Results

The proposed 16 and 32-point transforms are implemented in the TMuC 0.9 software. Simulations results are presented for the full sequences for intra, random access (RA) and low delay (LD) configurations for high efficiency (HE) and low complexity (LC) with common test conditions. Table 8 shows a summary of the results. Detailed results are provided in the accompanying excel file. The encoding was performed on a LINUX cluster. The decoding was performed on a PC using Microsoft Visual C++ (64-bit). From the table, it can be seen that the BD-rate results are nearly identical to those for TMuC0.9.

Table 8: BD-rate results for the proposed 16 and 32-point transforms.

- Conclusions

New efficient 16 and 32-point transforms are proposed for inclusion in the HEVC Test Model. The proposed transforms are scaled orthogonal integer transforms and support an elegant recursive factorization structure. The proposed 16 and 32-point transforms use 36 and 92 multiplications, respectively, compared to 44 and 116 multiplications for the corresponding transforms in the existing test model. A multiplication-free implementation for the proposed transforms is also provided. The BD-rate performance of the proposed transforms is nearly identical to the existing transforms.

- References

- W.-H. Chen, C. H. Smith, and S. C. Fralick, A fast computational algorithm for the discrete cosine transform, IEEE Trans. Commun., vol. COMM-25, pp. 1004-1009, Sep. 1977.

- C. Loeffler, A. Lightenberg, and G. Moschytz, Practical fast 1-D DCT algorithms with 11 multiplications,  Proc. IEEE ICASSP, vol. 2, pp. 988–991, Feb. 1989.

- Y.-M. Hong, M.-S. Cheon, I.-K. Kim, “Low-complexity 16x16 and 32x32 transforms and partial frequency transform”, JCTVC-C209, JCT-VC Meeting, Guangzhou, October 2010.

- K. R. Rao and P.Yip,“Discrete Cosine Transform: Algorithms, Advantages, Applications,” Academic Press (1990).

- V.Britanak, P.Yip, K.R.Rao, “Discrete Cosine and Sine Transforms: General Properties, Fast Algorithms and Integer Approximations,” Academic Press (2006).

# Patent rights declaration(s)

Qualcomm may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).