| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D257 WG11 Number: m19021 |
| --- | --- |

| Title: | Low complexity 32×32 transform by partial frequency transform |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | J. Sole, R. Joshi, M. Karczewicz Y.-M. Hong, M.-S. Cheon, I.-K. Kim | Email: joels@qualcomm.com Email: ilkoo.kim@samsung.com |  |
| Source: | Samsung Electronics and Qualcomm |  |  |

_____________________________

# Abstract

This proposal presents an approach to simplifying the 32×32 transform by computing only the 16 lowest frequency coefficients in each direction. This simplification reduces the number of multiplications by 74℅ while reducing the intermediate buffering requirements by 50℅. The loss in terms of BD-rate for the high efficiency intra, random access, and low-delay configurations is 0.31℅, 0.22℅, and 0.21℅, respectively. There is no impact on the BD-rate for the low complexity configuration.

- Introduction

Large block size transforms (32×32) are being considered for HEVC for improving coding efficiency. However, such transforms may be difficult and/or costly to implement in hardware. One particular problem is the need to buffer intermediate coefficients after transform in one direction. Several proposals [1, 2, 3] have aimed at reducing the complexity for the 32×32 transform while retaining most of the coding efficiency improvement.

This proposal combines proposals by Samsung Electronics [1] and Qualcomm [2] to obtain a transform configuration that significantly reduces the complexity and intermediate buffering requirements of the 32×32 transform by modifying the current transform core while achieving performance close to the full 32×32 transform.

- Technical description

The 32×32 transform core is modified to obtain a partial frequency transform. In each direction, only the 16 lowest frequency coefficients are computed. After the first 1-D forward transform, only 50℅ of the coefficients need to be buffered. Besides, the computational savings achieved by computing only the first 16 coefficients, some calculations to derive intermediate values can also be omitted because the 16 higher frequency coefficients are not computed. Not only is the calculation of transform cores simplified, the number of loops in the inverse transform process is reduced as well. Consequently, the number of operations for the 2-D inverse transform is significantly reduced. More details on complexity are given in section 3.

The entropy coding of the retained coefficients for the VLC method remains unchanged. For non-VLC methods, the retained 16×16 block of coefficients is scanned as if it was generated by a 16×16 transform. Then, for each coefficient, the context is also derived assuming a 16×16 transform. Thus, the unused part of the 32×32 block of coefficients is never scanned and no contexts are assigned to it. However, the probability models used for entropy-coding correspond to those for the 32×32 transform block.

- Complexity measurement

This section analyzes the complexity of the partial frequency transform and compares it with the other approaches. Table 1 details the number of operations and intermediate buffering requirement for the current Chen’s 32 transform core and for the proposed 32 partial frequency transform.

Table 1. Number of operations and intermediate buffering requirement for 32×32 2-D inverse transforms

| # of operations / memory ratio | Chen 32×32 | 32×32 partial frequency |
| --- | --- | --- |
| Additions | 12416 | 7776 |
| Multiplications | 7424 | 4128 |
| Shifts | 4224 | 3168 |
| All | 24064 | 15072 |
| Memory ratio | 1 | 0.5 |

- Results

The proposed modifications are implemented in the TMuC 0.9 software. Simulations results are presented for the full sequences with high efficiency common test conditions (note that there is no change in performance for the low complexity case). Table 2 shows the results. The BD-rate loss is 0.31℅, 0.22℅ and 0.21℅ for intra, random access and low delay configurations, respectively. Detailed results are provided in the accompanying excel file.

Table 2. BD-rate for the partial frequency 32×32 transform

| BD-rate | Intra |  |  | Random access |  |  | Low Delay |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.15 | 0.06 | -0.02 | 0.11 | 0.05 | 0.32 |  |  |  |
| Class B | 0.40 | 0.64 | 0.65 | 0.34 | 0.64 | 0.60 | 0.44 | 0.13 | 0.10 |
| Class C | 0.19 | 0.40 | 0.30 | 0.22 | 0.23 | 0.21 | 0.17 | -0.04 | 0.06 |
| Class D | 0.20 | 0.37 | 0.20 | 0.13 | 0.41 | 0.41 | 0.09 | -0.48 | 0.30 |
| Class E | 0.57 | 0.50 | 0.49 |  |  |  | 0.06 | 0.13 | -0.38 |
| All | 0.31 | 0.44 | 0.37 | 0.22 | 0.39 | 0.41 | 0.21 | -0.06 | 0.05 |
| Enc T [%] | 93% |  |  | 96% |  |  | 93% |  |  |
| Dec T [%] | 98% |  |  | 99% |  |  | 94% |  |  |

- Conclusions

Two previous proposals [1, 2] have been combined to achieve a simplification of the 32×32 transform that greatly reduces complexity and intermediate buffering requirements. The loss in terms of BD-rate for the high efficiency intra, random access, and low-delay configurations is 0.31℅, 0.22℅, and 0.21℅, respectively. There is no impact on the BD-rate for the low complexity configuration.

- References

- Y.-M. Hong, M.-S. Cheon, I.-K. Kim, “Low-complexity 16x16 and 32x32 transforms and partial frequency transform”, JCTVC-C209, JCT-VC Meeting, Guangzhou, CN, October 2010.

- J. Sole, R. Joshi, M. Karczewicz, “Reduced complexity 32x32 transform by coefficient zero-out”, JCTVC-C237, JCT-VC Meeting, Guangzhou, CN, October 2010.

- M. Budagavi, A. Gupte, “DCT+Hadamard low complexity large transform for Inter coding”, JCTVC-C255, JCT-VC Meeting, Guangzhou, CN, October 2010.

- M. Sadafale, M. Budagavi, “Low-complexity, configurable transform architecture for HEVC”, JCTVC-C2226, JCT-VC Meeting, Guangzhou, CN, October 2010.

# Patent rights declaration(s)

Qualcomm may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Samsung Electronics Co., Ltd. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).