| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D259 |
| --- | --- |

| Title: |  |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Ali Tabatabai 1730 1st Street San Jose, CA 95112 | Tel: 1-408-352-4715 Email: ali.tabatabai@am.sony.com |  |
| Source: | CE6 Coordinator |  |  |

_____________________________

# Abstract

This contribution provides a summary of Core Experiment 6, Intra Prediction Improvements.  From a total of 9 proposed CEs 8 had been cross-checked with full match by at least on organization. For cross checking, the recommended test conditions of intra-only were used for both high compression efficiency and low complexity as defined in the document JCTVC-C506_r3.

# Introduction

Intra prediction improvement core experiments was divided into 4 categories

- CE6.a: Block Based Intra Prediction

- CE6.b: Line/Pixel based Intra Prediction

- CE6.c: Edge Based Intra Prediction

- CE6.d: Parallel Intra Coding

Table 1 summarizes the technologies tested together with the list of the proponents and cross-checkers:

| Core Experiments | Technology | Proponent | Cross-checker(s) |
| --- | --- | --- | --- |
| CE6.a.1 | Bidirectional Intra Prediction | Toshiba JCTVC-C079 JCTVC-D108 | Sharp Labs. of America JCTVC-D066 Nokia JCTVC-D196 |
| CE6.a.2 | Simplified Bidirectional Intra Prediction (Simplified BIP) | Qualcomm VCEG-AG11 JCTVC-D? | Toshiba JCTVC-D? |
| CE6.a.3 | Chroma intra prediction from reconstructed luma samples | Samsung JCTVC-C206 JCTVC-D350 | Microsoft JCTVC-D145 Toshiba JCTVC-D110 |
| CE6.a.4 | Overlapped block intra prediction | MediaTek JCTVC-C193 |  |
| CE6.a.5 | Optimizing the combination of current intra predictors | Santa Clara University JCTVC-C111, JCTVC-D026 | Samsung JCTVC-D348 |
| CE6.b.1 | Short distance intra prediction | Huawei/Hisilicon & Microsoft JCTVC-C101, JCTVC-C270 JCTVC-D299 | LGE JCTVC-D124 |
| CE6.b.2 | Combined Intra Prediction | BBC JCTVC-C213 JCTVC-D191 | Sharp Labs. of America JCTVC-D065 |
| CE6.c | Differential coding of intra modes (DCIM) | Sharp &Sony JCTVC-C169, JCTVC-C176 JCTVC-D279 | NEC JCTVC-D205 Panasonic JCTVC-D218 NHK JCTVC-D177 |
| CE6.d | Parallel Intra Coding | Sharp Labs. of America JCTVC-B112 JCTVC-D074 | Toshiba JCTVC-D111 |

# Summary of Experimental Results

## Software

All tests are run based on TMuC0.9 anchor.

## Complexity

For complexity measurements, the encoding and decoding times have been the most frequently reported measures.

## Visual

NEC has conducted visual evaluation of DCIM (JCTVC-D205)

## Coding Efficiency

CE6.a.1

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.6 | -1.0 | -1.1 | -2.3 | -1.7 | -1.7 |
| Class B | -1.6 | -1.0 | -1.1 | -2.0 | -1.5 | -1.5 |
| Class C | -1.4 | -0.7 | -0.7 | -1.7 | -1.1 | -1.1 |
| Class D | -1.1 | -0.5 | -0.5 | -1.6 | -0.9 | -0.9 |
| Class E | -1.7 | -1.3 | -1.1 | -2.6 | -2.2 | -1.8 |
| All | -1.4 | -0.9 | -0.9 | -2.0 | -1.4 | -1.4 |
| Enc Time[%] | 107% |  |  | 114% |  |  |
| Dec Time[%] | 106% |  |  | 110% |  |  |

CE6.a.3

| T1: LM_5_modes |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Intra |  |  |  | Intra LoCo |  |  |  |  |
|  | Y BD-rate | U BD-rate |  | V BD-rate | Y BD-rate |  | U BD-rate | V BD-rate |  |
| Class A | -1.4 | -7.4 |  | -3.6 | -1.1 |  | -7.7 | -3.1 |  |
| Class B | -1.5 | -7.8 |  | -4.2 | -1.3 |  | -8.1 | -3.8 |  |
| Class C | -1.9 | -7.7 |  | -8.2 | -1.9 |  | -7.1 | -7.7 |  |
| Class D | -1.3 | -5.5 |  | -5.3 | -1.3 |  | -5.1 | -4.7 |  |
| Class E | -1.1 | -4.6 |  | -5.2 | -0.5 |  | -3.4 | -4.6 |  |
| All | -1.5 | -6.7 |  | -5.5 | -1.3 |  | -6.4 | -4.9 |  |
| Enc Time[%] | 100% |  |  |  | 101% |  |  |  |  |
| Dec Time[%] | 101% |  |  |  | 101% |  |  |  |  |
|  |  | Random access |  |  |  |  | Random access LoCo |  |  |
|  | Y BD-rate | U BD-rate |  | V BD-rate | Y BD-rate |  | U BD-rate | V BD-rate |  |
| Class A | -0.6 | -7.2 |  | -3.2 | -0.5 |  | -7.1 | -2.1 |  |
| Class B | -0.7 | -8.3 |  | -4.3 | -0.6 |  | -6.0 | -1.7 |  |
| Class C | -0.9 | -6.2 |  | -6.3 | -0.8 |  | -5.1 | -5.1 |  |
| Class D | -0.6 | -4.1 |  | -3.7 | -0.6 |  | -3.1 | -2.9 |  |
| Class E |  |  |  |  |  |  |  |  |  |
| All | -0.7 | -6.5 |  | -4.5 | -0.6 |  | -5.1 | -3.0 |  |
| Enc Time[%] | 100% |  |  |  | 100% |  |  |  |  |
| Dec Time[%] | 100% |  |  |  | 100% |  |  |  |  |
|  | Low delay |  |  |  | Low delay LoCo |  |  |  |  |
|  | Y BD-rate | U BD-rate |  | V BD-rate | Y BD-rate |  | U BD-rate | V BD-rate |  |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | -0.1 | -2.4 |  | -1.3 | -0.2 |  | -2.0 | -1.3 |  |
| Class C | -0.3 | -2.4 |  | -2.7 | -0.3 |  | -2.2 | -1.8 |  |
| Class D | -0.2 | -2.2 |  | -1.3 | -0.2 |  | -1.0 | -0.8 |  |
| Class E | -0.2 | -2.4 |  | -3.3 | 0.1 |  | -2.7 | -3.7 |  |
| All | -0.2 | -2.3 |  | -2.0 | -0.1 |  | -1.9 | -1.7 |  |
| Enc Time[%] | 101% |  |  |  | 102% |  |  |  |  |
| Dec Time[%] | 100% |  |  |  | 99% |  |  |  |  |
| T2: LM_3_modes |  |  |  |  |  |  |  |  |  |
|  | Intra |  |  |  | Intra LoCo |  |  |  |  |
|  | Y BD-rate | U BD-rate |  | V BD-rate | Y BD-rate |  | U BD-rate | V BD-rate |  |
| Class A | -1.4 | -7.3 |  | -3.6 | -1.1 |  | -7.5 | -2.8 |  |
| Class B | -1.5 | -7.6 |  | -3.9 | -1.2 |  | -7.7 | -3.5 |  |
| Class C | -1.8 | -7.5 |  | -8.0 | -1.7 |  | -6.8 | -7.3 |  |
| Class D | -1.2 | -5.0 |  | -5.0 | -1.1 |  | -4.7 | -4.4 |  |
| Class E | -1.1 | -4.3 |  | -5.1 | -0.5 |  | -3.6 | -4.0 |  |
| All | -1.4 | -6.4 |  | -5.2 | -1.2 |  | -6.1 | -4.5 |  |
| Enc Time[%] | 96% |  |  |  | 94% |  |  |  |  |
| Dec Time[%] | 100% |  |  |  | 101% |  |  |  |  |
|  |  | Random access |  |  |  |  | Random access LoCo |  |  |
|  | Y BD-rate | U BD-rate |  | V BD-rate | Y BD-rate |  | U BD-rate | V BD-rate |  |
| Class A | -0.6 | -7.1 |  | -3.0 | -0.4 |  | -6.6 | -1.6 |  |
| Class B | -0.7 | -8.0 |  | -3.5 | -0.5 |  | -5.5 | -1.3 |  |
| Class C | -0.8 | -5.9 |  | -6.0 | -0.7 |  | -4.7 | -4.7 |  |
| Class D | -0.5 | -3.7 |  | -3.1 | -0.5 |  | -2.7 | -2.6 |  |
| Class E |  |  |  |  |  |  |  |  |  |
| All | -0.7 | -6.2 |  | -4.0 | -0.6 |  | -4.7 | -2.6 |  |
| Enc Time[%] | 100% |  |  |  | 100% |  |  |  |  |
| Dec Time[%] | 100% |  |  |  | 101% |  |  |  |  |
|  | Low delay |  |  |  | Low delay LoCo |  |  |  |  |
|  | Y BD-rate | U BD-rate |  | V BD-rate | Y BD-rate |  | U BD-rate | V BD-rate |  |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | -0.2 | -2.3 |  | -1.3 | -0.1 |  | -2.1 | -1.2 |  |
| Class C | -0.3 | -2.4 |  | -2.6 | -0.2 |  | -2.1 | -1.9 |  |
| Class D | -0.1 | -1.9 |  | -1.3 | -0.2 |  | -1.1 | -1.0 |  |
| Class E | -0.2 | -1.5 |  | -3.8 | 0.0 |  | -2.9 | -3.1 |  |
| All | -0.2 | -2.1 |  | -2.1 | -0.1 |  | -2.0 | -1.7 |  |
| Enc Time[%] | 101% |  |  |  | 102% |  |  |  |  |
| Dec Time[%] | 100% |  |  |  | 99% |  |  |  |  |

| T3: code-word re-assigment |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Intra |  |  |  | Intra LoCo |  |  |  |  |
|  | Y BD-rate |  | U BD-rate | V BD-rate | Y BD-rate |  | U BD-rate | V BD-rate |  |
| Class A | -0.8 |  | 0.1 | 0.3 | -0.6 |  | -0.3 | -0.1 |  |
| Class B | -0.7 |  | 0.1 | 0.1 | -0.6 |  | -0.5 | -0.4 |  |
| Class C | -0.7 |  | 0.1 | 0.2 | -0.7 |  | -0.5 | -0.4 |  |
| Class D | -0.5 |  | 0.2 | 0.3 | -0.5 |  | -0.3 | -0.3 |  |
| Class E | -1.0 |  | -0.6 | -0.3 | -0.5 |  | -0.1 | -0.2 |  |
| All | -0.7 |  | 0.0 | 0.1 | -0.6 |  | -0.4 | -0.3 |  |
| Enc Time[%] | 99% |  |  |  | 99% |  |  |  |  |
| Dec Time[%] | 100% |  |  |  | 100% |  |  |  |  |
|  |  |  | Random access |  |  |  | Random access LoCo |  |  |
|  | Y BD-rate |  | U BD-rate | V BD-rate | Y BD-rate |  | U BD-rate | V BD-rate |  |
| Class A | -0.4 |  | -0.1 | 0.4 | -0.3 |  | -0.5 | -0.2 |  |
| Class B | -0.3 |  | 0.0 | 0.1 | -0.2 |  | -0.4 | -0.3 |  |
| Class C | -0.3 |  | 0.1 | 0.1 | -0.3 |  | -0.3 | -0.2 |  |
| Class D | -0.2 |  | -0.1 | 0.4 | -0.2 |  | -0.2 | -0.1 |  |
| Class E |  |  |  |  |  |  |  |  |  |
| All | -0.3 |  | 0.0 | 0.2 | -0.2 |  | -0.3 | -0.2 |  |
| Enc Time[%] | 99% |  |  |  | 98% |  |  |  |  |
| Dec Time[%] | 100% |  |  |  | 99% |  |  |  |  |
|  | Low delay |  |  |  | Low delay LoCo |  |  |  |  |
|  | Y BD-rate |  | U BD-rate | V BD-rate | Y BD-rate |  | U BD-rate | V BD-rate |  |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | -0.1 |  | -0.1 | -0.2 | -0.1 |  | -0.1 | -0.1 |  |
| Class C | -0.2 |  | 0.0 | 0.0 | -0.1 |  | -0.4 | -0.2 |  |
| Class D | -0.1 |  | -0.9 | 0.1 | -0.1 |  | 0.4 | -0.2 |  |
| Class E | 0.0 |  | -0.5 | -0.4 | -0.1 |  | 0.3 | -0.1 |  |
| All | -0.1 |  | -0.3 | -0.1 | -0.1 |  | 0.0 | -0.1 |  |
| Enc Time[%] | 98% |  |  |  | 100% |  |  |  |  |
| Dec Time[%] | 99% |  |  |  | 99% |  |  |  |  |

| T4: DM + DC |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Intra |  |  |  |  | Intra LoCo |  |  |  |  |
|  | Y BD-rate |  | U BD-rate |  | V BD-rate | Y BD-rate |  | U BD-rate | V BD-rate |  |
| Class A | -0.8 |  | -0.3 |  | 0.0 | -0.7 |  | 0.4 | 1.4 |  |
| Class B | -0.6 |  | 0.3 |  | 0.4 | -0.7 |  | 0.6 | 1.4 |  |
| Class C | -0.6 |  | 0.5 |  | 0.6 | -0.5 |  | 0.7 | 0.9 |  |
| Class D | -0.4 |  | 0.8 |  | 0.7 | -0.3 |  | 1.0 | 0.9 |  |
| Class E | -1.0 |  | -0.2 |  | -0.2 | -0.6 |  | 0.8 | 1.9 |  |
| All | -0.6 |  | 0.3 |  | 0.4 | -0.5 |  | 0.7 | 1.3 |  |
| Enc Time[%] | 93% |  |  |  |  | 90% |  |  |  |  |
| Dec Time[%] | 100% |  |  |  |  | 100% |  |  |  |  |
|  |  |  | Random access |  |  |  |  | Random access LoCo |  |  |
|  | Y BD-rate |  | U BD-rate |  | V BD-rate | Y BD-rate |  | U BD-rate | V BD-rate |  |
| Class A | -0.4 |  | 0.2 |  | 0.9 | -0.4 |  | 1.0 | 2.1 |  |
| Class B | -0.3 |  | 0.5 |  | 0.9 | -0.3 |  | 1.1 | 1.5 |  |
| Class C | -0.2 |  | 0.4 |  | 0.7 | -0.2 |  | 1.2 | 1.5 |  |
| Class D | -0.2 |  | 0.9 |  | 1.2 | -0.1 |  | 1.6 | 1.3 |  |
| Class E |  |  |  |  |  |  |  |  |  |  |
| All | -0.3 |  | 0.5 |  | 0.9 | -0.2 |  | 1.2 | 1.5 |  |
| Enc Time[%] | 99% |  |  |  |  | 99% |  |  |  |  |
| Dec Time[%] | 100% |  |  |  |  | 99% |  |  |  |  |
|  | Low delay |  |  |  |  | Low delay LoCo |  |  |  |  |
|  | Y BD-rate |  | U BD-rate |  | V BD-rate | Y BD-rate |  | U BD-rate | V BD-rate |  |
| Class A |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.1 |  | -0.1 |  | -0.1 | 0.0 |  | 0.0 | 0.3 |  |
| Class C | -0.1 |  | -0.3 |  | -0.1 | 0.0 |  | -0.1 | 0.0 |  |
| Class D | 0.0 |  | -0.6 |  | 0.3 | 0.0 |  | 0.0 | 0.0 |  |
| Class E | -0.2 |  | 0.3 |  | -0.2 | -0.1 |  | 0.6 | 1.7 |  |
| All | -0.1 |  | -0.2 |  | 0.0 | 0.0 |  | 0.1 | 0.4 |  |
| Enc Time[%] | 99% |  |  |  |  | 99% |  |  |  |  |
| Dec Time[%] | 99% |  |  |  |  | 99% |  |  |  |  |

CE6.a.5

| TEST#1 | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.2 | -0.2 | -0.2 | -0.5 | -0.3 | -0.4 |
| Class B | -0.2 | -0.2 | -0.3 | -0.5 | -0.4 | -0.4 |
| Class C | -0.1 | -0.1 | -0.1 | -0.2 | -0.2 | -0.2 |
| Class D | 0.0 | 0.0 | 0.0 | -0.2 | -0.2 | -0.2 |
| Class E | -0.4 | -0.4 | -0.5 | -0.8 | -0.8 | -0.6 |
| All | -0.2 | -0.2 | -0.2 | -0.4 | -0.4 | -0.3 |
| Enc Time[%] | 102% |  |  | 99% |  |  |
| Dec Time[%] | 97% |  |  | 99% |  |  |

Table 1 - UIP with MPS compared with Anchor

| TEST#2 | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.1 | -0.2 | -0.1 | 0.7 | 0.6 | 0.4 |
| Class B | -0.2 | -0.3 | -0.3 | 0.7 | 0.5 | 0.3 |
| Class C | 0.0 | -0.1 | -0.1 | 1.1 | 0.8 | 0.7 |
| Class D | 0.0 | 0.0 | 0.0 | 1.0 | 0.7 | 0.6 |
| Class E | -0.4 | -0.3 | -0.5 | 0.7 | 0.1 | 0.4 |
| All | -0.1 | -0.2 | -0.2 | 0.9 | 0.6 | 0.5 |
| Enc Time[%] | 103% |  |  | 101% |  |  |
| Dec Time[%] | 98% |  |  | 97% |  |  |

Table 2 - UIP with MPS and Planar enabled compared with Anchor:

CE6.b.1

|  | Intra |  |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate |  | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -3.5 | -1.9 |  | -1.9 | -4.5 | -3.3 | -3.4 |
| Class B | -3.0 | -1.4 |  | -1.7 | -4.4 | -2.5 | -3.2 |
| Class C | -5.2 | -2.8 |  | -3.1 | -7.0 | -3.8 | -4.4 |
| Class D | -5.4 | -2.9 |  | -2.9 | -6.8 | -3.9 | -4.2 |
| Class E | -4.5 | -3.0 |  | -2.7 | -5.3 | -6.3 | -5.6 |
| All | -4.3 | -2.4 |  | -2.5 | -5.7 | -3.8 | -4.1 |
| Enc Time[%] | 161% |  |  |  | 146% |  |  |
| Dec Time[%] | 100% |  |  |  | 105% |  |  |

CE6.c

Experiment-1

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.7 | -0.6 | -0.3 | -1.5 | -0.8 | -0.4 |
| Class B | -2.1 | -1.1 | -1.2 | -2.2 | -1.8 | -1.9 |
| Class C | -2.4 | -1.1 | -1.2 | -2.9 | -1.9 | -2.0 |
| Class D | -1.5 | -0.4 | -0.3 | -1.8 | -0.8 | -1.0 |
| Class E | -3.1 | -2.2 | -1.9 | -3.1 | -1.5 | -1.7 |
| All | -2.1 | -1.1 | -1.0 | -2.3 | -1.4 | -1.5 |
| Enc Time[%] | 109% |  |  | 115% |  |  |
| Dec Time[%] | 106% |  |  | 123% |  |  |

Experiment II

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.8 | -0.9 | -0.5 | -1.8 | -0.8 | -0.4 |
| Class B | -2.3 | -1.4 | -1.4 | -2.6 | -2.0 | -2.0 |
| Class C | -2.5 | -1.3 | -1.4 | -3.0 | -1.9 | -2.1 |
| Class D | -1.6 | -0.5 | -0.6 | -2.0 | -0.8 | -1.2 |
| Class E | -3.3 | -2.1 | -1.9 | -3.2 | -1.5 | -1.6 |
| All | -2.3 | -1.2 | -1.2 | -2.6 | -1.5 | -1.6 |
| Enc Time[%] | 106% |  |  | 106% |  |  |
| Dec Time[%] | 106% |  |  | 117% |  |  |