| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D261 |
| --- | --- |

| Title: | Improvements on transform coefficients coding in LCEC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jun Xu 1730 N. First Street San Jose, CA 95112, USA | Tel: Email: | +1-408-352-4086 jun.xu@am.sony.com |
|  | Munsi Haque 1730 N. First Street San Jose, CA 95112, USA | Tel: Email: | +1-408-352-4099 munsi.haque@am.sony.com |
|  | Ali Tabatabai 1730 N. First Street San Jose, CA 95112, USA | Tel: Email: | +1-408-352-4715 ali.tabatabai@am.sony.com |
| Source: | Sony Electronics Inc. |  |  |

_____________________________

# Abstract

In this proposal, several techniques are presented to improve the performance of transform coefficient coding in Low Complexity Entropy Coding of HEVC. First, different sorting tables are used for different types of TU in the proposed algorithm. These sorting tables are trained, based on separate set of video sequences from VCEG. Experimental results show that there are 1.7%, 0.7% and 0.2% BD-rate improvements on average compared to TMuC0.9. There is no increase in complexity for the proposed algorithm. Second, run-mode adaptation using swapping tables is introduced to TMuC0.9. Third, the encoding of entire TU with sizes of 16x16 and 32x32 is implemented with differentiated tables based on training.

# Introduction

In TMuC0.9, LCEC coding for transform coefficients encodes up to 8x8 transform coefficients, which means 4x4 and 8x8 blocks are encoded entirely and 16x16 and 32x32 TUs are truncated to 8x8 coefficients. These coefficients are scanned to 1-D array and encoded by three steps: 1) last nonzero coefficient position, 2) run mode and 3) level mode. In HM implementation, run mode shares the same sorting table for both 4x4 blocks and truncated 8x8 blocks. The solution in HM has the following drawbacks: 1) one single sorting table cannot describe the diverse properties of “run”; 2) one single sorting table lacks adaptation to variation of “run” in real-time; 3) Simple truncation in coefficients of large TU is sub-optimal in a RD fashion.

To address the disadvantages of current TMuC0.9, solutions are provided in this proposal to address each individual drawback. First, differentiation of sorting tables in terms of TU size, PU mode and luma/chroma-type are introduced. To test the robustness of this differentiation, sorting tables are obtained by using totally different video sequences instead of the test sequences. The training sequences are the commonly used VCEG video sequences with CIF and HD (720p) sizes. Experimental results show that the tables obtained with VCEG sequences provide very similar performance compared to the tables obtained with HEVC test sequences. Second, an adaptive sorting table is used for run-mode as done by swapping to encode the last non-zero coefficient position.  Third, the encoding of entire TU with size of 16x16 and 32x32 is implemented based on C210 [1].

- Proposed solution

## Sorting table differentiation

As observed, the distributions of runs in run-mode vary for different TU sizes and different PU modes. So differentiated sorting tables are introduced. As a result, there are dedicated sorting tables for luma of 4x4, 8x8 and 16x16/32x32 in I frames and luma of 4x4, 8x8 and 16x16/32x32 in P/B frames as well as for chroma. There are 7 sorting tables in total used in this proposal, compared to a single sorting table in HM as shown in Table 1. The VLC tables are the same 11 VLC tables proposed by TEN.

TMuC0.9 has one table represented by g_auiLumaRun8x8[29][2][64] in TMuC-0.9, where the table parameters could be as follows: maxrun=29,level=0 or 1, run=0 to 63.

This proposal has 7 such tables, but with different sizes as shown in Table 1. The table entries have similar structures as in g_auiLumaRun8x8. These tables are allocated as follows:

- 2 Tables (Intra/Inter) with 16x2x16 sizes for luma 4x4

- 5 Tables with 29x2x64 entries, where 5 tables = 2 (Intra/Inter) x 2 (luma 8x8 and 16x16/32x32 cases) + 1 (chroma).

Note: These tables contain unused entries that can be removed to save memory.

Instead of training tables based on test video sequences, sorting tables are obtained using totally different training video sequences. These training sequences are the commonly used VCEG video sequences with CIF and HD (720p) sizes. In this way, the algorithm demonstrates robustness against various video sequences.

Table 1 Comparison between proposed differentiated tables and TMuC0.9

|  | TMuC0.9 | Proposal |
| --- | --- | --- |
| Number of sorting tables | 1 | 7 |
| Table sizes | 29x2x64 | 16x2x16x2+29x2x64x5 |

## Adaptive run-mode coding

Like the adaptive coding of last nonzero coefficient position in TMuC0.9, an adaptive sorting table is used for run-mode coding. The sorting table is updated by swapping entries in the table after each run is encoded. The sorting table is reset to initial values based on training at the beginning of each frame. The advantage of this method is that there is no large increase in memory size.  This proposal replaces the sorting table (g_auiLumaRun8x8) in TMuC0.9 with a new table having the same size of 29x2x64 entries. In addition it needs a similar sized buffer for swapping.

## Large TU coding

Encoding of entire TUs with size of 16x16 and 32x32 is implemented by following instructions in C210 [1]. Differentiated tables on intra/inter are used for run-mode coding of TUs with size of 16x16 and 32x32. This is combined with the differentiated tables for run-mode proposed in Section 2.1.

The proposal has 7 such tables, but with different sizes as shown in Table 2. The table entries have similar structures as in  g_auiLumaRun8x8. These tables are allocated as follows:

- 2 Tables (Intra/Inter) with 16x2x16 sizes for luma 4x4

- 3 Tables with 29x2x64 entries, where 3 tables = 2 (Intra/Inter) for luma 8x8 case + 1 (chroma).

- 2 Tables with 64x2x256 entries, where 2 tables = 2 (Intra/Inter) for luma 16x16 and 32x32.

Note: These tables contain unused entries that can be removed to save memory.

Table 2 Comparison between the proposed technique and TMuC0.9

|  | TMuC0.9 | Proposal |
| --- | --- | --- |
| Number of sorting tables | 1 | 9 |
| Table sizes | 29x2x64 | 16x2x16x2+29x2x64x3+64x2x256x2 |

- Experimental results

## Software and coding conditions

The proposed algorithm is implemented in HM1.0 or TMuC0.9. Experiments are conducted by using the same configurations according to the common test conditions for low-complexity configuration [2].

## Results using differentiated sorting tables

Table 3 shows the results using 7 differentiated sorting tables for run-mode. The only difference between the tested technique and TMuC0.9 is those sorting tables. The BD-rate savings on average are 1.7%, 0.7% and 0.2% for Intra, RA and LD respectively. There is no complexity introduced by this technique on both encoder and decoder sides.

Table 3 Experimental results using differentiated sorting tables

|  | Intra LoCo |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -2.0 | -1.5 | -1.5 |
| Class B | -1.5 | -1.5 | -1.6 |
| Class C | -1.5 | -1.3 | -1.4 |
| Class D | -1.8 | -1.6 | -1.6 |
| Class E | -1.7 | -1.5 | -1.4 |
| All | -1.7 | -1.5 | -1.5 |
|  |  | Random access LoCo |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.8 | -0.3 | -0.6 |
| Class B | -0.7 | -0.7 | -0.6 |
| Class C | -0.6 | -0.5 | -0.6 |
| Class D | -0.7 | -0.7 | -0.7 |
| Class E |  |  |  |
| All | -0.7 | -0.6 | -0.6 |
|  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |
| Class B | -0.3 | -0.2 | -0.3 |
| Class C | -0.3 | -0.2 | -0.3 |
| Class D | -0.3 | -0.2 | -0.3 |
| Class E | -0.2 | 0.3 | -0.4 |
| All | -0.2 | 0.0 | -0.3 |

## Results using adaptive run-mode coding

Table 4 shows the results using adaptive sorting tables for run-mode. The difference between the tested technique and TMuC0.9 is to update the sorting table after each run is encoded. The initial values for the sorting table are obtained based on training with VCEG sequences. The BD-rate savings on average are 1.3%, 0.7% and 0.0% for Intra, RA and LD respectively.

Table 4 Experimental results using adaptive run-mode coding

|  | Intra LoCo |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.6 | -1.3 | -1.3 |
| Class B | -1.3 | -1.3 | -1.3 |
| Class C | -1.2 | -1.1 | -1.1 |
| Class D | -1.3 | -1.1 | -1.2 |
| Class E | -1.3 | -0.9 | -1.0 |
| All | -1.3 | -1.1 | -1.2 |
|  |  | Random access LoCo |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.9 | -0.2 | -0.4 |
| Class B | -0.8 | -0.1 | -0.2 |
| Class C | -0.7 | 0.1 | -0.1 |
| Class D | -0.5 | -0.1 | 0.0 |
| Class E |  |  |  |
| All | -0.7 | -0.1 | -0.1 |
|  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |
| Class B | 0.0 | 1.7 | 1.4 |
| Class C | -0.3 | -0.2 | -0.3 |
| Class D | 0.3 | 1.4 | 0.9 |
| Class E | -0.2 | 0.3 | -0.4 |
| All | 0.0 | 0.9 | 0.5 |

## Results using large TU coding and differentiated tables

In this experiment, we implement to encode entire TUs with size of 16x16 and 32x32. Also, differentiated tables are introduced for Intra/Inter luma of 16x16 and 32x32. Combined with the technique proposed in Section 2.1, there are 7 sorting tables used for run-mode. Table 5 shows the results using large TU coding and differentiated sorting tables for different TU sizes and types in run-mode. The sorting tables are obtained based on training with VCEG test sequences. The BD-rate savings on average are 2.7%, 1.5 % and 0.2 % for Intra, RA and LD respectively.

Table 5 Experimental results using large TU coding and differentiated tables

|  | Intra LoCo |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -3.8 | -1.5 | -1.7 |
| Class B | -3.1 | -1.7 | -1.8 |
| Class C | -1.8 | -1.4 | -1.5 |
| Class D | -2.1 | -1.7 | -2.0 |
| Class E | -3.2 | -2.7 | -2.6 |
| All | -2.7 | -1.8 | -1.9 |
|  |  | Random access LoCo |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.7 | 0.3 | -0.1 |
| Class B | -2.1 | 0.0 | 0.0 |
| Class C | -1.1 | -0.3 | -0.3 |
| Class D | -1.0 | -0.7 | -0.7 |
| Class E |  |  |  |
| All | -1.5 | -0.2 | -0.3 |
|  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |
| Class B | -0.3 | 1.7 | 1.7 |
| Class C | -0.2 | 0.5 | 0.5 |
| Class D | -0.3 | 0.5 | 0.2 |
| Class E | 0.2 | 2.5 | 1.8 |
| All | -0.2 | 1.2 | 1.1 |

# Conclusion

In this document, several techniques are proposed to address shortcomings of current transform coding in LCEC of TMuC0.9. Table differentiation and adaptive sorting tables provide adaptation to various distributions of runs for different TU sizes and types. Experimental results demonstrate that each technique can improve the coding efficiency of TMuC0.9.  There will be extra tables introduced by proposed solutions. However, there are redundancies among those tables. The table sizes can be further reduced.

We propose to consider these techniques in HM for the performance improvements with reasonable cost.

# Reference

- Samsung, “Efficient coefficient coding method for large transform in VLC mode”, JCTVC-C210, Oct. 2010

- F. Bossen, “Common TM Test Conditions,” JCTVC-C500, 3rd. Meeting, Guangzhou, CN, 7-15 Oct. 2010

# Patent rights declaration(s)

(NOTE – Activities in the JCT-VC and contributions to the JCT-VC are subject to the common patent policy for ITU-T/ITU-R/ISO/IEC. A statement of that policy can be found at
http://www.itu.int/ITU-T/dbase/patent/patent-policy.html, with further information available at http://www.itu.int/ITU-T/ipr/index.html and in the ISO/IEC Directives. The form to be used for the formal reporting of patent rights to ITU-T/ITU-R/ISO/IEC can be found at http://www.itu.int/ITU-T/ipr/index.html. Contributions to the JCT-VC proposing normative technical content shall contain a non-binding informal notice of whether the submitter may have patent rights that would be necessary for implementation of the resulting standard. The provided informal notice shall indicate the category of anticipated licensing terms according to the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form. This obligation to provide an informal notice is supplemental to, and does not replace, any existing obligations of parties with technology included in a final or draft standard to submit formal IPR declarations to ITU-T/ITU-R/ISO/IEC. An example of an informal IPR notification statement for a contribution is provided below.)

Sony Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).