| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D262 WG11 Number: m19026 |
| --- | --- |

| Title: | Parallel Context Processing for the significance map in high coding efficiency |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | J. Sole, R. Joshi, I. S. Chong, M. Coban, M. Karczewicz 5775 Morehouse Drive, San Diego, CA 92121-1714 USA | Tel: +1-858-845-2429 Email: joels@qualcomm.com |  |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

This proposal presents a technique for the parallelization of context processing to improve the throughput of the entropy coder for the high efficiency case. The position of the last significant coefficient is encoded before the position of the other significant coefficients within a block. The position of the last coefficient is encoded explicitly by signaling its X and Y coordinates with a unary code. X and Y signaling is independent. The context derivation for the significance map is simplified to further enhance parallelization. The parallelization improvements of the proposal come at no cost in performance. The BD-rate for the high efficiency intra, random access, and low-delay configurations is 0.06℅, 0.01℅, and  -0.17℅, respectively.

- Introduction

The CABAC engine is highly serial, which poses practical problems for real-time processing, especially at high bit-rates. The serial nature of CABAC stems from the following three blocks: binarizer, context modeler, and binary arithmetic coder (BAC), as discussed in [1]. One of the solutions to alleviate serial nature of CABAC is syntax element partitioning, which parallelizes the BAC, the context modeler and the binarizer [1]. In this solution, all the syntax elements are grouped into several groups: MB information, prediction information, CBP information, significance map (significance and last), and coefficient (i.e., sign and level). Each group is processed separately. For the significance map and coefficient group whose total bins are larger than other groups (30% and 26%, respectively) additional parallel context processing (PCP) schemes are introduced (i.e., coefficient sign PCP, coefficients level BinIdx 0 PCP, and significance map PCP). All these solutions increase the speed of CABAC processing on average. However, the above approach has limitations when considering hardware (HW) implementations. First, balancing each group’s load is not obvious because it is highly content and QP dependent. This makes it hard to guarantee the worst case efficiency in the HW implementation sense. Furthermore, speculation-based parallelization approaches increase HW area and complexity significantly. Therefore, it is not easy to guarantee a level of parallelization in CABAC with reasonable HW complexity cost.

In spite of these limitations, the above solutions can alleviate the serial intrinsic nature of CABAC. Therefore, we pursue the direction proposed in [1, 2] to further alleviate the drawback of serial nature of CABAC. In [2], there were three types of parallel context processing schemes: coefficient sign PCP, coefficients level BinIdx 0 PCP, and significance map PCP. Among these, significance map PCP requires more focus, because at high bit-rates, a large portion of the bins (around 30% of the total) are generated by the significance map.

The significance map encoding in the HM is carried out by interleaving the bin indicating the significance of a coefficient (significant_coeff_flag)  and the bin indicating whether the coefficient is the last significant within the block (last_significant_coeff_flag) when the flag significant_coeff_flag equals 1. The drawback of this approach is that the decision regarding encoding of the last_significant_coeff_flag for a particular transform coefficient is dependent on the value of the significant_coeff_flag. This makes it difficult to parallelize the encoding of the significance map using speculative computing. Although significance map PCP (i.e., N significance 1 last) in [2] tries to tackle this problem, the performance loss (around 0.1~0.3%) is not negligible even for N=2. In this proposal, the significance map PCP has been improved, so that it can easily conform with speculative computing by reducing the complexity of speculative computation. Note that more complex speculative computation leads to increase in power consumption.

Several scans have been proposed for the TMuC, among them the zigzag, HHI [2] and the wavefront [3]. This proposal has been tested for these three scans reaching equivalent results. The rest of this proposal describes the current setting, that is, the HHI scan (macro HHI_DISABLE_SCAN equals 0).

- Technical description

## Last Significant Coefficient Signaling

An aim of this proposal is to separate the encoding of the significant_coeff_flag and the last_significant_coeff_flag. To do that, first, the position of the last significant coefficient is coded, and once the last nonzero coefficient is determined, then the significant_coeff_flags are coded. The position of the last coefficient in the block is coded by explicitly signaling its (X, Y)-coordinates. ‘X’ indicates the column number and ‘Y’ the row number (see Fig. 1). The coordinates are coded as a unary code starting from position 0 and moving to the next line (row or column) until the line of the last coefficient is met. For example, if the X coordinate of the coefficient is 0, the symbol 1 is encoded, if the coordinate is 1, symbols 01 are encoded, if it is 2, symbols 001 are encoded, and so on. The unary number is encoded with CABAC and for each block size the context depends solely on the line number. Therefore, there is no context derivation overhead. In addition, the X and Y coordinates are encoded independently.

Figure 1. Explicit encoding of X and Y coordinates for the last significant coefficient

This method has several advantages over the current one:

- It benefits the implementation of parallel algorithms by the separation of the significant coefficient and the last coefficient planes (see Fig. 2; for more detail compare to Fig. 9 in [1]).

- Additionally, the X and Y component are encoded independently, and thus, allowing further parallelization within the last coefficient encoding algorithm itself.

- Parallelization and speculative computation of the significant coefficients become easier because the stopping point (i.e., last significant coefficient of the block) is known beforehand.

- The worst-case scenario for the number of bins for last flag is reduced. For an N×N block, the maximum number of bins for the last flag goes from N*N-1 down to 2*N-2 bins.

Figure 2. Parallelization of context processing for significance map

## Reduced Context Selection for Significant Map

On the lines of JCT-VC contribution [3], a modification in the context neighbourhood for the significant_coeff_flag is proposed in order to further enhance parallelization. Current neighbourhood is depicted in Fig. 3 for position X. The 9 positions in gray and one of the diagonal neighbours (depicted by o) are used. The dependency between coefficients on the same diagonal is removed by non-inclusion of the diagonal element in the context. That is, the serial bottleneck of context derivation for coefficients within a diagonal is eliminated so all of them can be computed in parallel. This enhancement comes at a cost of less than 0.1% in BD-rate.

Figure 3. Context neighborhood for significant coefficient

A second modification is introduced in order to simplify the context and improve the performance. The position of the last significant coefficient is known when the significant_coeff_flags are encoded. This information is utilized in the following way. If the last significant coefficient is found within the first 10 coefficients (gray area in Fig. 4), then a set of contexts depending solely on the scan position is used. There is no cost in computing these contexts, since the scan position is available. If the last significant coefficient is not one of the first 10 coefficients in the scanning order, then the more complex regular contexts are computed.

Figure 4.  Coefficients in gray use a simple context based on the scanning position

- Results

The proposed modifications described above are implemented in the TMuC 0.9 software. Simulation results are presented for the full sequences with high efficiency common test conditions. Table 2 shows the results. The BD-rate is very close to the baseline, being 0.06℅, 0.01℅ and -0.17℅ for intra, random access and low delay configurations, respectively (negative numbers indicate BD-rate gains). Similarly, regarding complexity, no meaningful difference is observed with respect to the baseline, both at the encoder and the decoder. Detailed results are provided in the accompanying excel file.

Table 2. BD-rate for the proposal (high efficiency)

| BD-rate | Intra |  |  | Random access |  |  | Low delay |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.04 | -0.25 | -0.14 | -0.05 | -0.66 | -0.21 |  |  |  |
| Class B | 0.06 | -0.28 | -0.27 | 0.00 | -0.76 | -0.77 | -0.23 | -1.40 | -1.47 |
| Class C | 0.06 | 0.01 | -0.01 | 0.06 | -1.09 | -1.01 | -0.14 | -1.23 | -1.30 |
| Class D | -0.05 | -0.20 | -0.01 | 0.01 | -1.14 | -1.10 | -0.27 | -2.90 | -1.98 |
| Class E | 0.27 | -0.22 | -0.67 |  |  |  | -0.01 | -0.67 | -2.32 |
| All | 0.06 | -0.18 | -0.20 | 0.01 | -0.93 | -0.85 | -0.17 | -1.59 | -1.72 |
| Enc Time[%] | 96% |  |  | 101% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  | 102% |  |  |

- Conclusions

An algorithm for parallelization of the significance map encoding is proposed. The parallelization advantage of the algorithm comes at no performance cost. The BD-rate is 0.06℅, 0.01℅ and -0.17℅ for intra, random access and low delay configurations, respectively.

- References

- M. Budagavi, M. U. Demircin, “Parallel context processing techniques for high coding efficiency entropy coding in HEVC”, JCTVC-B088, 2nd JCT-VC Meeting, Geneva, CH, July 2010.

- M. Budagavi, “TI Parallel context processing (PCP) proposal”, JCTVC-C062, 3rd JCT-VC Meeting, Guangzhou, CN, Oct 2010.

- M. Winken et al, “Description of video coding technology proposal by Fraunhofer HHI”, JCTVC-A116, 1st JCT-VC Meeting, Dresden, DE, April 2010.

- V. Sze, M. Budagavi, “Parallelization of HHI_TRANSFORM_CODING”, JCTVC-C227, 3rd JCT-VC Meeting, Guangzhou, CN, October 2010.

# Patent rights declaration(s)

Qualcomm may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).