| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: , CH, - January, 2011 | Document: JCTVC-D263 |
| --- | --- |

| Title: | Parallel deblocking filter |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Masaru Ikeda Junichi Tanaka Teruhiko Suzuki Gate City Osaki West Tower East Tec, 1-11-1, Osaki, Shinagawa-ku , Tokyo, 141-0032 JAPAN | Tel: Email: | +81 3 5435 3308 Masaru.Ikeda@jp.sony.com Tanaka.Junichi@jp.sony.com teruhikos@jp.sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

# Abstract

This document gives proposal about deblocking filter in TMuC-0.9. First, the filter process is made parallel by changing the order of horizontal filtering (across vertical edge) and vertical filtering (across horizontal edge). Second, edge judgment of block boundary for deblocking filter is made parallel by changing the position of lines which are used for the judgment.

This document has five-part, introduction is presented in the 1st part, proposal and the effect are presented in the 2nd part, the experimental results are presented in the 3rd part, conclusion is presented in the 4th part and references are presented in the 5th part.

# Introduction

In conventional deblocking filter [1], horizontal filter (across vertical edge) is dependent on vertical filter (across horizontal edge) between 1st Largest Coding Unit (LCU) and 2nd LCU because the filtering is first performed across vertical edges, then across horizontal edges in LCU. As shown Figure 1 the red circles show pixels performed by both vertical filter across horizontal edges (H1_0, H1_1) in 1st LCU and horizontal filter across vertical edges (V2_0, V2_1) in 2nd LCU. Vertical filtering across horizontal edges (H1_0, H1_1) in 1st LCU needs to be completed before horizontal filtering across vertical edges (V2_0, V2_1) in 2nd LCU. In other words, the parallelism can only be performed within LCTB, and so we would like to propose the parallelism of deblocking filter as follows.

Figure 1: Relationship between 1st LCU and 2nd LCU

# Proposal

## Parallelism of filtering

We would like to propose to make deblocking filter parallel by changing the order of horizontal filtering (across vertical edge) and vertical filtering (across horizontal edge). The way to change is as follows. First, the filter operates across all the vertical edges in processing frame, then across all the horizontal edges in the frame. As a result horizontal filtering (across vertical edge) is independent of vertical filtering (across horizontal edge), so both horizontal and vertical filtering can be performed in parallel in processing frame respectively.

## Parallelism of edge judgment of block boundary

As shown Figure 2-1(a), vertical judgment across horizontal edge uses both 3rd and 6th lines in conventional filter, and the judgment in #n LCU which uses red pixels depends on horizontal filtering across vertical edge in #n LCU. We propose the edge judgment uses both 4th and 5th lines to remove the dependency as shown Figure 2-1(b), because these lines are composed of not filtered pixels but reconstruct pixels. The proposal judgment leads to decide whether all block edges in processing frame are filtered or not on ahead and in parallel.

Figure 2-1: Edge judgment of block boundary for deblocking filter

## Expected effects

Proposed method gives us some effects.

- More flexible deblocking filter configuration

- We can freely design the number of the parallelization according to the design specification and start the filtering from desired position in the same direction (vertical/horizontal) edges, because the filtered pixels don’t overlap in adjacent edge as shown Figure 2-2.

- Parallel delocking filter can operate serially in LCU unit as conventional method. For example, the filter is performed as shown Figure 2-3. First, the filter operates across vertical edges in #n and #n+1 LCTB (Figure 2-3(a)). Second, the filter operates across horizontal edges in #n LCTB (Figure 2-3(b)), and the filtering in #n LCTB is completed. So, vertical filtering across horizontal edge in #n LCU needs that horizontal filtering across vertical edge in #n+1 LCU is completed. The proposed filtering can also operate serially in LCU as mentioned above.

- Process of deblocking filter across the slice boundary.

- It isn’t friendly to operate across the slice boundary between 1st and 2nd slice with the conventional filter, because the filtering in 2nd slice can’t start operating across edges without completing the filtering across both vertical and horizontal edges in all LCTBs in 1st slice.

The proposed filter can start operating across vertical edges all together in both 1st and 2nd slice, because one horizontal filtering across vertical edge is independent of the others as shown Figure2-2. Vertical filter can’t operate across horizontal edges in 1st LCU in 2nd slice without completing horizontal filtering across vertical edges in final LCU-line in 1st slice, because as shown Figure 2-4 the pink edge is filtered with using the blue pixels with the violet square which are operated by horizontal filtering in 1st slice. But the vertical filter in 2nd slice first can operate across horizontal edges below 2nd LCU-line, then across the edges in 1st LCU-line finally or as soon as the filtering is completed across vertical edges in final LCU-line in 1st slice. So, it is friendly to operate across the slice boundary with the proposed filter.

Figure 2-2: Filtered pixels in adjacent edge

Figure 2-3: Proposed filter process in LCU unit

Figure 2-4: Proposed filter process in slice boundary

# Experimental Results

First the experimental result of the parallelism of filtering and judgment is shown below. Second the result of the parallelism of only filtering is shown below.

## Parallelism of filtering and judgment

BD-rate for intra in both high efficiency and low complexity is shown in Table 3-1, that for random access is shown in Table 3-2 and that for low delay is shown in Table 3-3.

BD-rate of intra is 0.1 in both conditions (high efficiency and low complexity), that of random access is 0.1 in both conditions and that of low delay is 0.2 in both conditions in average, so they show slightly larger, but are still small enough.

|  | Intra High Efficiency |  |  | Intra Low Complexity |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate (%) | U BD-rate (%) | V BD-rate (%) | Y BD-rate (%) | U BD-rate (%) | V BD-rate (%) |
| Class A | 0.1 | 0.0 | 0.0 | 0.1 | 0.0 | 0.0 |
| Class B | 0.1 | 0.0 | 0.0 | 0.1 | 0.0 | 0.0 |
| Class C | 0.1 | 0.0 | 0.0 | 0.1 | -0.1 | -0.1 |
| Class D | 0.1 | 0.0 | 0.0 | 0.1 | 0.0 | -0.1 |
| Class E | 0.1 | 0.0 | 0.0 | 0.1 | 0.0 | 0.0 |
| All | 0.1 | 0.0 | 0.0 | 0.1 | 0.0 | 0.0 |

Table 3-1: BD-Rate for intra

|  | Random access High Efficiency |  |  | Random access Low Complexity |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate (%) | U BD-rate (%) | V BD-rate (%) | Y BD-rate (%) | U BD-rate (%) | V BD-rate (%) |
| Class A | 0.1 | 0.2 | 0.3 | 0.1 | 0.0 | 0.1 |
| Class B | 0.1 | 0.1 | -0.1 | 0.1 | 0.1 | 0.0 |
| Class C | 0.1 | 0.0 | -0.1 | 0.1 | 0.1 | 0.0 |
| Class D | 0.1 | -0.1 | 0.1 | 0.1 | 0.0 | 0.1 |
| Class E |  |  |  |  |  |  |
| All | 0.1 | 0.0 | 0.0 | 0.1 | 0.1 | 0.0 |

Figure 3-2: BD-Rate for random access

|  | Low delay High Efficiency |  |  | Low delay Low Complexity |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate (%) | U BD-rate (%) | V BD-rate (%) | Y BD-rate (%) | U BD-rate (%) | V BD-rate (%) |
| Class A |  |  |  |  |  |  |
| Class B | 0.2 | -0.1 | -0.2 | 0.2 | 0.0 | -0.1 |
| Class C | 0.1 | 0.0 | 0.1 | 0.2 | -0.1 | 0.1 |
| Class D | 0.1 | -0.4 | 0.1 | 0.1 | 0.3 | 0.2 |
| Class E | 0.4 | 0.4 | 0.0 | 0.4 | 0.3 | 0.1 |
| All | 0.2 | -0.1 | 0.0 | 0.2 | 0.1 | 0.1 |

Figure 3-3: BD-Rate for low delay

## Parallelism of filtering

BD-rate for intra in both high efficiency and low complexity is shown in Table 3-4, that for random access is shown in Table 3-5 and that for low delay is shown in Table 3-6.

BD-rate of intra is 0.0 in both conditions (high efficiency and low complexity), that of random access is 0.0 in both conditions and that of low delay is 0.0 in both conditions in average, and some of them show slightly gain. So, parallelism of filtering has as same performance as conventional filter.

|  | Intra High Efficiency |  |  | Intra Low Complexity |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate (%) | U BD-rate (%) | V BD-rate (%) | Y BD-rate (%) | U BD-rate (%) | V BD-rate (%) |
| Class A | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class B | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | -0.1 | -0.1 |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | -0.1 |
| Class E | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| All | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |

Table 3-4: BD-Rate for intra

|  | Random access High Efficiency |  |  | Random access Low Complexity |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate (%) | U BD-rate (%) | V BD-rate (%) | Y BD-rate (%) | U BD-rate (%) | V BD-rate (%) |
| Class A | 0.0 | 0.2 | 0.3 | 0.0 | 0.1 | 0.0 |
| Class B | 0.0 | 0.0 | 0.0 | 0.0 | 0.1 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.1 | 0.1 | 0.0 | 0.0 | 0.0 |
| Class E |  |  |  |  |  |  |
| All | 0.0 | 0.1 | 0.0 | 0.0 | 0.0 | 0.0 |

Figure 3-5: BD-Rate for random access

|  | Low delay High Efficiency |  |  | Low delay Low Complexity |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate (%) | U BD-rate (%) | V BD-rate (%) | Y BD-rate (%) | U BD-rate (%) | V BD-rate (%) |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | 0.0 | -0.3 | 0.0 | -0.1 | -0.1 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class D | 0.1 | -0.5 | 0.3 | -0.1 | 0.0 | -0.1 |
| Class E | -0.1 | 0.2 | -0.1 | 0.0 | 0.1 | 0.0 |
| All | 0.0 | -0.1 | 0.0 | 0.0 | 0.0 | 0.0 |

Figure 3-6: BD-Rate for low delay

# Conclusion

Parallelism of filtering and judgment has slight losses but they are still small enough, and it has more flexible configuration than conventional filter and parallelism of only filtering. So we would like to propose parallelism of both filtering and judgment.

# References

[1] JCT-VC, “Draft Test Model under Consideration”, JCTVC-B205, JCT-VC Meeting, Geneva, July 2010.

Sony Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).