| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D264 WG11 Number: m19028 |
| --- | --- |

| Title: | CE7: A crosscheck of Toshiba’s proposal (JCTVC-D107) by Qualcomm |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | R. Joshi, and P. Chen | Email: rajanj@qualcomm.com Tel.: +1-858-658-4511 |  |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

This contribution provides a summary of a cross-check performed by Qualcomm of Toshiba’s one-dimensional directional unified transform (1DDUT) proposal (JCTVC-D107). The results of our crosscheck matched those reported by Toshiba.

# Introduction

Toshiba’s 1DDUT aims to reduce the number of KLT matrices used in the original MDDT proposal. 1DDUT uses only 2 matrices, one is a KLT matrix based on training and the other one is the DCT type-II matrix. A total of 4 combinations of these 2 matrices can be used to transform the intra prediction residual. The specific mapping used is entirely dependent on the intra prediction mode. After transform, a total of 9 fixed scans are used to encode the quantized transform coefficient.

- Results

Results for Intra and random access (RA) configurations for both high efficiency (HE) and low complexity (LC) were crosschecked. Table 1 shows the summary of the results. More detailed coding results can be found in the Excel attachment. No YUV files were written when the decoding times were measured. Encoding was done on a LINUX cluster and decoding was done on a PC using Microsoft Visual C++ (64-bit).

Table 1: BD-rate results for intra and RA configurations for HE and LC

# Conclusions

We have verified Toshiba’s results for one-dimensional directional unified transform (1DDUT) proposal (JCTVC-D107). The proposed method achieves BD-rate improvement of 2.1% - 2.3% for Intra and 0.8% - 0.9% for RA. Encoding and decoding complexity is similar to TMuC 0.9.