| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D265 |
| --- | --- |

| Title: | Cross-check results of JCTVC-D234: random access support for HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Woo-Jin Han M/M Platform Lab DMC R&D Center Suwon, Korea | Tel: Email: | +82-10-3329-6393 wjhan.han@samsung.com |
| Source: | Samsung Electronics |  |  |

_____________________________

# Cross-checking activities

Two schemes, IDR and deferred decoding refresh (DDR), which are proposed in JCTVC-D234, have been tested. Both schemes are developed to add the random accessibility feature to the current HM. Since they are irrelevant to the low-delay configuration, only the results of the random access configuration are verified.

Firstly, we have received the full source code and the simulation results. The source code was investigated to verify the correctness between the software and the description and we confirm that both methods (IDR and DDR) are implemented as described in the technical description in JCTVC-D234.

The simulation using the high-efficiency configuration was performed. The results are summarized as follows.

| HM0.9 IDR vs. HM0.9 default |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Random access |  |  | Random access LoCo |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 7.8 | 10.3 | 9.8 | 7.1 | 7.4 | 7.4 |
| Class B | 8.6 | 13.0 | 13.0 | 7.4 | 8.1 | 7.3 |
| Class C | 5.4 | 6.1 | 6.5 | 4.6 | 4.9 | 5.0 |
| Class D | 4.9 | 5.9 | 6.3 | 4.2 | 4.5 | 4.7 |
| Class E |  |  |  |  |  |  |
| All | 6.7 | 8.9 | 9.0 | 5.8 | 6.2 | 6.0 |
| Enc Time[%] | 98% |  |  | 98% |  |  |
| Dec Time[%] | 98% |  |  | 98% |  |  |

| HM0.9 DDR vs. HM0.9 default |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Random access |  |  | Random access LoCo |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 2.9 | 4.2 | 4.1 | 2.3 | 1.9 | 2.1 |
| Class B | 2.7 | 5.6 | 6.2 | 1.7 | 1.8 | 1.5 |
| Class C | 1.4 | 1.5 | 1.8 | 0.7 | 0.5 | 0.5 |
| Class D | 1.0 | 1.7 | 2.0 | 0.3 | 0.2 | 0.4 |
| Class E |  |  |  |  |  |  |
| All | 1.9 | 3.3 | 3.6 | 1.1 | 1.0 | 1.0 |
| Enc Time[%] | 96% |  |  | 93% |  |  |
| Dec Time[%] | 97% |  |  | 97% |  |  |

| HM0.9 DDR vs. HM0.9 IDR |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Random access |  |  | Random access LoCo |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -4.4 | -5.4 | -5.1 | -4.5 | -5.1 | -4.9 |
| Class B | -5.4 | -6.5 | -6.0 | -5.3 | -5.8 | -5.4 |
| Class C | -3.8 | -4.3 | -4.3 | -3.8 | -4.2 | -4.3 |
| Class D | -3.8 | -3.9 | -4.0 | -3.7 | -4.2 | -4.2 |
| Class E |  |  |  |  |  |  |
| All | -4.4 | -5.1 | -4.9 | -4.4 | -4.8 | -4.7 |
| Enc Time[%] | 98% |  |  | 95% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |

All results including the PSNR, bit-rates and the running time are matched to the values reported in the JCTVC-D234, thus we would like to confirm that all reported results could be reproduced without any problem.

In addition, we ran the decoder to start the decoding process from the arbitrary position by providing the command-line option ‘-s <starting position in output order>’ for several bit-streams by both IDR and DDR cases, and we could find that both IDR and DDR allows the random accessibility feature while the current HM decoder couldn’t.

# Comments

The following is the summary of the changes proposed in JCTVC-D234:

- IDR

- DDR

- Software enhancement enabling to specify the random accessibility type in the encoder side by ‘-dr <random accessibility type>’ option

- Software enhancement enabling to start the decoding process from the arbitrary position by ‘-s <staring position in output order>’ option

As a cross-checker, we would like to recommend adopting 1st, 3rd, and 4th items since they are relatively trivial and already proven to be useful in the previous standards.

Although we strongly agree that the DDR is useful and provides good coding efficiency compared to the IDR solution, we are not sure that this approach is the best one to achieve the same functionality.

From our understanding, there was an initial consensus to re-use AVC high-level syntax schemes in HEVC standard. Since the similar functionality and coding efficiency could be achieved by the schemes existing in AVC, rather than adopting the concept of DDR at this stage, we would like to suggest more discussion in the group especially with considerations of the high-level syntax design of HEVC standard.