| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D267 WG11 Number: m19032 |
| --- | --- |

| Title: | Crosscheck of Institute for Infocomm Research's temporal merge proposal (JCTVC-D051) by Qualcomm |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | R. Joshi and W.-J. Chien | Email: rajanj@qualcomm.com Tel.: +1-858-658-4511 |  |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

This contribution provides a summary of a cross-check performed by Qualcomm of Institute for Infocomm Research's temporal merge proposal (JCTVC-D051). The results from our crosscheck matched those provided by Institute for Infocomm Research.

# Introduction

This contribution provides verification results for JCTVC-D051 “Merge/Skip/Direct Simplification” proposed by Institute for Infocomm Research. A skip mode is replaced by a “Temporal Merge” mode. The prediction mechanism of the proposed temporal merge mode is identical to that of the co-located predictor in AMVP scheme. Bi-directional prediction (whenever possible) is done on the reference frames with reference indices of zero and the motion vectors are copied from the co-located CUs. Like in the Skip mode, no coefficients are decoded when the temporal merge mode is used. In the proposed design, the direct mode is removed, leaving behind the original (spatial) Merge modes and the proposed Temporal Merge mode.

- Results

Results for random access (RA) and low delay (LD) configurations for both high efficiency (HE) and low complexity (LC) were crosschecked. Table 1 shows the summary of the results. More detailed coding results can be found in the Excel attachment. No YUV files were written when measuring the decoding times. Encoding was done on a LINUX cluster and decoding was done on a PC using Microsoft Visual C++ (64-bit). Overall, the encoding time is reduced by 15 – 26%. The decoding times are similar to TMuC v09. For HE cases, there is a small loss in coding efficiency for luma components, whereas chroma components exhibit losses as well as gains. For LC configurations, there is BD-rate gains for luma as well as chroma. Also, the performance for low delay configurations is better than that for random access configurations.

Table 1: BD-rate results for RA and LD configurations for HE and LC

# Conclusions

We have verified Institute of Infocomm’s results for “temporal merge” proposal (JCTVC-D051). This method reduces encoding times by 15 – 26% with improved BD-rate performance for LC configurations and small loss (up to 0.21%) for luma component in HE configurations.