| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D269 |
| --- | --- |

| Title: | Verification of Qualcomm transform proposal (JCTVC-D256) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Cross-verification |  |  |
| Author(s) or Contact(s): | Madhukar Budagavi Texas Instruments Inc., USA | Tel: Email: | madhukar@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

This contribution presents results of cross-verification of Qualcomm transform proposal in JCTVC-D256. The PSNR/bit-rate numbers obtained were compared to those provided by Qualcomm. The Qualcomm and TI numbers matched. Qualcomm transform has similar BD-Rate performance as TMuC-0.9 on average (average BD-Rate is 0.0% for all six coding scenarios – Intra, Intra LoCo, Random access, Random access Loco, Low delay, Low delay LoCo). TMuC-0.9 decoder and encoder with Qualcomm transform have run times that are comparable to TMuC-0.9 (which uses Chen’s implementation). Decoding time ratios on the same PC are in the range of 100%-102% range. Encoder time ratios on heterogeneous Linux cluster range from 94%-104%.

# Simulation results

Encoder simulations were run on Linux cluster. Decoding of bitstreams was carried out sequentially on a PC with Intel(R) Core(TM)2 CPU 6600 @ 2.4GHz, 3.25 GB of RAM. Table below presents a summary of BD-Rate and encoder/decoder run-times in comparison with TMuC-0.9 anchors.

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.1 | 0.0 | -0.1 | 0.0 | 0.1 | 0.1 |
| Class B | 0.0 | -0.1 | -0.1 | 0.1 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class E | -0.1 | -0.1 | -0.2 | 0.0 | 0.0 | 0.0 |
| All | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 101% |  |  | 104% |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  |
|  | Random access |  |  | Random access LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.1 | 0.1 | 0.1 | 0.0 | 0.1 | 0.2 |
| Class B | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | 0.1 | 0.0 | -0.1 | -0.1 |
| Class D | 0.0 | 0.0 | 0.1 | 0.1 | 0.0 | 0.2 |
| Class E |  |  |  |  |  |  |
| All | 0.0 | 0.0 | 0.1 | 0.0 | 0.0 | 0.1 |
| Enc Time[%] | 94% |  |  | 94% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | -0.1 | 0.0 | 0.0 | 0.0 | -0.2 |
| Class C | 0.0 | 0.0 | -0.1 | 0.1 | -0.1 | 0.0 |
| Class D | 0.0 | -0.8 | 0.0 | -0.1 | 0.1 | -0.1 |
| Class E | -0.1 | -0.2 | -0.4 | -0.1 | -0.2 | 0.2 |
| All | 0.0 | -0.3 | -0.1 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 94% |  |  | 94% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |