| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D271 |
| --- | --- |

| Title: | CE4 : Chroma Interpolation Filtering using High Precision Filter |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jeong-Pil Kim Dae-Yeon Kim Yung-Lyul Lee Jeongyeon Lim | Tel: Email: Tel: Email: | +82-2-3408-3753 jpkim@dms.sejong.ac.kr dykim@dms.sejong.ac.kr yllee@sejong.ac.kr +82-31-710-5196 jeongyeon@sktelecom.com |
| Source: | Sejong university and SK Telecom, Rep. of Korea |  |  |

_____________________________

# Abstract

In this contribution, the experimental results and computational complexity of chroma interpolation filtering using high precision filter (HPF) are reported. This is one of proposals in CE4 on interpolation for MC (chroma). The experimental results show that the average BD-rate improvements on chroma U and V components are 7.5% and 8.0%, respectively, in the random access case of low complexity, and the average BD-rate improvements on U and V components are 4.2% and 5.1%, respectively, in the random access case of high efficiency. The average BD-rate improvements on U and V components are 8.1% and 9.7%, respectively, in the low delay case of low complexity, and the average BD-rate improvements on U and V components are 4.0% and 4.6%, respectively, in the low delay case of high efficiency. The average BD-rate improvements on Luma are 0.2% and 0.2%, respectively, in the random access case of low complexity and high efficiency, and the average BD-rate improvements on Luma are 0.9% and 0.5%, respectively, in the low delay case of low complexity and high efficiency. The complexity of the proposed method is also analyzed.

# Introduction

In this contribution, the experimental results and computational complexities of the proposed chroma interpolation filtering using high precision filter [1] are reported. This is one of the proposals in CE4 [2].

# Chroma interpolation filtering using high precision filter

The proposed method using 6-tab high precision filtering and linear filtering can be referred to from JCTVC-C126 [1].

The proposed method improves the coding efficiency of chroma U and V components but increases the computational complexity of those components in the decoder side compared with the bilinear interpolation in HM 0.9. Since the proposed method needs more integer pixels than the bilinear interpolation filtering in HM in order to reduce the prediction error, the computational complexity of the proposed method is increased. The number of pixel-by-pixel computations for interpolating the sub-pixels of Fig.1 is shown in Table 1.

Fig. 1. Integer pixels (blue blocks with capital letters), 1/2&/1/4-pixel positions (yellow blocks with lower-case letters) and 1/8-pixel positions (gray blocks).

Table 1. The pixel-by-pixel computational complexity of the proposed method vs. bilinear filtering

| Position |  | Proposed method |  | Bilinear filtering |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Multiplications | Additions | Multiplications | Additions |
| b, h | step1 | 2 | 5 | 0 | 1 |
|  | step1 | 2 | 5 |  |  |
| a, c, d, n | step2 | 1 | 1 |  |  |
|  | step1+step2 | 3 | 6 | 2 | 2 |
|  | step1 | 2 | 5 |  |  |
| oa, ob, oc, od, oe, oq, pe, pq | step2 | 2 | 1 |  |  |
|  | step1+step2 | 4 | 6 | 2 | 2 |
|  | step1 | 2 | 5 |  |  |
| j | step2 | 2 | 5 |  |  |
|  | 6*step1+step2 | 14 | 35 | 0 | 3 |
| e, g, p, r |  | 4 | 11 | 6 | 6 |
|  | step1 | 2 | 5 |  |  |
| f, i, k, q | step2 | 3 | 6 |  |  |
|  | 6*step1+step2 | 15 | 36 | 6 | 6 |
| oh, oj, or, oz, pf, pl, pt, px |  | 5 | 11 | 6 | 6 |
|  | step1 | 2 | 5 |  |  |
| oi, ow, pa, pb, pc, pd, pi, pw | step2 | 4 | 6 |  |  |
|  | 6*step1+step2 | 16 | 36 | 6 | 6 |
|  | step1 | 2 | 5 |  |  |
| of, ol, ot, ox, ph, pj, pr, pz | step2 | 4 | 6 |  |  |
|  | 6*step1+step2 | 16 | 36 | 6 | 6 |
| Remaining 16 sub-pixels | bilinear filtering |  |  | 6 | 6 |

The number of computations in step 1 applying the 6-tap FIR filter is described as follows:

{(E + J) - 5 * (F + I) + 20 * (G + H) + 16} / 32, an example of 6-tab filtering at the sub-pel position b

The number of computations in step 2 applying the linear filter at each position is as follows:

((G<<5) + b1 + 32) / 64, an example of linear filtering

Table 2 shows the complexity reduction when block-based filter is used. The # of computations can be referred to from the provided excel sheet.

Table 2. Block-based filtering to reduce complexity

|  |  |  | Multiple |  |  |
| --- | --- | --- | --- | --- | --- |
| Block size N | N*N |  | * | + | sum |
| 2 | 4 | (1) : Sum | 1700 | 3210 | 4910 |
|  |  | (2) : Worst/(N*N) | 11.00 | 23.50 | 34.50 |
|  |  | (3) : (1)/number of fractional pel | 26.98 | 50.95 | 77.94 |
|  |  | (4) : (3)/(N*N) | 6.75 | 12.74 | 19.48 |
| 4 | 16 | (1) : Sum | 5960 | 10740 | 16700 |
|  |  | (2) : Worst/(N*N) | 8.50 | 17.25 | 25.75 |
|  |  | (3) : (1)/number of fractional pel | 94.60 | 170.48 | 265.08 |
|  |  | (4) : (3)/(N*N) | 5.91 | 10.65 | 16.57 |
| 8 | 64 | (1) : Sum | 22160 | 38760 | 60920 |
|  |  | (2) : Worst/(N*N) | 7.25 | 14.13 | 21.38 |
|  |  | (3) : (1)/number of fractional pel | 351.75 | 615.24 | 966.98 |
|  |  | (4) : (3)/(N*N) | 5.50 | 9.61 | 15.11 |
| 16 | 256 | (1) : Sum | 85280 | 146640 | 231920 |
|  |  | (2) : Worst/(N*N) | 6.63 | 12.56 | 19.19 |
|  |  | (3) : (1)/number of fractional pel | 1353.65 | 2327.62 | 3681.27 |
|  |  | (4) : (3)/(N*N) | 5.29 | 9.09 | 14.38 |
| 32 | 1024 | (1) : Sum | 334400 | 569760 | 904160 |
|  |  | (2) : Worst/(N*N) | 6.31 | 11.78 | 18.09 |
|  |  | (3) : (1)/number of fractional pel | 5307.94 | 9043.81 | 14351.75 |
|  |  | (4) : (3)/(N*N) | 5.18 | 8.83 | 14.02 |

# Experimental Results

To verify the efficiency of the proposed method, the experiments were performed by using the reference software TMuC 0.9 under the common test conditions on JCTVC-C500 [3]. TMuC 0.9 and the proposed method were complied on Microsoft Visual Studio 2008 and compared in the same platform under Window7 64bit OS

In the random access and low delay case, the encoding times of the proposed method are increased about 1% compared with those of TMuC0.9 encoder.

Comparing the proposed method with TMuC 0.9 in the decoder side, the average decoding times are increased about 11% and 3%, respectively, in the random access case of low complexity and high efficiency, and the average decoding times are increased about 9% and 3%, respectively, in the low delay case of low complexity and high efficiency.

Table 3. Results in the case of random access (high efficiency and low complexity)

|  | Random access |  |  |  | Random access LoCo |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | YUV | Y | U | V | YUV |
| Class A | -0.1 | -1.7 | -2.4 | -0.7 | -0.1 | -2.7 | -3.2 | -1.1 |
| Class B | -0.1 | -3.9 | -5.6 | -1.6 | -0.1 | -7.2 | -7.0 | -2.3 |
| Class C | -0.2 | -4.3 | -4.3 | -1.7 | -0.2 | -8.2 | -8.0 | -3.0 |
| Class D | -0.3 | -5.5 | -6.6 | -2.2 | -0.2 | -9.5 | -11.5 | -3.2 |
| Class E |  |  |  |  |  |  |  |  |
| All | -0.2 | -4.2 | -5.1 | -1.7 | -0.2 | -7.5 | -8.0 | -2.6 |
| Enc Time[%] | 101% |  |  |  | 101% |  |  |  |
| Dec Time[%] | 103% |  |  |  | 111% |  |  |  |

Table 4.Results in the case of low delay (high efficiency and low complexity)

|  | Low delay |  |  |  | Low delay LoCo |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | YUV | Y | U | V | YUV |
| Class A |  |  |  |  |  |  |  |  |
| Class B | -0.4 | -2.4 | -3.3 | -1.3 | -0.9 | -9.7 | -11.5 | -3.9 |
| Class C | -0.5 | -3.7 | -4.4 | -2.0 | -0.9 | -10.2 | -9.5 | -4.7 |
| Class D | -0.7 | -5.6 | -5.9 | -2.3 | -1.3 | -9.7 | -11.6 | -4.3 |
| Class E | -0.2 | -5.1 | -5.6 | -1.8 | -0.2 | -0.5 | -4.2 | -0.8 |
| All | -0.5 | -4.0 | -4.6 | -1.8 | -0.9 | -8.1 | -9.7 | -3.6 |
| Enc Time[%] | 101% |  |  |  | 101% |  |  |  |
| Dec Time[%] | 103% |  |  |  | 109% |  |  |  |

# References

[1] Dae-Yeon Kim, Jeonn-Pil Kim, Yung-Lyul Lee, Jeongyeon Lim, Jinhan Song, "Chroma Interpolation Filtering using High Precision Filter", JCTVC of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd meeting: Guangzhou, CN, 7-15 October, 2010.

[2] Elena Alshina, "Core Experiment 4 : Interpolation for MC (Chroma)", JCTVC of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd meeting: Guangzhou, CN, 7-15 October, 2010.

[3] Frank Bossen, “Common test conditions and software reference configurations”, JCT-VC of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd meeting: Guangzhou, CN, 7-15 October, 2010.

# Conclusion

This contribution proposes a chroma interpolation method using 6-tap filtering and linear filtering with high precision. The experimental results show bit saving in chroma U and V components in all cases. We suggest adopting the best method among many CE4 proposals for a chroma interpolation method.

# Patent rights declaration(s)

Sejong University and SK telecom Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).