| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D272 |
| --- | --- |

| Title: | CE4 : Cross-check of Samsung’s DCT-IF 6-tab chroma filtering |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Jeong-Pil Kim Dae-Yeon Kim Yung-Lyul Lee Jeongyeon Lim | Tel: Email: Tel: Email: | +82-2-3408-3753 jpkim@dms.sejong.ac.kr dykim@dms.sejong.ac.kr yllee@sejong.ac.kr +82-31-710-5196 jeongyeon@sktelecom.com |
| Source: | Sejong university and SK Telecom, Rep. of Korea |  |  |

_____________________________

# Abstract

This document reports the verification of DCT-IF 6-tab chroma interpolation provided by Samsung. The verification task has been completed. The PSNR values and bitrates computed by Sejong University are closely matched those provided by Samsung. However, the encoding times and decoding times computed by Samsung show some differences compared with those computed by Sejong University and SKT.

# Experimental Results

TMuC 0.9 chroma filtering and Samsung’s DCT-IF 6-tab filtering were complied on Visual C++ 2008 and compared in the same platform under Window7 64bit OS. The experimental conditions were the same as JCTVC-C500 [1].

Table 1. Results in the case of random access (high efficiency and low complexity)

|  | Random access |  |  |  | Random access LoCo |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | YUV | Y | U | V | YUV |
| Class A | -0.1 | -2.4 | -3.2 | -1.0 | -0.1 | -3.5 | -4.1 | -1.5 |
| Class B | -0.1 | -4.8 | -6.6 | -1.9 | -0.1 | -7.5 | -7.8 | -2.5 |
| Class C | -0.2 | -5.1 | -5.0 | -2.0 | -0.2 | -8.4 | -8.2 | -3.0 |
| Class D | -0.3 | -6.5 | -8.1 | -2.5 | -0.2 | -9.6 | -11.8 | -3.3 |
| Class E |  |  |  |  |  |  |  |  |
| All | -0.2 | -5.0 | -6.1 | -2.0 | -0.1 | -7.8 | -8.5 | -2.7 |
| Enc Time[%] | 102% |  |  |  | 103% |  |  |  |
| Dec Time[%] | 106% |  |  |  | 118% |  |  |  |

Table 2. Results in the case of low delay (high efficiency and low complexity)

|  | Low delay |  |  |  | Low delay LoCo |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | YUV | Y | U | V | YUV |
| Class A |  |  |  |  |  |  |  |  |
| Class B | -0.5 | -3.7 | -4.3 | -1.6 | -0.9 | -9.3 | -10.9 | -3.7 |
| Class C | -0.6 | -4.2 | -4.5 | -2.1 | -0.9 | -10.9 | -10.1 | -5.0 |
| Class D | -0.8 | -8.8 | -10.1 | -3.1 | -1.4 | -10.7 | -13.5 | -4.5 |
| Class E | -0.3 | -4.5 | -5.0 | -1.7 | -0.3 | 6.2 | -0.6 | 0.3 |
| All | -0.6 | -5.3 | -5.9 | -2.1 | -0.9 | -7.1 | -9.4 | -3.5 |
| Enc Time[%] | 101% |  |  |  | 103% |  |  |  |
| Dec Time[%] | 105% |  |  |  | 116% |  |  |  |

# References

[1] Frank Bossen, “Common test conditions and software reference configurations”, JCT-VC of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd meeting: Guangzhou, CN, 7-15 October, 2010.