| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D273 WG11 Number: m19038 |
| --- | --- |

| Title: | Modified derivation process of temporal motion vector predictor |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Toshiyasu Sugio Takahiro Nishi | Tel: Email: | +81-50-3587-2047 { sugio.toshiyasu, nishi.takahiro }@jp.panasonic.com |
| Source: | Panasonic |  |  |

_____________________________

# Abstract

In this contribution, a modified derivation process of temporal motion vector predictor is proposed. It proposes that if a motion vector to be referred as co-located partition information were not available, the other motion vector in the partition may be used to derive temporal motion vector predictor same as AVC specification. Experimental result shows 0.4% bit saving for high efficiency and 0.3% for low complexity on average in the random access scenario relative to the TMuC0.9-hm.

- Introduction

Temporal motion vector predictor is important as one of the motion vector candidates as reported in JCTVC-B081 [1]. In the TMuC0.9-hm, if a motion vector to be referred as co-located partition information were not available, temporal motion vector predictor is not added to the motion vector predictor list, even if the co-located partition has another motion vector. This contribution proposes to modify the derivation process of temporal motion vector predictor to refer any of available motion vectors in the co-located partition.

# Proposed Method

An example of the derivation process of temporal motion vector predictor in the TMuC0.9-hm s is shown in Figure 1. In the example, slice type is equal to B and collocated_from_l0_flag is equal to 0, which means that the reference picture list 1 is used to identify the co-located partition. In this case, a motion vector mvL0 in the co-located partition, if available, is used to derive temporal motion vector predictor mvL0Col (left of Figure 1). Otherwise, i.e. in the case that co-located partition were coded as an intra prediction mode or as an inter prediction mode which has a motion vector mvL1 only, temporal motion vector predictor is not added to the motion vector predictor list, even if the co-located partition has a motion vector mvL1 (right of Figure 1).

Figure 1 derivation process of temporal motion vector predictor of the TMuC0.9-hm

Proposed derivation process of temporal motion vector predictor is shown in Figure 2. In this process, if the motion vector mvL0 were not available, availability of mvL1 is checked and, if available, the mvL1 is used to derive temporal motion vector predictor mvL1Col.

Figure 2 modified method

This is conceptually same as AVC specification, and it was modified to suit the TMuC0.9-hm. The flowchart of proposed selection process of mvCol is shown in Figure 3. Non-highlighted blocks are same as original TMuC0.9-hm process, and highlighted blocks are newly added in this proposal.

Figure 3 flowchart of selection process of mvCol for temporal motion predictor

# Experimental results

A proposed method was implemented on TMuC0.9-hm software and tested for four inter coding configurations. Table 1 shows its RD performance and execution time of the software relative to TMuC0.9-hm. In random access cases, proposed method saves 0.4% bits for high efficiency and 0.3% bits for low complexity on average. The difference in the encoding and decoding time is not obvious. On the other hand, in low delay case, there is no difference in RD performance because mvL1 cannot be available when mvL0 is not available due to the disabled prediction using only list 1 as proposed in [2].

Table 1 The RD performance of proposed method (anchor: TMuC0.9-hm)

Figure 4 and 5 show the BD rate for each sequences in random access case for high efficiency and low complexity, in which negative value means better RD performance relative to TMuC0.9-hm. It was observed that the proposed method saves bits for all sequences.

Figure 4 BD-Rate of random access case for high efficiency

Figure 5 BD-Rate of random access case for low complexity

# Conclusion

In this contribution, a modified derivation process of temporal motion vector predictor is proposed. An experimental result shows that a proposed method saves 0.4% bits for high efficiency and 0.3% bits for low complexity on average in the random access scenario. The proposal saves the number of bits consistently for all test sequences, and no negative impact on BD rate was observed. Moreover, the complexity impact is negligible as there is almost no difference in the encoding and decoding time relative to TMuC0.9-hm. Based on this consistent BD rate gain and negligible complexity impact, we recommend that proposed modified derivation process of temporal motion vector predictor be adopted in the HM/WD of HEVC.

# References

- Kazushi Sato, ”On Motion Vector Coding”, JCTVC-B081, July 2010

- Bin Li et al., ”Redundancy reduction in B-frame coding at temporal level zero”, JCTVC-C278, October 2010

# Patent rights declaration(s)

Panasonic Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).