| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D274 WG11 Number: m19039 |
| --- | --- |

| Title: | Modified usage of predicted motion vectors in forward directional bi-predictive coding frame |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Toshiyasu Sugio Takahiro Nishi | Tel: Email: | +81-50-3587-2047 {sugio.toshiyasu, nishi.takahiro }@jp.panasonic.com |
| Source: | Panasonic |  |  |

_____________________________

# Abstract

In this contribution, a modified usage of temporally/spatially predicted motion vector is proposed for the special B-frame, which uses forward directional bi-prediction. An experimental result shows 1.0% bits saving for high efficiency and 2.2% bits saving for low complexity on average in the low delay scenario relative to the TMuC0.9-hm.

- Introduction

In the common test conditions [1], the special B-frames, which use forward directional bi-prediction with reference picture list 0 and 1, are used as an alternative of the conventional P-frames in both of the low delay and random-access scenarios (Figure 1). These special B-frames (hereafter, referred as “Bf-frames” in this contribution) have two reference picture lists, i.e. list 0 and list 1, but the prediction using only list 1 (hereafter, referred as list 1 “uni-prediction” in contrast with “bi-prediction”) is disabled when list 0 and list 1 are identical as proposed in [2]. So Bf-frame just uses forward directional bi-prediction and list 0 uni-prediction, and it also has a flexibility to use forward directional bi-prediction referring to one single frame. However, there are some rooms for improvement in Bf-frame coding efficiency due to the disabled list 1 uni-prediction. This contribution proposes a modified usage of temporally/spatially predicted motion vector for skipped and direct prediction units and for the competition based motion vector prediction in Bf-frame.

Figure 1 position of Bf-frame for random access scenario (left) and low delay scenario (right)

# Proposed method

## Overview

This proposal includes the following four key points for Bf-frames:

- Uni-prediction in Skipped Prediction Units

- Bi-prediction in Direct Prediction Units

- Enhanced Reference Index Decision for Skipped and Direct PU

- Enhanced Temporal Motion Vector Predictor

## Uni-prediction in Skipped Prediction Units

In the TMuC0.9-hm, the list 1 uni-prediction has been disabled for Bf-frame, so the number of motion vector predictors for list 1 is usually less than that for list 0. So, in Bf-frame, accuracy of motion vector prediction from list 1 for bi-prediction tends to be worse than that from list 0. However, skipped prediction units in the TMuC0.9-hm are using bi-prediction only. As the result, the performance of skipped prediction units has been worsen by the prediction based on the list 1 and it cannot be recovered by residual signals due to the mode.

Based on the above consideration, in our proposal, only list 0 uni-prediction is used for skipped prediction units instead of bi-prediction in Bf-frame (Figure 2)

Figure 2 flowchart of selection process of used reference list number for skipped prediction units.

## Bi-prediction in Direct Prediction Units

In the TMuC0.9-hm, an explicit signaling by the flag “inter_pred_idc” is used to switch bi-prediction or uni-prediction for direct prediction units, which is same as normal inter prediction units. However, in Bf-frame, difference between the two RD-costs of bi-prediction and list 0 uni-prediction tends to be small in direct prediction units because one single reference frame is often used for both uni- and bi-prediction and motion vectors are likely to point the very near position. So, it is considered to be not worth while sending the flag for such a small difference.

That’s why, in our proposal, we use one single prediction scheme for direct prediction unit and the switching by inter_pred_idc has been removed for direct prediction unit in Bf-frames. Then, considering the combination with our proposal in the previous section, we use bi-prediction for direct prediction units in Bf-frame in order to keep a prediction mode which can reduce the motion vector overhead for bi-prediction (Figure 3).

Figure 3 flowchart of selection process of used reference list number for direct prediction units

## Enhanced Reference Index Decision for Skipped and Direct PU

In the TMuC0.9-hm, reference indices of skipped and direct prediction units are always set to 0. So, even if there were plural of available motion vectors which reference indices are 1, spatially neighboring motion vectors are regarded to be not available and motion vector (0, 0) is added to motion vector predictor candidates for skipped and direct prediction units.

In our proposal, reference indices for skipped and direct prediction units follow the majority of reference indices of neighboring prediction units.

## Enhanced Temporal Motion Vector Predictor

We proposed a modified derivation process of temporal motion vector predictor in [3]. In that proposal, if the motion vector mvL0 of co-located partition were not available, availability of mvL1 of co-located partition is checked and, if available, the mvL1 is used to derive temporal motion vector predictor. However, this process doesn’t work in Bf-frames because mvL1 cannot be available when mvL0 is not available due to the disabled list 1 uni-prediction.

In order to improve the coding efficiency in Bf-frames, in this proposal, two temporal motion vector predictors (mvL0Col and mvL1Col), which derived from motion vector mvL0 and mvL1 of co-located partition respectively, are added to motion vector predictor candidates in Bf-frames (Figure 4).

Figure 4 two temporal motion vector predictors in proposed method

# Experimental results

Proposed method is implemented on TMuC0.9-hm software and tested for four inter coding configurations. Table 1 shows its RD performance and execution time of the software relative to TMuC0.9-hm. In random access cases, proposed method saves 0.1% bits for high efficiency and 0.3% bits for low complexity on average. The difference in the encoding and decoding time is not obvious. In current configuration of random access cases, as there is small number of Bf-frames less than 1/8, its improvement of RD performance is small. On the other hand, in low delay case, proposed method saves 1.0% bits for high efficiency and 2.2% bits for low complexity on average. And proposed method reduces encoding and decoding time slightly because the number of RD-cost evaluation loop has been decreased in encoder side and the computation time to create bi-prediction image for motion compensation has been reduced in decoder side due to using uni-prediction for skipped prediction units in Bf-frames.

Table 1 The RD performance of proposed method (anchor: TMuC0.9-hm)

Figure 5 and 6 shows the BD rate for each sequence in random access cases for high efficiency and low complexity, in which negative value means better RD performance relative to TMuC0.9-hm. And Figure 7 and 8 shows the BD rate for each sequence in low delay cases for high efficiency and low complexity. Bit reduction was observed for almost all sequences, especially in low delay cases.

Figure 5 BD-Rate of random access case for high efficiency

Figure 6 BD-Rate of random access case for low complexity

Figure 7 BD-Rate of low delay case for high efficiency

Figure 8 BD-Rate of low delay case for low complexity

Table 2 shows the proposed changes in the prediction unit syntax of the WD1 of HEVC [4] (highlighted parts).

| Table 2 Prediction unit syntax including proposed method prediction_unit( x0, y0, log2PUWidth, log2PUHeight ) { | Descriptor |
| --- | --- |
| if( skip_flag[ x0 ][ y0 ] ) { |  |
| if (Ref_List1 = = Ref_List0) { |  |
| inter_pred_idc[ x0 ][ y0 ] = Pred_L0 |  |
| } |  |
| if( NumMVPCand( L0 ) > 1 ) |  |
| mvp_idx_l0[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| if(NumMVPCand( L1 )>1 && inter_pred_idc[ x0 ][ y0 ] != Pred_L0) |  |
| mvp_idx_l1[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } else if( PredMode = = MODE_INTRA ) { |  |
| prev_intra_luma_pred_flag[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| if( !prev_intra_luma_pred_flag[ x0 ][ y0 ] ) |  |
| rem_intra_luma_pred_mode[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } else { /* MODE_MERGE, MODE_DIRECT, MODE_INTER */ |  |
| if( merge_flag[ x0 ][ y0 ] && NumMergeCandidates > 1 ) { |  |
| merge_left_flag[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| } else if( PredMode = = MODE_DIRECT ) { |  |
| if (Ref_List1 = = Ref_List0) { |  |
| inter_pred_idc[ x0 ][ y0 ] = BiPred |  |
| } |  |
| else if( slice_type = = B ) |  |
| inter_pred_idc[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| if( inter_pred_idc[ x0 ][ y0 ] != Pred_L1 && NumMVPCand( L0 ) > 1 ) |  |
| mvp_idx_l0[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| if( inter_pred_idc[ x0 ][ y0 ] != Pred_L0 && NumMVPCand( L1 ) > 1 ) |  |
| mvp_idx_l1[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } else { |  |
| if( slice_type = = B ) |  |
| inter_pred_idc[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| if( inter_pred_idc[ x0 ][ y0 ] ! = Pred_L1 ) { |  |
| if( num_ref_idx_l0_active_minus1 > 0 ) |  |
| ref_idx_l0[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| mvd_l0[ x0 ][ y0 ][ 0 ] | se(v) ¦ ae(v) |
| mvd_l0[ x0 ][ y0 ][ 1 ] | se(v) ¦ ae(v) |
| if( NumMVPCand( L0 ) > 1 ) |  |
| mvp_idx_l0 | ue(v) ¦ ae(v) |
| } |  |
| if( inter_pred_idc[ x0 ][ y0 ] != Pred_L0 ) { |  |
| if( num_ref_idx_l1_active_minus1 > 0 ) |  |
| ref_idx_l1[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| mvd_l1[ x0 ][ y0 ][ 0 ] | se(v) ¦ ae(v) |
| mvd_l1[ x0 ][ y0 ][ 1 ] | se(v) ¦ ae(v) |
| if( NumMVPCand( L1 ) > 1 ) |  |
| mvp_idx_l1 | ue(v) ¦ ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |

# Conclusion

In this contribution, a modified usage of temporally/spatially predicted motion vector is proposed for Bf-frames. An experimental result shows that the proposed method saves 1.0% bits for high efficiency and 2.2% bits for low complexity on average in the low delay scenario. Moreover, proposed method reduces encoding and decoding time slightly relative to TMuC0.9-hm. Based on this BD rate gain without major complexity impact, we propose that this approach be adopted in the HM/WD of HEVC.

# References

- Frank et al., “Common test conditions and software reference configurations”, JCTVC-C500, October 2010

- Bin Li et al., “Redundancy reduction in B-frame coding at temporal level zero”, JCTVC-C278, October 2010

- Toshiyasu Sugio et al., “Modified derivation process of temporal motion vector predictor”, JCTVC-D273, January 2011

- Thomas Wiegand et al., “WD1: Working Draft 1 of High-Efficiency Video Coding”, JCTVC-C403, October 2010

# Patent rights declaration(s)

Panasonic Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).