| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D275 WG11 Number: m19040 |
| --- | --- |

| Title: | Adaptive Coding of PredMode Syntax Elements |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wen Zhang Dong Wang Yingjie Hong Wen Zuo Ming Li | Tel: Email: | +86-755-26776534 zhang.wen4@zte.com.cn wang.dong@zte.com.cn hong.yingjie@zte.com.cn zuo.wen@zte.com.cn li.ming42@zte.com.cn |
| Source: | ZTE Corporation |  |  |

_____________________________

# Abstract

In the current TMuC0.9-hm, several syntax elements, i.e. SkipFlag, MergeFlag and the flags in codePredMode, codePartSize functions, are used to signal different inter-frame prediction modes for CUs. Because of the varying statistical features of the PredMode related elements among slices, fixed coding order and mapping approach cannot always achieve high efficiency in representing the PredMode information. To tackle this problem, an adaptive coding method for PredMode elements is proposed.

# Introduction

In the current TMuC0.9-hm, each CU employs some syntax elements of PredMode to signal its inter-frame prediction mode, where fixed coding order and mapping method are used to code PredMode information. However, the statistical characteristics of different elements for PredMode always varies among slices, which will leave some redundancy in coding PredMode information, especially when SKIP_MODE is not the highest probable case in the current coding slice. From the perspective of the efficiency issue of variable length coding, the event of the highest probability should be assigned with the shortest codeword. Therefore, the coding efficiency of the PredMode information can be further improved by adaptively adjusting the coding order for the related elements at the slice level. To enable the HEVC with the ability of adaptive coding of PredMode information, a new syntax element pred_mode_coding_type is added in the slice header to specify the coding type of the PredMode related elements in current slice. The encoder determines the value of pred_mode_coding_type based on the estimated statistical features of the previous coded slices, which results in adaptive coding of the PredMode syntax elements at slice or frame level.

- Proposed Method

In the current syntax for HEVC, the representation of inter-frame PredMode is related to several functions, i.e.  codeSkipFlag, codeMergeFlag, and codePredMode together with codePartSize, which are used to write the signaling bits for inter modes. In the proposed method, a new syntax element namely pred_mode_coding_type is introduced in P/B-slice header to indicate coding type used in current slice to code the syntax elements of PredMode. Since there are 3 coding functions, the coding order of the elements of SkipFlag, MergeFlag and inter-frame prediction information can be determined by specifying the coding order of the first two elements, where each coding order is referred as a coding type. The newly defined coding types by pred_mode_coding_type are listed in Table 1, and the syntax elements of the proposed slice header are given in Table 2. Here, the encoder determines pred_mode_coding_type value for the current slice according to the statistical results of the occurrence of the three inter-frame prediction methods obtained from the previous coded slices, which equip the codec with the ability of adaptive coding of the syntax PredMode at the slice or frame level.

Table 1 pred_mode_coding_type for different cases

| SKIP_MERGE | Coding SkipFlag first, then MergeFlag |
| --- | --- |
| SKIP_INTER | Coding Skipflag first, then InterFlag |
| MERGE_SKIP | Coding MergeFlag first, then SkipFlag |
| MERGE_INTER | Coding MergeFlag first, then InterFlag |
| INTER_SKIP | Coding InterFlag first, then SkipFlag |
| INTER_MERGE | Coding InterFlag first then MergeFlag |

Table 2 Syntax added in slice header

| slice_header( ) { | C | Descriptor |
| --- | --- | --- |
| first_lctb_in_slice | 2 | ue(v) |
| slice_type | 2 | ue(v) |
| pic_parameter_set_id | 2 | ue(v) |
| frame_num | 2 | u(v) |
| if( IdrPicFlag ) |  |  |
| idr_pic_id | 2 | ue(v) |
| pic_order_cnt_lsb | 2 | u(v) |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |  |
| pred_mode_coding_type | 2 | ue(v) |
| num_ref_idx_active_override_flag | 2 | u(1) |

- Experimental results

The proposed method was evaluated using the recommended common test condition presented in JCTVC-C500. The four configurations, i.e. RAHE, RALC, LDHE and LDLC, were employed in the experiments to get the anchors. The test platform is HP server E5506/2.13 GHz with Windows NT server 2003. The test results for all sequences from class A to E are shown in Table 3.

Table 3 Test results of PredMode adaptive coding

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.2 | 0.0 | 0.0 | -1.2 | -0.8 | -0.8 |
| Class B | -0.1 | -0.1 | -0.2 | -1.0 | -0.7 | -0.7 |
| Class C | -0.1 | -0.1 | 0.0 | -1.4 | -1.1 | -1.1 |
| Class D | -0.2 | -0.2 | -0.1 | -1.3 | -1.1 | -1.0 |
| Class E |  |  |  |  |  |  |
| All | -0.1 | -0.1 | -0.1 | -1.2 | -0.9 | -0.9 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 102% |  |  | 100% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.1 | -0.1 | -0.1 | -0.5 | -0.8 | -0.9 |
| Class C | 0.0 | -0.1 | -0.2 | -0.6 | -0.8 | -0.6 |
| Class D | -0.1 | -0.9 | 0.0 | -1.0 | -1.1 | -1.0 |
| Class E | -0.3 | -0.4 | -0.3 | -0.1 | 0.0 | -0.1 |
| All | -0.1 | -0.4 | -0.1 | -0.6 | -0.8 | -0.7 |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | 99% |  |  | 97% |  |  |

# Conclusions

In this document, an adaptive coding method for PredMode information is proposed, which enables the HEVC codec to further remove the redundancy information in the current syntax design. As demonstrated by the experimental results, the proposed method achieves efficiency improvement with even lower complexity. Moreover, the proposed method could certainly be used for other syntax elements, such as Cbf, PartSize, InterDir, MvpIdx, etc. From this point of view, it is recommended to consider the adoption of this adaptive coding method for syntax elements of PredMode, and to establish a CE or AHG on the issue of adaptive syntax coding at slice or frame level.

# Patent rights declaration(s)

ZTE Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).