| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D276 |
| --- | --- |

| Title: | Rate-Distortion Optimized Transforms for Intra Block Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Feng Zou, Oscar C. Au, Chao Pang, Jingjing Dai, Xingyu Zhang and Xing Wen HKUST, Clear Water Bay KLN, Hong Kong | Tel: Email: | 852-51328827 fengzou@ust.hk |
| Source: | The Hong Kong University of Science and Technology |  |  |

_____________________________

# Abstract

This contribution proposes a rate-distortion (RD) optimized transform scheme for NxN TU in HM0.9 [1]. The transform pairs are trained off-line and implemented using matrix multiplication in HM0.9 software. This technique achieves 3.2% BDBR in Intra Low Complexity (LoCo) condition and 2.0% BDBR in Intra High Efficiency (HE) condition.

# Introduction

Many techniques are proposed to improve intra block coding efficiency in TMuC [2], such as MDDT [3] from Qualcomm, ROT [4] from Samsung and related simplification techniques. In most proposals, the residue data is classified according to the intra prediction mode. However, due to the data variation, this classification does not lead to optimal data separation. It means that even though the residue blocks are using the same prediction mode, they may exhibit different statistical or structural properties.

In this proposal, we are going to find a set of RD optimized transform pairs, which can minimize the total RD cost in Equation 1.

(1)

where X is the residue signal , and Y is the dequantized coefficient. C and R are the column and row transforms respectively.  is a predefined Lagrange parameter.

In order to get the RD optimized transform pairs, we preliminarily classify the residue data into 5 groups according to prediction mode (one for DC predicted residue, the other four for residue using neighboring prediction direction shown in Figure 1). For each group, except for DC predicted group, we use the iteration method to get the optimal transform matrices C and R which can minimize the Equation 1 for this group. For DC predicted group, we fix the transform as DCT. After the optimal transform pairs are obtained, we reclassify the residue data according to Equation 1. After several transform optimization and residue data reclassification, the total RD cost expressed in Equation 1 is converged. The transform matrices do not change too much.

Figure 1.   Initial mode grouping in intra prediction mode order

# Implementation

The transform pairs are trained off-line using several sequences (not included in the test sequences). The proposed transform scheme is implemented in HM0.9 software using integer matrix multiplication.

(2)

The matrix elements are scaled by 128 in the implementation. For hardware implementation convenience, we use only one matrix multiplication unit, which can be used for forward and backward matrix multiplication.

(3)

Currently, we implemented the proposed transform for 4x4 and 8x8 TU only. For the other TU sizes, traditional DCT is used.

In the implementation, for each CU, after the best PU structure is obtained using traditional DCT, the four transforms are tested. At the encoder, one bit flag is used to indicate whether the proposed the transform is used or not. If it is not used, traditional DCT is employed. If the proposed transform is used, two bit flag is used to indicate the transform index.

# Test conditions and simulation results

The anchor is HM0.9 with encoding conditions specified in the common test conditions [4] for HM0.9 using the intra high-efficiency encoder_intra.cfg file and intra low complexity encoder_intra_loco.cfg. The results are verified at the decoder. The negative number in Table I means bit rate saving against the anchor. Note that the proposed transform scheme is only implemented in 4x4 and 8x8 TU size, and not yet implemented in UV component. Traditional zigzag scanning order is used. We expect further improvement by incorporating the different scanning order techniques. The proposed scheme is also can be used for inter block coding.

Table I. Simulation results for HE and LoCo configuration.

| Class Name | Sequence Name | Intra HE (BDBR %) |  | Intra LoCo (BDBR %) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Proposed | ROT | Proposed | ROT |
| Class A |  |  |  |  |  |
|  | Traffic | -2.2 | -2.7 | -3.6 | -3.7 |
|  | PeopleOnStreet | -3.0 | -2.9 | -4.4 | -3.9 |
| Class B |  |  |  |  |  |
|  | Kimono | -0.5 | -1.5 | -0.9 | -3.2 |
|  | ParkScene | -1.4 | -1.9 | -3.2 | -3.1 |
|  | Cactus | -2.0 | -2.4 | -3.3 | -3.4 |
|  | BasketballDrive | -1.6 | -2.8 | -2.6 | -3.5 |
|  | BQTerrace | -1.8 | -2.0 | -3.0 | -2.8 |
| Class C |  |  |  |  |  |
|  | BasketballDrill | -2.8 | -3.4 | -3.7 | -4.0 |
|  | BQMall | -2.3 | -2.7 | -3.8 | -3.7 |
|  | Partyscene | -2.3 | -2.5 | -3.0 | -2.9 |
|  | RaceHorsesC | -1.6 | -2.4 | -3.0 | -3.4 |
| Class D |  |  |  |  |  |
|  | BasketballPass | -2.2 | -2.8 | -3.3 | -3.5 |
|  | BQSquare | -2.5 | -2.6 | -3.1 | -3.1 |
|  | BlowingBubbles | -2.4 | -2.6 | -3.3 | -3.2 |
|  | RaceHorses | -2.1 | -2.6 | -3.5 | -3.6 |
| Class E |  |  |  |  |  |
|  | Vidyo1 | -2.2 | -3.2 | -3.3 | -4.3 |
|  | Vidyo3 | -1.8 | -2.5 | -3.6 | -3.6 |
|  | Vidyo4 | -1.7 | -2.9 | -3.0 | -3.8 |
| Average |  |  |  |  |  |
|  | Class A | -2.6 | -2.8 | -4.0 | -3.8 |
|  | Class B | -1.5 | -2.1 | -2.6 | -3.2 |
|  | Class C | -2.2 | -2.8 | -3.3 | -3.5 |
|  | Class D | -2.3 | -2.7 | -3.3 | -3.3 |
|  | Class E | -1.9 | -2.8 | -3.3 | -3.9 |
|  | All | -2.0 | -2.6 | -3.2 | -3.5 |

Note: ROT transform supports 4x4 up to 64x64 TU sizes, while currently the proposed scheme is only implemented on 4x4 and 8x8 TU sizes.

Table II: Simulation results for HE and LoCo configuration. The anchor is HM0.9.

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -2.6 | -1.4 | -1.5 | -4.0 | -0.2 | 0.2 |
| Class B | -1.5 | -0.3 | -0.2 | -2.6 | 0.1 | 0.2 |
| Class C | -2.2 | -0.3 | -0.3 | -3.3 | -0.1 | -0.1 |
| Class D | -2.3 | -0.4 | -0.4 | -3.3 | -0.2 | -0.2 |
| Class E | -1.9 | -0.7 | -0.5 | -3.3 | 0.9 | 0.6 |
| All | -2.0 | -0.5 | -0.5 | -3.2 | 0.1 | 0.1 |
| Enc Time[%] | 287% |  |  | 351% |  |  |
| Dec Time[%] | 113% |  |  | 125% |  |  |

# Conclusion

The proposed RD optimized transform achieves 3.2% BDBR in LoCo setting and 2.0% BDBR in HE setting, without causing significant decoder complexity increase. We recommend that the JCT-VC evaluate this transform scheme in a core experiment.

# Patent rights declaration(s)

The Hong Kong University of Science and Technology may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- JCT-VC, TMuC-0.9 Reference Software, https://hevc.hhi.fraunhofer.de/svn/svn_TMuCSoftware/tags/0.9/  and http://hevc.kw.bbc.co.uk/git/w/jctvc-tmuc.git.

- JCT-VC, “Test Model under Consideration”, JCTVC-B205, Joint Collaborative Team on Video Coding meeting, July 2010, Geneva, Switzerland.

- “Qualcomm’s Response to the Call for Proposals on Video Compression Technology”, JCTVC-A121, Joint Collaborative Team on Video Coding Meeting, April 2010, Dresden, Germany.

- “Samsung’s Response to the Call for Proposals on Video Compression Technology”, JCTVC-A124, Joint Collaborative Team on Video Coding Meeting, April 2010, Dresden, Germany.

- “Common test conditions and software reference configurations,” F. Bossen, JCTVC-C500, Oct 2010, Guangzhou,China.