| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D277 WG11 Number: m19042 |
| --- | --- |

| Title: | CE9: Cross-check result of skip tools (3.1.p) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Seyoon Jeong, Hui Yong Kim, and Jin Soo Choi ETRI, 138 Gajeong-Ro, Yuseong-Gu, Daejeon, 305-700, Korea Kyungyong Kim, Sangmin Kim, and Gwanghoon Park Media Lab., Kyung Hee University 1, Seochun, Kiheung, Youngin, Gyonggi, 449-701 | Tel: Email: | +82-42-860-5724 {jsy, hykim5, jschoi} @etri.re.kr +82-31-201-3680 {kimky, jephiros, ghpark} @khu.ac.kr |
| Source: | ETRI and Kyung Hee University |  |  |

_____________________________

# Abstract

This contribution summarizes the verification result of Modified SKIP with AMVP1 (3.1.p) by NTTDOCOMO. The software has been verified and the experimental results are confirmed.

- Verification of source code change

HHI and NTTDOCOMO have provided the patched TMuC 0.9-CE9-HHI-DOCOMO2 source code.

|  | Skip | Direct | Merge |
| --- | --- | --- | --- |
| 3.1.p | Modified AMVP 1 | 0 | CU based*** |

***	In these cases, the HHI_MRG_CU macro is not defined and the HHI_MRG_CBF_2Nx2N macro is not defined for CU merging.[1]

By comparing the original TMuC 0.9 source code with the patched version, we found that some of pred_mode routines in the prediction_unit() was eliminated in order to disable the direct modes. We checked the software and have no special issue to raise.

- Verification result

The provided source code was compiled with Microsoft Visual Studio 2008 on Windows XP 32-bit operating system.

|  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.1 | -0.1 | 0.1 | -0.5 | -0.4 | -0.2 |
| Class B | 0.2 | 0.2 | 0.2 | 0.0 | -0.1 | 0.0 |
| Class C | 0.2 | 0.2 | 0.4 | -0.1 | 0.2 | 0.2 |
| Class D | 0.0 | 0.1 | 0.5 | -0.2 | -0.2 | 0.1 |
| Class E |  |  |  |  |  |  |
| All | 0.1 | 0.1 | 0.3 | -0.2 | -0.1 | 0.1 |
| Enc Time[%] | 77% |  |  | 78% |  |  |
| Dec Time[%] | 100% |  |  | 98% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.3 | -0.1 | -0.2 | -0.2 | -0.6 | -1.0 |
| Class C | 0.3 | 0.2 | 0.1 | -0.4 | -0.7 | -0.7 |
| Class D | 0.2 | -0.6 | 0.2 | -0.5 | -0.7 | -0.9 |
| Class E | -1.2 | -3.0 | -2.5 | -1.8 | -1.5 | -1.4 |
| All | 0.0 | -0.7 | -0.5 | -0.6 | -0.8 | -1.0 |
| Enc Time[%] | 91% |  |  | 88% |  |  |
| Dec Time[%] | 110% |  |  | 97% |  |  |

We found no problem in compiling, building, encoding and decoding.

Bit rates and PSNR results were compared: The results are listed in “JCTVC-D277_cross-check_result_(ETRI and KHU).xlsx”. Our experiment results were exactly matched with those provided by NTTDOCOMO except only one case. The small miss-match occurred in S07 BQTerrace, QP22, RA/HE test case where about 0.32% bitrate difference have found.

Reference

| [1] | JCTVC-C509r3 “Core Experiment 9: Motion Vector Coding “ |
| --- | --- |