| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D278 |
| --- | --- |

| Title: | Improved Signaling and Binarization of Chroma Intra Prediction Mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jingjing Dai, Oscar C. Au, Feng Zou, Chao Pang, Xing Wen Department of Electronic and Computer Engineering Hong Kong University of Science and Technology Clear WaterBay, Kowloon, Hong Kong | Tel: 852-90825166 Email: jjdai@ust.hk |  |
| Source: | Hong Kong University of Science and Technology |  |  |

_____________________________

# Abstract

This contribution proposes an improved signaling and binarization method for Chroma Intra Prediction Mode. In current TMuC software, the U and V components share the same intra prediction mode, and five modes can be chosen. In the order of mode number, the five modes are: Vertical, Horizontal, DC, Down-right and Luma angle mode. We find that the current binarization of these five modes in Tmuc is not efficient, and we propose an improved signaling and binarization method. Experimental results show that RD improvement is achieved in all intra configurations.

# Introduction

In the current TMuC software, the U and V components share the same intra prediction mode, and there are five modes that can be chosen, which are shown in Table 1.

Table 1 Five chroma intra prediction mode

| mode num | TMuC |
| --- | --- |
| 0 | Vertical |
| 1 | Horizontal |
| 2 | DC |
| 3 | Down-right |
| 4 | Luma angle mode |

These five modes are binarized according to Figure 1. We find that such kind of binarization is not very efficient. As seen from Figure 1, TMuC always assigns the shortest code to mode 0, and the longest code to mode 4, which indicates that the probability of choosing mode 0 as the optimal one is the highest. However, according to our observation, the choice of best mode does not always behave like this.

Mode Num = 0 ？

N

Y

Output 0

Binarize (Mode Num – 1) Using Unary Code

Output 1

Figure 1 Binarization of Chroma Intra Prediction Mode in TMuC

# Proposed method

We observe that under different luma prediction modes, the probability of vertical mode chosen as the best chroma prediction mode is not always the largest. In fact, the best intra prediction mode of chroma components tend to be similar to that of the luma component, which indicates that the probability of the luma angle mode chosen as the best prediction mode for chroma is high. Based on this, we propose an improved signaling of chroma intra prediction mode. In our scheme, the luma angle mode is always given the shortest code. Considering that when the luma prediction mode is 0—3, there exists redundancy among the five modes list in Tab. 1, we propose that the available intra prediction modes for chroma are different when the luma angle is different, which are specified in Table 2.

Table 2 Available intra prediction modes for chroma intra prediction and its corresponding code number under different luma angles

| - Code Number | - Chroma Intra Prediction Mode |
| --- | --- |
| - Luma Angle is Vertical |  |
| - 0 | - Luma Angle（Vertical） |
| - 1 | - Horizontal |
| - 2 | - DC |
| - 3 | - Down-right |
| - Luma Angle is Horizontal |  |
| - 0 | - Luma Angle （Horizontal） |
| - 1 | - Vertical |
| - 2 | - DC |
| - 3 | - Down-right |
| - Luma Angle is DC |  |
| - 0 | - Luma Angle （DC） |
| - 1 | - Vertical |
| - 2 | - Horizontal |
| - 3 | - Down-right |
| - Luma Angle is Down-right |  |
| - 0 | - Luma Angle （Down-right） |
| - 1 | - Vertical |
| - 2 | - Horizontal |
| - 3 | - DC |
| - Luma Angle are others |  |
| - 0 | - Luma Angle |
| - 1 | - Vertical |
| - 2 | - Horizontal |
| - 3 | - DC |
| - 4 | - Down-right |

In the proposed scheme, the code number in Table 2 is binarized as Figure 1, and transmitted. At the encoder and the decoder, the relationship between the code number and the real intra prediction angle of chroma is obtained as follows:

If the code number is 0, the intra prediction angle of chroma is the same as that of luma;

Else, the real prediction mode is deduced by

((code number - 1) < luma prediction mode number) ? (code number - 1) : code number.           (1)

# Simulation results

The proposed improved signaling and binarization of chroma intra prediction mode was implemented in TMuC 0.9 and the test condition specified by JCTVC-B300 [1] was used for the simulation. The coding performances of the proposed signaling scheme over the current implementation in TMuC are shown in table 3-1 and table 3-2 for intra_low_complexity and intra_high_efficiency. It can be observed from these two tables that bit-rate reduction is achieved by the proposed improved signaling.

Table 3-1 Performance of proposed improved signaling under intra_LoCo configuration

|  | Intra_loco |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.6 | -0.3 | 0.0 |
| Class B | -0.6 | -0.5 | -0.3 |
| Class C | -0.7 | -0.5 | -0.4 |
| Class D | -0.5 | -0.3 | -0.2 |
| Class E | -0.5 | 0.0 | -0.1 |
| ALL | -0.6 | -0.3 | -0.3 |

Table 3-2 Performance of proposed improved signaling under intra_HE configuration

|  | Intra_HE |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.9 | 0.1 | 0.3 |
| Class B | -0.7 | 0.1 | 0.2 |
| Class C | -0.7 | 0.2 | 0.3 |
| Class D | -0.6 | 0.2 | 0.3 |
| Class E | -1.1 | -0.5 | -0.2 |
| ALL | -0.8 | 0.0 | 0.2 |

# Computational complexity analysis

The extra computation introduced in this proposal includes the luma angle dependent binarization and the computation of equation (1). The computation complexity of both these two is negligible.

# Conclusion

This proposal proposes an improved binarization of chroma intra prediction mode, and achieves 0.6% BDBR in Intra Low Complexity (LoCo) condition and 0.8% BDBR in Intra High Efficiency (HE) condition with negligible extra computation complexity. We recommend the standard committee to include this technique into HM or future core experiment.

# References

[1] JCT-VC, “Common test conditions and software reference configurations”, F. Bossen, JCTVC-C500, Oct 2010, China, Guangzhou.

# Patent rights declaration(s)

The Hong Kong University of Science and Technology may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).