| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D280 WG11 Number: m19045 |
| --- | --- |

| Title: | Performance report of DPCM-based memory compression on TMuC 0.9 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hirofumi Aoki Keiichi Chono Kenta Senzaki Junji Tajime Yuzo Senda 1753, Shimonumabe, Nakahara-Ku Kawasaki, Kanagawa, 211-8666 JAPAN | Tel: Email: | +81 50 3757 5438 hao@bu.jp.nec.com chono@ct.jp.nec.com k-senzaki@bp.jp.nec.com tajime@ak.jp.nec.com y-senda@bc.jp.nec.com |
| Source: | NEC Corporation |  |  |

_____________________________

# Abstract

This contribution presents a performance report of DPCM-based reference frame memory compression scheme proposed in JCTVC-B057, JCTVC-C094 and JCTVC-C095 on TMuC 0.9. For 12-bit to 7.5-bit compression in high efficiency configurations where IBDI is enabled, coding losses of the proposed scheme measured by BD-Rates are 1.2 % with fixed quantization and 0.7 % with adaptive quantization introduced in JCTVC-C095. It should be noted that they are less than those of fixed rounding presented in JCTVC-D045. As for memory access bandwidth, average reduction is 47.8 %. For 8-bit to 5.5-bit compression in low complexity configurations where IBDI is disabled, average coding losses are 8.2 % with fixed quantization and 4.7 % with adaptive quantization. Average memory access bandwidth reduction is 55.2 %. It should be also noted that the scheme have potential for more gain by encoder optimizations, since the results shown here are obtained with the same quantization matrix set both for luma and chroma components, and for all pictures of all sequences. It is proposed that the DPCM-based memory compression scheme is further studied in the context of Core Experiments.

- Introduction

At the 2nd JCT-VC meeting held in Geneva and the 3rd JCT-VC meeting held in Guangzhou, a DPCM-based in-loop reference frame memory compression scheme [1-3] has been proposed as an approach to reduce memory size and bandwidth required in motion compensation. In this contribution, the evaluation results of the DPCM-based scheme on TMuC 0.9 are reported.

- Algorithm Description

## Overview

Figure 1 and Figure 2 shows an example for memory compression with the proposed DPCM-based method [1, 2] for 1-D memory compression unit (MCU) and 2-D MCU, respectively. In every MCU, the top-left-most pixel is first coded with PCM and other top-most pixels are coded with rightward DPCM. In addition, left-most pixels except for the top-left-most one are coded with downward DPCM. As for other pixels, they are coded with adaptive DPCM.

For each pixel coded with adaptive DPCM, the prediction value is selected adaptively from its upper and left neighbors. The selection is based on comparison of the horizontal and vertical pixel differences around the pixel to be coded, in the same manner as the DC and AC prediction of the MPEG-4 standard.

Figure 1: 1-D DPCM prediction structure and bit allocation.

Figure 2: 2-D ADPCM prediction structure and bit allocation.

In order to compress differential signals, the proposed method employs nonlinear quantization matrices which are defined in sequence parameter set (SPS) or picture parameter set (PPS). Multiple quantization matrices can be used here. When multiple quantization matrices are used, an adaptive quantization technique is employed, i.e., the quantization matrix is selected for each MCU adaptively from predefined multiple quantization matrices so as to minimize the prediction error for the MCU. As the difference metric, Sum of Absolute Differences (SAD) is used in this contribution. The index to specify the selected matrix is embedded in compressed data.

Figure 3 shows an example for storage of compressed data in frame memory. Here an MCU consists of 8x1 pixels, each pixel has originally 8-bit depth and is coded with 5-bit DPCM, and the number of predefined quantization matrices is four. In the case of fixed quantization, total bit amount in a whole MCU is 43 bits. The arrangement of data in frame memory is left to each implementation but generally they are aligned, for example, in bytes. Supposing that MCU data are byte-aligned, bit width of storing each MCU is 48 bits and therefore 5 bits are kept unused. Coding the quantization matrix index needs 2 bits but it is less than the capacity of unused area. Therefore the index can be embedded into this area and no additional bit is needed.

Figure 3: Storage of compressed data in frame memory.

Syntax and semantics to signal quantization matrices and other DPCM parameters are shown in Appendix.

## Compression

The proposed memory compression algorithm is described below.

Given a 2-D image block consisting of WxH pixels, let o(x, y) be the original pixel value at a coordinate (x, y) of the compression unit. Compressed signal c(x, y) (x=0,.., W-1, y=0,.., H-1) from o(x, y) is then derived by:

c(x,y)=o(x,y);(x,y)=(0,0)iQ(x,y);(x,y)≠(0,0)

where iQ(x, y) represents an index to a quantization representative value, and it is obtained as follows.

iQ(x,y)=argmini=0,..,2D−1Qk(i)−e(x,y)

e(x,y)=(o(x,y)−p(x,y)+2B) % 2B

p(x,y)=r(x−1,0);if y=0r(0,y−1);otherwise, if x=0r(x,y−1);otherwise, if r(x−1,y−1)−r(x−1,y)<r(x−1,y−1)−r(x,y−1)r(x−1,y);otherwise,

r(x,y)=(p(x,y)+Qk(iQ(x,y))) % 2B,

where r(x, y) is the reconstructed pixel value of o(x, y), p(x, y) is the predicted pixel value for o(x, y), e(x, y) is the prediction error of o(x, y), Qk(i) is an i-th quantization representative of the k-th quantization matrix, B is bit depth of input pixels, and M is bit depth of compressed signals. The index to quantization matrices, k, is derived as follows.

k=argmink=0,..,NQ−1y=0H−1x=0W−1c(x,y)−o(x,y),

where NQ represents the number of quantization matrices.

## Decompression

Decompression of compressed signal c(x, y) is done simply by the following equation.

r(x,y)=c(x,y);(x,y)=(0,0)(p(x,y)+Qk(c(x,y)))%2B;(x,y)≠(0,0)

# Experimental Results

## Test conditions

The proposed scheme has been implemented on TMuC 0.9 and tested according to common test conditions described in JCTVC-C500[4]. Since intra-only configurations are not related with the proposal, they were skipped and following four configurations were used in this experiment.

- Random access, high efficiency (RA-HE)

- Low delay, high efficiency (LD-HE)

- Random access, low complexity (RA-LC)

- Low delay, low complexity (LD-LC)

In this experiment, parameters of the proposed scheme are set as follows.

- The MCU size is set to 4x4.

- The bit depth of quantized pixels is set to 5 for low complexity configurations, i.e., without IBDI, and 7 for high efficiency configurations, i.e., with 12-bit IBDI, respectively.

- Both of fixed quantization and adaptive quantization are tested. In the case of adaptive quantization, the number of quantization matrices is set to 4.

In the case of fixed quantization with single quantization matrix, following matrices are used for 5-bit DPCM to compress 8-bit signal and 7-bit DPCM to compress 12-bit signal, respectively.

[0, 1, 2, 3, 6, 9, 14, 18, 24, 31, 39, 48, 58, 72, 86, 103, 128, 153, 170, 184, 198, 208, 217, 225, 232, 238, 242, 247, 250, 253, 254, 255],

[0, 1, 3, 6, 9, 13, 17, 21, 26, 31, 37, 43, 49, 56, 63, 71, 79, 88, 97, 106, 116, 127, 138, 150, 162, 175, 189, 203, 218, 234, 250, 268, 386, 305, 325, 347, 369, 393, 418, 445, 473, 503, 535, 569, 605, 644, 685, 730, 778, 829, 882, 941, 1006, 1078, 1156, 1241, 1326, 1416, 1501, 1589, 1683, 1762, 1850, 1946, 2048, 2150, 2246, 2334, 2413, 2507, 2595, 2680, 2770, 2855, 2940, 3018, 3090, 3155, 3214, 3267, 3318, 3366, 3411, 3452, 3491, 3527, 3561, 3593, 3623, 3651, 3678, 3703, 3727, 3749, 3771, 3791, 3810, 3828, 3846, 3862, 3878, 3893, 3907, 3921, 3934, 3946, 3958, 3969, 3980, 3990, 3999, 4008, 4017, 4025, 4033, 4040, 4047, 4053, 4059, 4065, 4070, 4075, 4079, 4083, 4087, 4090, 4093, 4095].

In the case of adaptive quantization with four quantization matrices, following matrix sets are used or 5-bit DPCM to compress 8-bit signal and 7-bit DPCM to compress 12-bit signal, respectively.

[

[0, 1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 128, 241, 242, 243, 244, 245, 246, 247, 248, 249, 250, 251, 252, 253, 254, 255],

[0, 1, 2, 3, 5, 6, 8, 9, 11, 13, 15, 17, 20, 23, 27, 33, 128, 223, 229, 233, 236, 239, 241, 243, 245, 247, 248, 250, 251, 253, 254, 255],

[0, 1, 2, 3, 6, 9, 14, 18, 24, 31, 39, 48, 58, 72, 86, 103, 128, 153, 170, 184, 198, 208, 217, 225, 232, 238, 242, 247, 250, 253, 254, 255],

[0, 2, 5, 8, 12, 16, 20, 25, 31, 37, 44, 53, 64, 78, 93, 109, 128, 147, 163, 178, 192, 203, 212, 219, 225, 231, 236, 240, 244, 248, 251, 254]

],

[

[0, 1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16, 17, 19, 20, 21, 23, 24, 26, 28, 29, 31, 33, 35, 37, 39, 41, 43, 45, 48, 50, 53, 55, 58, 61, 64, 67, 70, 73, 77, 80, 84, 88, 92, 97, 101, 106, 111, 117, 123, 129, 136, 144, 152, 162, 173, 186, 203, 225, 2048, 3871, 3893, 3910, 3923, 3934, 3944, 3952, 3960, 3967, 3973, 3979, 3985, 3990, 3995, 3999, 4004, 4008, 4012, 4016, 4019, 4023, 4026, 4029, 4032, 4035, 4038, 4041, 4043, 4046, 4048, 4051, 4053, 4055, 4057, 4059, 4061, 4063, 4065, 4067, 4068, 4070, 4072, 4073, 4075, 4076, 4077, 4079, 4080, 4081, 4082, 4083, 4084, 4085, 4086, 4087, 4088, 4089, 4090, 4091, 4092, 4093, 4094, 4095],

[0, 1, 4, 7, 9, 12, 15, 18, 21, 24, 27, 30, 33, 36, 40, 43, 46, 49, 53, 56, 60, 63, 67, 70, 74, 77, 81, 85, 88, 92, 96, 100, 104, 108, 112, 116, 121, 125, 130, 134, 139, 144, 149, 154, 159, 165, 170, 176, 182, 189, 195, 203, 210, 218, 226, 235, 245, 256, 267, 280, 294, 312, 333, 361, 2048, 3735, 3763, 3784, 3802, 3816, 3829, 3840, 3851, 3861, 3870, 3878, 3886, 3893, 3901, 3907, 3914, 3920, 3926, 3931, 3937, 3942, 3947, 3952, 3957, 3962, 3966, 3971, 3975, 3980, 3984, 3988, 3992, 3996, 4000, 4004, 4008, 4011, 4015, 4019, 4022, 4026, 4029, 4033, 4036, 4040, 4043, 4047, 4050, 4053, 4056, 4060, 4063, 4066, 4069, 4072, 4075, 4078, 4081, 4084, 4087, 4089, 4092, 4095],

[0, 4, 8, 13, 17, 21, 26, 31, 35, 40, 45, 50, 55, 60, 65, 70, 75, 81, 86, 92, 97, 103, 108, 114, 120, 126, 132, 138, 144, 150, 156, 163, 169, 176, 183, 190, 197, 204, 212, 219, 227, 235, 243, 252, 261, 270, 279, 289, 299, 309, 320, 332, 344, 357, 370, 385, 400, 418, 437, 459, 483, 512, 549, 599, 2048, 3497, 3547, 3584, 3613, 3637, 3659, 3678, 3696, 3711, 3726, 3739, 3752, 3764, 3776, 3787, 3797, 3807, 3817, 3826, 3835, 3844, 3853, 3861, 3869, 3877, 3884, 3892, 3899, 3906, 3913, 3920, 3927, 3933, 3940, 3946, 3952, 3958, 3964, 3970, 3976, 3982, 3988, 3993, 3999, 4004, 4010, 4015, 4021, 4026, 4031, 4036, 4041, 4046, 4051, 4056, 4061, 4065, 4070, 4075, 4079, 4083, 4088, 4092],

[0, 10, 20, 30, 41, 52, 63, 75, 87, 99, 112, 125, 138, 152, 166, 180, 194, 209, 224, 240, 256, 272, 288, 306, 323, 341, 360, 379, 399, 419, 440, 461, 484, 507, 531, 556, 583, 610, 639, 669, 701, 735, 771, 809, 848, 889, 932, 979, 1029, 1082, 1139, 1199, 1262, 1325, 1391, 1454, 1515, 1578, 1645, 1708, 1766, 1828, 1899, 1971, 2048, 2125, 2197, 2268, 2330, 2388, 2451, 2518, 2581, 2642, 2705, 2771, 2834, 2897, 2957, 3014, 3067, 3117, 3164, 3207, 3248, 3287, 3325, 3361, 3395, 3427, 3457, 3486, 3513, 3540, 3565, 3589, 3612, 3635, 3656, 3677, 3697, 3717, 3736, 3755, 3773, 3790, 3808, 3824, 3840, 3856, 3872, 3887, 3902, 3916, 3930, 3944, 3958, 3971, 3984, 3997, 4009, 4021, 4033, 4044, 4055, 4066, 4076, 4086]

].

These matrices are signaled to decoders by embedding them into bitstreams using the syntax structure described in Appendix. In the case of four quantization matrices, although two extra bits are needed for embedding quantization matrix indices in each MCU, the capacity for byte-aligned storage of MCU data is preserved as follows:

- For low complexity configurations, i.e. without IBDI:
Fixed quantization: ((8+5*15)+7)/8*8 = (83+7)/8*8 = 88
Adaptive quantization: (8+5*15+2)+7)/*8 = (85+7)/8*8 = 88

- For high efficiency configurations, i.e. with 12-bit IBDI:
Fixed quantization: ((12+7*15)+7)/8*8 = (117+7)/8*8 = 120
Adaptive quantization: (12+7*15+2)+7)/*8 = (119+7)/8*8 = 120

Frame memory size and memory access bandwidth are therefore same as that with fixed quantization.

## Computing platforms

Our computing platform used for simulations is shown below.

- OS: Windows 7 Professional 64-bit

- CPU: Intel Xeon X5680 3.46GHz, 6 physical cores x 2 CPUs (12 cores in total)

- Memory: 32GiB

The encoder and decoder executables were generated with Microsoft Visual C++ 2008 SP1. We ran up to 12 encodings at the same time. Since Xeon CPU has automated over-clocking functionality, individual encoding time can be significantly varied.

## Coding efficiency and complexity

As coding efficiency measure, 4-point BD-Rates against anchors were computed. As complexity measure, relative encoding and decoding times against anchors were computed. Tables 1, 2, 3 and 4 show the summary of simulation results for RA-HE, LD-HE, RA-LC and LD-LC configurations, respectively. For RA-HE and LD-HE configurations, results of fixed rounding presented in JCTVC-D045 [5] are also shown and compared with those of the proposed scheme. More detailed results are shown in the attached spreadsheets.

These results show that average coding losses of the proposed scheme with fixed quantization measured by BD-Rate for the luma component are 0.6 %, 1.8 %, 6.7 % and 9.7 % for RA-HE, LD-HE, RA-LC and LD-LC configurations, respectively. They are less than those of fixed rounding for high efficiency configurations. It is also shown that more gains are achieved with adaptive quantization. Its coding losses are 0.3 %, 1.1 %, 3.7 % and 5.6 % for RA-HE, LD-HE, RA-LC and LD-LC configurations, respectively. On average, coding losses for high efficiency configurations are 1.2 % with fixed quantization and 0.7 % with adaptive quantization, and those for low complexity configurations are 8.2 % with fixed quantization and 4.7 % with adaptive quantization. It should be noted that the method have potential for more gain by encoder optimizations, since the results shown here are obtained with the same quantization matrix set for all pictures of all sequences.

It is also shown that complexity of the proposed scheme is not trivial on TMuC 0.9, in particular with adaptive quantization where four quantization matrices are used. It should be noted that the cost calculations for adaptive quantization can be easily parallelized and the computation time increase can be kept almost same as that of the fixed quantization case. In addition, the current software implementation is not so optimized. Further efforts may be necessary to reduce the complexity of the proposed scheme while keeping its compression performance.

Table 1: Coding efficiency and complexity performance for RA-HE configuration.

|  | Fixed Quantization |  |  | Adaptive Quantization |  |  | Fixed Rounding |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-Rates (%) |  |  | BD-Rates % |  |  | BD-Rates % |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.4 | 0.6 | 0.6 | 0.2 | 0.2 | 0.3 | 0.9 | 1.4 | 1.4 |
| Class B | 0.5 | 0.5 | 0.6 | 0.3 | 0.2 | 0.1 | 1.1 | 2.4 | 1.6 |
| Class C | 0.7 | 0.4 | 0.4 | 0.4 | 0.1 | 0.1 | 0.8 | 1.4 | 1.3 |
| Class D | 0.7 | 0.5 | 0.6 | 0.4 | 0.1 | 0.2 | 0.5 | 1.4 | 1.8 |
| Overall | 0.6 | 0.5 | 0.6 | 0.3 | 0.1 | 0.2 | 0.8 | 1.7 | 1.5 |
| Enc. Time (%) | 100 |  |  | 100 |  |  | 101 |  |  |
| Dec. Time (%) | 130 |  |  | 202 |  |  | 108 |  |  |

Table 2: Coding efficiency and complexity performance for LD-HE configuration.

|  | Fixed Quantization |  |  | Adaptive Quantization |  |  | Fixed Rounding |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-Rates (%) |  |  | BD-Rates % |  |  | BD-Rates % |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class B | 1.0 | 1.7 | 3.1 | 0.6 | 0.8 | 0.5 | 2.2 | 4.4 | 5.3 |
| Class C | 1.0 | 0.8 | 0.8 | 0.7 | 0.3 | 0.4 | 1.5 | 3.1 | 3.2 |
| Class D | 0.9 | 1.1 | 1.8 | 0.5 | 0.0 | 0.7 | 1.1 | 9.4 | 10.5 |
| Class E | 5.1 | 8.8 | 7.6 | 3.2 | 0.7 | 1.6 | 8.5 | 0.3 | 0.3 |
| Overall | 1.8 | 2.6 | 3.0 | 1.1 | 0.5 | 0.8 | 2.9 | 4.6 | 5.1 |
| Enc. Time (%) | 100 |  |  | 100 |  |  | 101 |  |  |
| Dec. Time (%) | 128 |  |  | 193 |  |  | 108 |  |  |

Table 3: Coding efficiency and complexity performance for RA-LC configuration.

|  | Fixed Quantization |  |  | Adaptive Quantization |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-Rates (%) |  |  | BD-Rates % |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 5.6 | 2.5 | 2.2 | 2.5 | 0.5 | 0.5 |
| Class B | 6.6 | 2.8 | 2.7 | 3.2 | 0.6 | 0.7 |
| Class C | 6.7 | 3.9 | 3.8 | 4.1 | 1.6 | 1.5 |
| Class D | 7.3 | 3.3 | 4.0 | 4.5 | 1.2 | 1.8 |
| Overall | 6.7 | 3.2 | 3.3 | 3.7 | 1.0 | 1.2 |
| Enc. Time (%) | 101 |  |  | 101 |  |  |
| Dec. Time (%) | 161 |  |  | 321 |  |  |

Table 4: Coding efficiency and complexity performance for LD-LC configuration.

|  | Fixed Quantization |  |  | Adaptive Quantization |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-Rates (%) |  |  | BD-Rates % |  |  |
|  | Y | U | V | Y | U | V |
| Class B | 8.1 | 2.8 | 2.7 | 4.2 | 0.8 | 0.7 |
| Class C | 8.2 | 4.1 | 4.4 | 5.2 | 1.6 | 1.9 |
| Class D | 7.5 | 4.2 | 4.8 | 4.6 | 1.4 | 1.7 |
| Class E | 17.5 | 5.6 | 7.8 | 9.9 | 3.0 | 3.2 |
| Overall | 9.7 | 4.0 | 4.6 | 5.6 | 1.6 | 1.7 |
| Enc. Time (%) | 102 |  |  | 101 |  |  |
| Dec. Time (%) | 158 |  |  | 308 |  |  |

## Memory size and memory access bandwidth

As for memory size, reduction ratios are as follows.

- 8-bit to 5.5-bit for low complexity configurations: Each MCU has 85 bits (85 = 8+15*5+2) or 83 bits, and its byte-aligned size is 88 bits, while the bit width of original pixels is 128 bits.

- 12-bit to 7.5-bit when 12-bit IBDI is enabled: Each MCU has 119 bits (119 = 12 + 15*7+2) or 117 bits, and its byte-aligned size is 120 bits, while the bit width of original pixels is 192 bits.

Memory access bandwidth was measured with the tool implemented on the 0.9-ahg-memory branch of TMuC 0.9. Tables 5, 6, 7 and 8 shows average memory access bandwidth increase against anchors for RA-HE, LD-HE, RA-LC and LD-LC configurations, respectively. Given that memory access alignment size is 64 bits and burst read size is 512 bits, which are aligned with common DDR3 SDRAM architecture, average reduction ratios for RA-HE, LD-HE, RA-LC and LD-LC configurations are 47.3 %, 48.2 %, 55.4 % and 55.0 %, respectively. On average, memory access bandwidth reduction ratios are 47.8 % for high efficiency configurations and 55.2 % for low complexity configurations. It can be said that memory size and memory access bandwidth are effectively reduced by the proposed DPCM-based memory compression method.

Table 5: Memory access bandwidth increase % for RA-HE configuration.

|  | 8bit/8bit | 32bit/ 64bit | 32bit/128bit | 64bit/128bit | 64bit/256bit | 64bit/512bit | 64bit/256bit w/ FIFO cache | 64bit/512bit w/ FIFO cache |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class A | -24.8% | -29.0% | -31.8% | -33.3% | -38.8% | -48.4% | -38.2% | -38.9% |
| Class B | -27.1% | -31.3% | -34.5% | -35.3% | -39.0% | -45.1% | -37.2% | -37.7% |
| Class C | -24.6% | -29.4% | -32.2% | -33.8% | -39.1% | -48.0% | -41.7% | -42.4% |
| Class D | -21.5% | -26.8% | -29.7% | -31.9% | -38.5% | -48.9% | -43.0% | -43.6% |
| All | -24.7% | -29.3% | -32.2% | -33.7% | -38.9% | -47.3% | -40.1% | -40.7% |

Table 6: Memory access bandwidth increase % for LD-HE configuration.

|  | 8bit/8bit | 32bit/ 64bit | 32bit/128bit | 64bit/128bit | 64bit/256bit | 64bit/512bit | 64bit/256bit w/ FIFO cache | 64bit/512bit w/ FIFO cache |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class B | -22.4% | -27.3% | -30.3% | -32.1% | -38.1% | -45.6% | -38.5% | -39.2% |
| Class C | -18.8% | -24.4% | -27.2% | -30.3% | -38.6% | -48.5% | -44.5% | -45.7% |
| Class D | -13.8% | -20.1% | -23.5% | -27.3% | -38.2% | -49.9% | -42.9% | -44.1% |
| Class E | -30.9% | -32.6% | -34.3% | -34.8% | -37.8% | -50.0% | -37.2% | -39.6% |
| All | -20.9% | -25.7% | -28.6% | -31.0% | -38.2% | -48.2% | -40.9% | -42.1% |

Table 7: Memory access bandwidth increase % for RA-LC configuration.

|  | 8bit/8bit | 32bit/ 64bit | 32bit/128bit | 64bit/128bit | 64bit/256bit | 64bit/512bit | 64bit/256bit w/ FIFO cache | 64bit/512bit w/ FIFO cache |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class A | -17.2% | -22.2% | -27.7% | -29.9% | -36.7% | -54.3% | -35.8% | -37.6% |
| Class B | -19.1% | -24.1% | -30.1% | -31.4% | -38.7% | -53.4% | -32.9% | -33.6% |
| Class C | -16.7% | -22.8% | -28.3% | -30.4% | -38.5% | -56.2% | -37.2% | -38.4% |
| Class D | -12.7% | -19.9% | -25.3% | -28.3% | -37.6% | -57.8% | -35.2% | -35.3% |
| All | -16.5% | -22.4% | -28.0% | -30.1% | -38.1% | -55.4% | -35.1% | -35.9% |

Table 8: Memory access bandwidth increase % for LD-LC configuration.

|  | 8bit/8bit | 32bit/ 64bit | 32bit/128bit | 64bit/128bit | 64bit/256bit | 64bit/512bit | 64bit/256bit w/ FIFO cache | 64bit/512bit w/ FIFO cache |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class B | -12.1% | -19.6% | -27.0% | -29.2% | -36.2% | -52.9% | -36.1% | -37.1% |
| Class C | -8.5% | -17.4% | -24.8% | -28.7% | -37.9% | -58.2% | -39.2% | -41.4% |
| Class D | -2.8% | -12.9% | -22.3% | -27.6% | -39.6% | -60.7% | -46.1% | -47.6% |
| Class E | -25.4% | -27.1% | -29.8% | -30.6% | -32.8% | -46.5% | -35.0% | -35.9% |
| All | -11.4% | -18.8% | -25.8% | -28.9% | -36.9% | -55.0% | -39.2% | -40.6% |

# Conclusions

In this contribution, a performance report of DPCM-based reference frame memory compression scheme [1-3] on TMuC 0.9 was presented. Experimental results have shown that average coding losses of the proposed scheme measured by BD-Rates for 12-bit to 7.5-bit compression in high efficiency configurations where IBDI is enabled are 1.2 % with fixed quantization and 0.7 % with adaptive quantization introduced in JCTVC-C095. It should be noted that they are less than those of fixed rounding presented in JCTVC-D045. As for memory access bandwidth, average reduction is 47.8 %. For 8-bit to 5.5-bit compression in low complexity configurations where IBDI is disabled, average coding losses are 8.2 % with fixed quantization and 4.7 % with adaptive quantization. Average memory access bandwidth reduction is 55.2 %. It should be also noted that the scheme have potential for more gain by encoder optimizations, since the results shown here are obtained with the same quantization matrix set both for luma and chroma components, and for all pictures of all sequences. It is proposed that the DPCM-based memory compression scheme is further studied in the context of Core Experiments.

# References

[1] H. Aoki, et al., “TE2: 1-D DPCM-based memory compression,” JCT-VC document JCTVC-C093 (WG11 m18116), Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd meeting, Guangzhou, October 2010.

[2] H. Aoki, et al., “An extension of DPCM-based memory compression to 2-D structure with ADPCM,” JCT-VC document JCTVC-C094 (WG11 m18117), Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd meeting, Guangzhou, October 2010.

[3] H. Aoki, et al., “Performance improvement of DPCM-based memory compression by adaptive quantization,” JCT-VC document JCTVC-C095 (WG11 m18118), Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd meeting, Guangzhou, October 2010.

[4] F. Bossen, “Common test conditions and software reference configurations,” JCT-VC document JCTVC-C095 (WG11 m18118), Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd meeting, Guangzhou, October 2010.

[5] K. Chono, et al., “Rounding-error conscious memory compression method for IBDI,” JCT-VC document JCTVC-D045, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 4th meeting, Daegu, January 2011.

# Patent rights declaration(s)

NEC Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Appendix: Proposed syntax and semantics

Syntax for signaling side information about the proposed memory compression method and semantics associated with the proposed syntax are shown below. In the current implementation for this contribution, memory_compression_parameter_set() is put into seq_param_set_rbsp(). It should be noted that it can also be put into pic_param_set_rbsp(), in order to fit the quantizer characteristics to each pictures or to switch memory compression on and off.

| memory_compression_parameter_set ( ) { | C | Descriptor |
| --- | --- | --- |
| memory_compression_flag | 0 ¦ 1 | u(1) |
| if(memory_compression_flag) { |  |  |
| dpcm_bit_depth_luma_minus1 | 0 ¦ 1 | ue(v) |
| DpcmBitDepthY = dpcm_bit_depth_luma_minus+1 |  |  |
| if(DpcmBitDepthY < BitDepthY+increased_bit_depth_luma) { |  |  |
| luma_dpcm_unit_horizontal_size_minus1 | 0 ¦ 1 | ue(v) |
| luma_dpcm_unit_vertical_size_minus1 | 0 ¦ 1 | ue(v) |
| log2_num_luma_dpcm_qtables | 0 ¦ 1 | ue(v) |
| numLumaDpcmQtables = 1 << log2_num_luma_dpcm_qtables |  |  |
| for( i = 0; i < numLumaDpcmQtables; i++ ){ |  |  |
| luma_dpcm_qlist_present_flag[i] | 0 ¦ 1 | u(1) |
| if(luma_dpcm_qlist_present_flag[i]){ |  |  |
| dpcm_qlist(0, i, DpcmBitDepthY, BitDepthY+increased_bit_depth_luma) |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| dpcm_bit_depth_chroma_minus1 | 0 ¦ 1 | ue(v) |
| DpcmBitDepthC = dpcm_bit_depth_chroma_minus+1 |  |  |
| if(DpcmBitDepthC < BitDepthC+increased_bit_depth_chroma) { |  |  |
| chroma_dpcm_unit_horizontal_size_minus1 | 0 ¦ 1 | ue(v) |
| chroma_dpcm_unit_vertical_size_minus1 | 0 ¦ 1 | ue(v) |
| log2_num_chroma_dpcm_qtables | 0 ¦ 1 | ue(v) |
| numLumaDpcmQtables = 1 << log2_num_luma_dpcm_qtables |  |  |
| for( i = 0; i < numChromaDpcmQtables; i++ ){ |  |  |
| chroma_dpcm_qlist_present_flag[i] | 0 ¦ 1 | u(1) |
| if(chroma_dpcm_qlist_present_flag[i]){ |  |  |
| dpcm_qlist(1, i, DpcmBitDepthC, BitDepthC+increased_bit_depth_chroma) |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |

| dpcm_qlist ( ycIdx, tabIdx, dpcmBitDepth, bitDepth ) { | C | Descriptor |
| --- | --- | --- |
| dpcm_qlist_asymmetry_flag | 0 ¦ 1 | u(1) |
| sizeofDpcmQlist = 1 << dpcmBitDepth |  |  |
| deltaDpcmQscale = 1 |  |  |
| dpcmQlist[ycIdx][tabIdx][0] = 0 |  |  |
| if( dpcm_qtable_asymmetry_flag == 1 ) { |  |  |
| for( i = 1; i < sizeofDpcmQlist; i++ ) { |  |  |
| second_delta_dpcm_qscale | 0 ¦ 1 | se(v) |
| deltaDpcmQscale += second_delta_dpcm_qscale |  |  |
| dpcmQlist[ycIdx][tabIdx][i] = dpcmQlist[ycIdx][tabIdx][i–1] + deltaDpcmQscale |  |  |
| } |  |  |
| } else { |  |  |
| for( i = 1; i < sizeofDpcmQlist/2; i++ ) { |  |  |
| second_delta_dpcm_qscale | 0 ¦ 1 | se(v) |
| deltaDpcmQscale += second_delta_dpcm_qscale |  |  |
| dpcmQlist[ycIdx][tabIdx][i] = dpcmQlist[ycIdx][tabIdx][i-1] + deltaDpcmQscale |  |  |
| } |  |  |
| dpcmQlist[ycIdx][tabIdx][ sizeofDpcmQlist/2] = 1 << (bitDepth–1) |  |  |
| for( i = sizeofDpcmQlist /2+1; i < sizeofDpcmQlist; i++ ) { |  |  |
| dpcmQlist[ycIdx][tabIdx][i] = (1 << bitDepth) – dpcmQlist[ycIdx][tabIdx][sizeofDpcmQlist–i] |  |  |
| } |  |  |
| } |  |  |
| } |  |  |

Semantics associated with the proposed syntax structure is described below.

memory_compression_flag equal to 1 specifies that parameters for frame memory compression are present. memory_compression_flag equal to 0 specifies that these parameters are not present. When memory_compression_flag is equal to 1, reference frames are compressed by the proposed memory compression method before being stored in memory and they are decompressed after being read from the memory. When memory_compression_flag is equal to 0, no frame memory compression is done.

dpcm_bit_depth_luma_minus1 specifies the number of bits allocated for the representation of DPCM prediction errors for the luma component. The value of dpcm_bit_depth_luma_minus1 shall be in the range of 0 to BitDepthY+increased_bit_depth_luma-1, inclusive.

luma_dpcm_unit_horizontal_size_minus1 specifies the width of MCU for the luma component. The value of luma_dpcm_unit_horizontal_size_minus1 shall be equal to or greater than 0.

luma_dpcm_unit_vertical_size_minus1 specifies the height of MCU for the luma component. The value of luma_dpcm_unit_vertical_size_minus1 shall be equal to or greater than 0.

log2_num_luma_dpcm_qtables specifies the number of nonlinear quantization matrices that are used to compress DPCM prediction errors for the luma component.

luma_dpcm_qlist_present_flag[i] equal to 1 specifies that a quantization representative value list syntax structure is present to specify the (i+1)th nonlinear quantization matrix used to compress DPCM prediction errors for the luma component. luma_dpcm_qlist_present_flag equal to 0 specifies that the default nonlinear quantization matrix is used as the (i+1)th nonlinear quantization value list for the luma component. The default nonlinear quantization matrix is defined as follows.

For 5-bit DPCM in order to compress 8-bit signal, the default nonlinear quantization matrix composed of 32 elements is defined by:

[0, 1, 2, 3, 6, 9, 14, 18, 24, 31, 39, 48, 58, 72, 86, 103, 128, 153, 170, 184, 198, 208, 217, 225, 232, 238, 242, 247, 250, 253, 254, 255]

For 7-bit DPCM in order to compress 12-bit signal, the default nonlinear quantization matrix composed of 128 elements is defined by:

[0, 4, 8, 12, 16, 20, 24, 28, 32, 36, 40, 44, 48, 60, 72, 84, 96, 108, 120, 132, 144, 164, 184, 204, 224, 240, 256, 272, 288, 312, 336, 360, 384, 412, 440, 468, 496, 528, 560, 592, 624, 660, 696, 732, 768, 808, 848, 888, 928, 984, 1040, 1096, 1152, 1208, 1264, 1320, 1376, 1444, 1512, 1580, 1648, 1748, 1848, 1948, 2048, 2148, 2248, 2348, 2448, 2516, 2584, 2652, 2720, 2776, 2832, 2888, 2944, 3000, 3056, 3112, 3168, 3208, 3248, 3288, 3328, 3364, 3400, 3436, 3472, 3504, 3536, 3568, 3600, 3628, 3656, 3684, 3712, 3736, 3760, 3784, 3808, 3824, 3840, 3856, 3872, 3892, 3912, 3932, 3952, 3964, 3976, 3988, 4000, 4012, 4024, 4036, 4048, 4052, 4056, 4060, 4064, 4068, 4072, 4076, 4080, 4084, 4088, 4092]

dpcm_bit_depth_chroma_minus1 specifies the number of bits allocated for the representation of DPCM prediction errors for the chroma component. The value of dpcm_bit_depth_chroma_minus1 shall be in the range of 0 to BitDepthC+increased_bit_depth_chroma-1, inclusive.

chroma_dpcm_unit_horizontal_size_minus1 specifies the width of MCU for the chroma component. The value of chroma_dpcm_unit_horizontal_size_minus1 shall be equal to or greater than 0.

chroma_dpcm_unit_vertical_size_minus1 specifies the height of MCU for the chroma component. The value of chroma_dpcm_unit_vertical_size_minus1 shall be equal to or greater than 0.

log2_num_chroma_dpcm_qtables specifies the number of nonlinear quantization matrices that are used to compress DPCM prediction errors for the chroma component.

chroma_dpcm_qlist_present_flag[i] equal to 1 specifies that a quantization representative value list syntax structure is present to specify the (i+1)th nonlinear quantization representative values used to compress DPCM prediction errors for the chroma component. chroma_dpcm_qlist_present_flag equal to 0 specifies that the (i+1)th quantization representative value list for the chroma component shall be inferred to be identical to that for the luma component.

dpcm_qlist_asymmetry_flag equal to 1 specifies that the quantization representative value list syntax structure contains all quantization representative values. dpcm_qlist_aymmetry_flag equal to 0 specifies that the syntax structure for the quantization representative value list contains a half of quantization representative values and remainder values shall be inferred as shown in section 3.1.

second_delta_dpcm_qscale is used to derive the (i+1)th element of the quantization representative value list, which is denoted by dpcmQlist[ycIdx][tabIdx][i], for i in the range of 0 and sizeofDpcmQlist-1, inclusive.