| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D283 |
| --- | --- |

| Title: | Further Encoder Improvement of intra mode decision |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Liang Zhao, Li Zhang, Xin Zhao, Siwei Ma, Debin Zhao, Wen Gao Institute of Digital Media, Peking University Beijing, China | Tel: Email: | +86-010-62753424 lzhao@jdl.ac.cn zhanglili@pku.edu.cn xzhao@jdl.ac.cn swma@pku.edu.cn dbzhao@jdl.ac.cn wgao@pku.edu.cn |
| Source: | Peking University |  |  |

_____________________________

# Abstract

Based on JCTVC-C207 which is adopted in the last JCT-VC meeting, this contribution presents a further simplification of the encoding scheme for unified directional intra prediction in this proposal. Experimental results show the proposed scheme provides 14% time savings in intra high efficiency test and 20% time savings in intra low complexity test on average compared to current default encoding scheme in HM 0.9 with almost the same coding efficiency.

# Introduction

In the current HM, unified directional intra prediction provides up to 34 directional prediction modes for different PUs. With the PU size of 4x4, 8x8, 16x16, 32x32 and 64x64, there are 17, 34, 34, 34 and 5 prediction modes available, respectively. Based on the adopted JCTVC-C207 [1], in the current HM 0.9, only parts of the directions are compared with RD optimization to reduce the high computational complexity. Although the encoding time has been significantly reduced, there is still room for further simplification of encoder. This proposal aims to optimize the encoder with much lower computational complexity while maintaining similar coding efficiency as the current HM design.

# Encoder Scheme

In the current HM 0.9, RD optimization is applied to the N best candidate modes which are selected by the rough mode decision (RMD) where all modes are compared in this decision. In the coarse mode decision, costs are composed of absolute sum of Hadamard Transformed coefficients of residual signal (HSAD) and the bits used for represent the information of directions. There is no transform, quantization, de-quantization and inverse transform in the coarse mode selection. As a result, the encoding time of coarse mode selection is much smaller than R-D optimization (RDO). Therefore, making better use of the information of rough mode selection to reduce the number of directions for RDO is an important problem for further encoder simplification.

In this proposal, we check less numbers of best RMD modes for RDO, and the most probable mode (MPM) is always included for RDO. In Table I, the default encoding scheme in HM is compared with the proposed scheme. Also the settings related to the numbers of best RMD modes for experiments are shown in Table II.

Table I. Comparison of different encoding schemes

|  | Default in HM | Proposed |
| --- | --- | --- |
| Modes for RD optimization | N best RMD modes for RDO | if MPM included in N best RMD modes N best RMD modes for RDO else N best RMD modes + MPM for RDO |

Table II. Different settings of N in proposed scheme for simulation

| Settings | Number of N |
| --- | --- |
| S1 |  |
| S2 |  |
| S3 |  |
| S4 |  |
| S5 |  |

# Simulation Results

To demonstrate the coding efficiency of our proposed method unified intra prediction with the new encoding scheme with five different parameter settings are compared with the default encoding scheme in HM0.9. The rate-distortion performance of Y, U and V components and encoder complexity compared to the default encoding scheme are shown in the following tables (Table II~VI). The experiments are conducted with 2 test configurations specified for HM 0.9 in JCTVC-C500 [2]. The test platform used is Inter® Xeon® X5450-3.00GHz with eight cores, 8.0 GB RAM.

Table II. BD-rates and encoder complexity of S1

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.15 | -0.25 | -0.22 | 0.27 | -0.41 | -0.43 |
| Class B | 0.00 | -0.44 | -0.42 | 0.15 | -0.61 | -0.63 |
| Class C | 0.18 | -0.47 | -0.49 | 0.42 | -0.48 | -0.48 |
| Class D | 0.26 | -0.47 | -0.49 | 0.49 | -0.43 | -0.46 |
| Class E | 0.04 | -0.19 | -0.28 | 0.24 | -0.41 | -0.40 |
| All | 0.12 | -0.39 | -0.40 | 0.31 | -0.49 | -0.50 |
| Enc Time[%] | 80% |  |  | 72% |  |  |

Table III. BD-rates and encoder complexity of S2

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.06 | -0.23 | -0.16 | 0.13 | -0.30 | -0.33 |
| Class B | -0.04 | -0.37 | -0.36 | 0.01 | -0.47 | -0.46 |
| Class C | 0.08 | -0.36 | -0.38 | 0.24 | -0.32 | -0.32 |
| Class D | 0.14 | -0.38 | -0.39 | 0.29 | -0.32 | -0.30 |
| Class E | -0.02 | -0.18 | -0.27 | 0.12 | -0.26 | -0.30 |
| All | 0.04 | -0.32 | -0.33 | 0.16 | -0.35 | -0.35 |
| Enc Time[%] | 83% |  |  | 76% |  |  |

Table IV. BD-rates and encoder complexity of S3

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.01 | -0.17 | -0.19 | 0.03 | -0.24 | -0.26 |
| Class B | -0.07 | -0.33 | -0.30 | -0.08 | -0.36 | -0.36 |
| Class C | 0.03 | -0.27 | -0.28 | 0.13 | -0.24 | -0.23 |
| Class D | 0.07 | -0.22 | -0.27 | 0.18 | -0.24 | -0.23 |
| Class E | -0.05 | -0.12 | -0.21 | 0.05 | -0.20 | -0.22 |
| All | -0.004 | -0.24 | -0.26 | 0.06 | -0.27 | -0.27 |
| Enc Time[%] | 86% |  |  | 80% |  |  |

Table V. BD-rates and encoder complexity of S4

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.02 | -0.17 | -0.14 | -0.03 | -0.19 | -0.20 |
| Class B | -0.09 | -0.29 | -0.27 | -0.15 | -0.29 | -0.28 |
| Class C | 0.00 | -0.21 | -0.21 | 0.04 | -0.16 | -0.14 |
| Class D | 0.04 | -0.18 | -0.21 | 0.08 | -0.18 | -0.14 |
| Class E | -0.06 | -0.11 | -0.23 | -0.01 | -0.14 | -0.17 |
| All | -0.03 | -0.20 | -0.22 | -0.02 | -0.20 | -0.19 |
| Enc Time[%] | 88% |  |  | 84% |  |  |

Table VI. BD-rates and encoder complexity of S5

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.12 | -0.17 | -0.19 | -0.28 | -0.22 | -0.23 |
| Class B | -0.19 | -0.26 | -0.27 | -0.37 | -0.27 | -0.27 |
| Class C | -0.07 | -0.12 | -0.13 | -0.10 | -0.11 | -0.12 |
| Class D | -0.05 | -0.10 | -0.14 | -0.07 | -0.09 | -0.11 |
| Class E | -0.20 | -0.16 | -0.26 | -0.26 | -0.24 | -0.23 |
| All | -0.13 | -0.17 | -0.20 | -0.21 | -0.18 | -0.19 |
| Enc Time[%] | 97% |  |  | 94% |  |  |

# Conclusion

This contribution proposes a simplification of encoding scheme for unified directional intra prediction. Experimental results show that the proposed method can reduce the computational complexity of unified intra prediction performance while remaining similar coding efficiency. Among the overall 5 solutions of the new encoding scheme, we observe that S3 makes better tradeoff between the encoder complexity and BD-rate saving, which provides 14% time savings in intra high efficiency test and 20% time savings in low complexity test on average compared to current default encoding scheme in HM 0.9 with almost the same coding efficiency. Therefore, we recommend adopting it to the HM software as default behaviour.

# Acknowledge

We would like to thank MediaTek Inc. for doing the crosscheck for us.

# Reference

- Yinji Piao, Junghye Min, Jianle Chen, “Encoder improvement of unified intra prediction,” JCTVC-C207, Guangzhou, October 2010.

- JCT-VC, “Common test conditions and software reference configurations”, JCTVC-C500, JCT-VC Meeting, Guangzhou, October 2010.

# Patent rights declaration(s)

Peking University may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).