| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D284 |
| --- | --- |

| Title: | CE7: Mode-dependent transform, residual reordering and coefficient scanning for intra prediction residue |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xin Zhao, Li Zhang, Siwei Ma, Wen Gao Institute of Digital Media, Peking University Beijing, China | Tel: Email: | +86-010-62753424 xzhao@jdl.ac.cn zhanglili@jdl.ac.cn swma@jdl.ac.cn wgao@jdl.ac.cn |
| Source: | Peking University |  |  |

_____________________________

# Abstract

In this contribution, the results from Peking Univ. for the Core Experiment 7 (CE7) on Alternative Transforms are provided, with both tool description and simulation results. Simulation results are obtained under the test conditions defined in CE7, and the detailed rate-distortion measurements using BD-rate values are provided in the accompanying spreadsheets.

# Introduction

In the 3rd JCT-VC meeting held in Guangzhou, the Core Experiment CE7 [1] is set up for a further investigation on Alternative Transforms. At that meeting, the contribution JCT-VC C089 from Peking Univ. [2] is included as a tool under test. In this contribution, the proposed method has been integrated on TMuC 0.9 (HM) with also further improvements. Simulation results are obtained under the CE7 defined test conditions including Intra High-Efficiency, Intra Low-Complexity, Random Access High-Efficiency, and Random Access Low-Complexity [3].

# Algorithm description

For intra prediction residue, a separable transform scheme with following formulation is used.

F=D·X·DT							(1)

where X indicates the residual block of N×N size, D denotes the transform matrix. Two transform matrices are utilized in the proposed scheme, one is DCT used in TMuC 0.9, and the other is a proposed integer matrix S. For 4×4 transform, S is designed as shown below

.					(2)

S4 is actually an approximation of 4×4 DST matrix proposed in [4], [5]. The basis of N×N DST is composed of sinusoids with following formulation,

(3)

Comparisons between the proposed integer basis and standard DST are shown in Figure 1. From Figure 1, it is shown that row vectors of S in (2) are good approximations of DST basis, but the components are small integers which are advantageous for low-complexity implementations.

Figure 1 Transform basis of continuous sine transform defined in (3) and the discrete version, and proposed transform

The calculation of proposed transform using S4 require only shifts and adds, and no multiplication is needed. For calculating the 1-D forward transform of a 4 point vector x, i.e., f=S4·x, a supposed low-complexity implementation is

a0=x0- x1；a1= x1+x3；a2=(x2<<3)- x2；a3=(x0+ x1)- x3

b0= a0<<2；b1= a1<<2

c0=b0+ a2

Finally, the transform coefficient vector f=[f0, f1, f2, f3]is obtained as

f0= c0- a0+(a1<<3)；f1=(a3<<3)-a3； f2=(a0<<3)-a2+b1+a1； f3=b1-a0-c0 -a1

With the above implementation, totally 15 adds and 6 shifts are needed for doing a 1-D 4-point forward transform.

For 1-D inverse transform of 4 point vector f, i.e., x=S4T·f, a similar supposed implementation is

a0=f0+ f2；a1= f2-f3；a2=(f1<<3)- f1；a3=f0- f2- f3

b0= a0<<2；b1= a1<<2

c0=b0+ a2

Finally, the transform coefficient vector x=[x0, x1, x2, x3] is obtained as

x0= c0- a0+ b1+a1；x1= a0 +c0- (a3<<3)； x2=(a3<<3)-a3； x3=(a0<<3)-a2-b1+a1

With the above implementation, totally 15 adds and 6 shifts are needed for doing a 1-D 4-point inverse transform.

For 8x8 transform, S is analytically designed as

.				(4)

The utilization of DCT matrix and proposed transform matrix is specified by the intra mode as shown Table 1

Table 1 Prediction modes grouping in different Intra modes

|  | DC prediction | Others |
| --- | --- | --- |
| 4×4 modes (0~17) | DCT (H.264/AVC) | S4 |
| 8×8 modes (0~33) | DCT (TMuC 0.9) | S8 |

Furthermore, before the transform stage, a certain residue reordering is manipulated. There are 3 residue reordering methods including, (a) raster scan, (b) horizontal flipping and (c) vertical flipping. The reordering methods for a 4×4 block are shown in Figure 2, and the reordering methods for 8×8 case are similar.

(a) 			 	 	  (b)		  	   (c)

Figure 2 Residual reordering methods for 4x4 blocks

The reordering method for a certain block is specified by the intra prediction direction shown in Figure 3.

b

a

a

c

Figure 3 Mapping from 33 prediction directions to residual reordering method in the angular mode of Intra 8x8 macroblocks.

After the transform coefficients are quantized, a mode-dependent coefficient scanning similar to [6] is implemented to map the 2-D coefficient matrix into 1-D vector. Differences compared to the implementations in [6] include:

- Fixed coefficient scanning orders are used instead of updating one in [6] to reduce complexity.

- There are 6 coefficient scanning methods in the proposed scheme, instead of 9 in [6].

The coefficient scanning method is selected according to intra prediction direction, as specified in Table 2

Table 2 Prediction modes grouping in different Intra modes

|  | Logical prediction modes (0~33) |
| --- | --- |
| Scanning Method #0 | 7~11 |
| Scanning Method #1 | 23~27 |
| Scanning Method #2 | DC |
| Scanning Method #3 | 1~2, 16~18, 32~33 |
| Scanning Method #4 | 3~6, 12~15 |
| Scanning Method #5 | 19~22, 28~31 |

# Simulation results

## RD results

The rate-distortion performance compared to the anchor in terms of BD Rate [7] for Y, U and V components are shown in Table 2 and 3. As Table 2 shows, for “Intra High Efficiency” test condition, the proposed algorithm results in average 2.0%, 1.8% and 1.7% BD-Rate reduction for Y, U and V components, respectively. for “Intra low complexity” test condition, the proposed algorithm results in average 2.5%, 1.5% and 1.5% BD-Rate reduction for Y, U and V components, respectively. For “Random Access, High Efficiency” test condition, the proposed algorithm results in average 0.9%, 0.4% and 0.4% BD-Rate reduction for Y, U and V components, respectively. For “Random Access, Low Complexity” test condition, the proposed algorithm results in average 0.9%, 0.5% and 0.5% BD-Rate reduction for Y, U and V components, respectively.

Table 2 RD results for Intra configurations

| Sequence name | Intra High Efficiency |  |  | Intra Low Complexity |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD_YRate | BD_URate | BD_VRate | BD_YRate | BD_URate | BD_VRate |
| Traffic | -2.2 | -1.9 | -1.9 | -2.1 | -0.9 | -0.6 |
| PeopleOnStreet | -2.7 | -2.7 | -2.7 | -2.5 | -0.4 | 0.1 |
| Kimono1 | -0.5 | -0.5 | -0.6 | 0.4 | 0.7 | 0.3 |
| ParkScene | -1.3 | -0.9 | -1.1 | -2.0 | -0.9 | -0.8 |
| Cactus | -1.9 | -1.9 | -1.4 | -2.1 | -1.3 | -0.7 |
| BasketballDrive | -1.7 | -1.7 | -1.8 | -4.0 | -3.7 | -3.8 |
| BQTerrace | -1.1 | -2.2 | -1.8 | -2.6 | -3.0 | -3.4 |
| BasketballDrill | -2.6 | -1.8 | -2.0 | -3.3 | -1.4 | -1.5 |
| BQMall | -2.5 | -2.3 | -2.2 | -3.3 | -2.8 | -2.9 |
| PartyScene | -2.1 | -1.6 | -1.5 | -1.8 | -1.9 | -2.0 |
| RaceHorses | -1.3 | -1.0 | -0.9 | -1.7 | -1.1 | -0.8 |
| BasketballPass | -2.2 | -1.8 | -1.8 | -3.3 | -2.3 | -2.4 |
| BQSquare | -2.2 | -1.7 | -1.5 | -1.8 | -1.6 | -1.6 |
| BlowingBubbles | -2.4 | -1.9 | -1.9 | -2.4 | -2.1 | -2.2 |
| RaceHorses | -2.0 | -1.8 | -1.8 | -2.5 | -1.4 | -1.3 |
| vidyo1 | -2.8 | -2.4 | -2.1 | -3.5 | -1.0 | -1.2 |
| vidyo3 | -2.6 | -2.4 | -2.4 | -4.0 | -1.4 | -2.0 |
| vidyo4 | -2.6 | -2.3 | -1.8 | -2.8 | -0.9 | -1.0 |
| Class A avg | -2.5 | -2.3 | -2.3 | -2.3 | -0.6 | -0.2 |
| Class B avg | -1.3 | -1.4 | -1.4 | -2.1 | -1.6 | -1.7 |
| Class C avg | -2.1 | -1.7 | -1.6 | -2.5 | -1.8 | -1.8 |
| Class D avg | -2.2 | -1.8 | -1.8 | -2.5 | -1.9 | -1.9 |
| Class E avg | -2.7 | -2.3 | -2.1 | -3.4 | -1.1 | -1.4 |
| All average | -2.0 | -1.8 | -1.7 | -2.5 | -1.5 | -1.5 |

Table 3 RD results for Random Access configurations

| Sequence name | Random Access High Efficiency |  |  | Random Access Low Complexity |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD_YRate | BD_URate | BD_VRate | BD_YRate | BD_URate | BD_VRate |
| Traffic | -1.1 | -0.4 | -0.4 | -1.0 | -0.4 | -0.4 |
| PeopleOnStreet | -1.0 | -0.4 | 0.3 | -0.8 | 0.4 | 0.7 |
| Kimono1 | -0.2 | -0.0 | -0.2 | -0.0 | 0.3 | -0.1 |
| ParkScene | -0.7 | -0.4 | -0.5 | -0.8 | -0.4 | -0.0 |
| Cactus | -1.0 | -0.4 | -0.2 | -0.9 | -0.2 | -0.0 |
| BasketballDrive | -1.0 | -0.4 | -0.4 | -2.0 | -1.2 | -1.1 |
| BQTerrace | -0.7 | -0.5 | -0.3 | -1.0 | -1.2 | -1.8 |
| BasketballDrill | -1.4 | -0.8 | -0.7 | -1.4 | -0.3 | -0.5 |
| BQMall | -1.0 | -0.6 | -1.1 | -1.1 | -1.1 | -1.2 |
| PartyScene | -0.9 | -0.4 | -0.5 | -0.6 | -0.6 | -0.8 |
| RaceHorses | -0.6 | -0.3 | -0.4 | -0.8 | -0.3 | -0.5 |
| BasketballPass | -1.1 | -0.5 | -0.7 | -1.2 | -0.6 | -0.6 |
| BQSquare | -0.8 | -0.3 | 0.1 | -0.4 | -0.9 | -0.6 |
| BlowingBubbles | -1.0 | -0.5 | -0.0 | -0.7 | -0.5 | -0.6 |
| RaceHorses | -0.8 | -0.6 | -0.6 | -0.8 | -0.2 | -0.1 |
| vidyo1 |  |  |  |  |  |  |
| vidyo3 |  |  |  |  |  |  |
| vidyo4 |  |  |  |  |  |  |
| Class A avg | -1.1 | -0.4 | -0.0 | -0.9 | -0.0 | -0.2 |
| Class B avg | -0.7 | -0.3 | -0.3 | -1.0 | -0.5 | -0.6 |
| Class C avg | -1.0 | -0.5 | -0.7 | -1.0 | -0.6 | -0.8 |
| Class D avg | -0.9 | -0.4 | -0.3 | -0.8 | -0.5 | -0.5 |
| Class E avg |  |  |  |  |  |  |
| All average | -0.9 | -0.4 | -0.4 | -0.9 | -0.5 | -0.5 |

## Complexity

For complexity, the additional complexity in sense of arithmetic operation counts, memory requirements and bit-precision change introduced by the proposed method is shown in Table 4.

Table 4 Complexity measurement of proposed scheme compared with TMuC0.9 (HM) anchor

| Complexity Metric | 4×4 Transform | 8×8 Transform |
| --- | --- | --- |
| operation counts for transform a N-point 1-D vector | DC: no change Others: 15 adds, 6 shifts | DC: no change Others: 64 muls, 56 adds |
| memory requirements for transforms | no change | no change |
| memory requirements for coefficient scanning | 6×4×16/8=48 byte (4 bits are needed for one scan position, 2 for x, 2 for y) | 6×6 ×64/8=288 byte (6 bits are needed for one scan position, 3 for x, 3 for y) |
| minimum bit-precision (9-bit input) | DC: no change Others: 19bits | DC: no change Others: 27bits |
| Other operations and memory requirements | Mode 1~8, 26~33: residual reordering process Others: no change | Mode 1~8, 26~33: residual reordering process Others: no change |

Also both encoding and decoding time are shown in Table 5 to measure the coding speed using proposed tool. The test environment is shown below for more information:

Encoding: Intel® Xeon® CPU X5660 @ 2.80GHz, 23.9GB of RAM

Decoding: Intel® Xeon® CPU X5450 @ 3.00GHz, 8.00GB of RAM

Table 5 Ratio of running time Tproposed/Tanchor

|  | Encoding |  | Decoding |  |
| --- | --- | --- | --- | --- |
|  | Ours | Crosscheck by MediaTek (JCTVC-D290) | Ours | Crosscheck by MediaTek (JCTVC-D290) |
| Intra HE | 104% | 103% | 104% | 105% |
| Intra Loco | 106% | 106% | 103% | 114% |
| RA HE | 107% | 101% | 108% | 108% |
| RA Loco | 99% | 99% | 115% | 114% |

# Conclusion

Based on the above simulation results, it is observed that the coding efficiency of the anchor can be further promoted with applicable tradeoff with additional complexity. Therefore, it is suggested to incorporate proposed tools to the development of HM test model.

# Acknowledgment

We would like to thank MediaTek Inc. for doing crosschecking of our proposal.

# References

[1]	Robert Cohen, Chuohao Yeo, Rajan Joshi, “CE7: Alternative Transforms”, Doc. JCTVC-C507_r3, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Guangzhou, CN, 7-15 October, 2010.

[2]	Xin Zhao, Li Zhang, Siwei Ma, Wen Gao, “TE7: Results for mode-dependent residual reordering for intra prediction residual”, Doc. JCTVC-C089, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Guangzhou, CN, 7-15 October, 2010.

[3]	Frank Bossen, “Common test conditions and software reference configurations”, Doc. JCTVC-C500, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Guangzhou, CN, 7-15 October, 2010.

[4] Chuohao Yeo, Yih Han Tan, Zhengguo Li, Susanto Rahardja, “TE7: Results for Mode-Dependent Fast Separable KLT for Block-based Intra Coding”, Doc. JCTVC-C037, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Guangzhou, CN, 7-15 October, 2010.

[5] Ankur Saxena, Felix C. Fernandes, “Jointly optimal intra prediction and adaptive primary transform”, Doc. JCTVC-C108, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Guangzhou, CN, 7-15 October, 2010.

[6] Y. Ye and M. Karczewicz, “Improved Intra Coding,” ITU-T Q.6/SG11 VCEG, VCEG-AG11, Shenzhen, China, Oct 2007.

[7] Gisle Bjontegaard, “Calculation of Average PSNR Differences between RD curves,” ITU-T SC16/Q6, VCEG-M33, Austin, USA, Apr 2001.

# Patent rights declaration(s)

Peking University may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).